# Ground Control Survey Report for the      U.S. Geological Survey

Contractor: Woolpert

Date: 2020-2021

Contract: G16PC00022

Task Order: 140G0220F0194

Project Name: USGS Ohio Statewide Phase 2

Table of Contents

Ground Control Survey Report for the      U.S. Geological Survey	1

1.	Survey Report	3

1.1.	Introduction	3

1.2.	Project Area	3

1.3.	Purpose	3

1.4.	Date of Survey	3

1.5.	Monumentation	4

1.6.	Accuracy Standards	4

1.7.	GPS Equipment	4

1.8.	Methodology	4

1.8.1.	Static GPS	4

1.8.2.	Real-time Kinematic (RTK) GPS	4

1.8.3.	GPS Data Analysis and Processing	4

1.8.4.	Datum Reference and Final Coordinates	4

1.8.5.	Quality Assurance	5

2.	Ground Control/Geodetic Control Coordinates	5

2.1.	Geodetic Control	5

2.2.	Ground Control	6

2.3.	Geodetic Control- Geodetic Coordinate System NAD83	22

2.4.	Ground Control-Geodetic Coordinate System NAD83	23

3.	GPS Control Diagram	39

4.	NGS Datasheets	43

4.1.	NGS Data Sheet	43

# Survey Report

## Introduction

This report contains a comprehensive outline of the Ground Control Survey that supported the lidar data collected for the task order. All survey activity was performed to achieve ground control accuracies that meet or exceed the National Mapping Accuracy Standards (NMAS).

## Project Area

The project area consists of approximately 12,101 square miles in Ohio.

Figure 1.2.1 – Defined Project Area

## Purpose

The purpose of this survey was to establish three-dimensional coordinates for 106 lidar control points, 229 non-vegetated check points, and 162 vegetated check points. The points were collected per the flight layout and were uniformly dispersed over the project area.

## Date of Survey

Ground control field operations were conducted from, November 2020 to December 2020 and from February 2021 to October 2021.

## Monumentation

Prior to aerial imagery acquisition, Woolpert field crews performed a field reconnaissance to verify the existence and suitability of pre-selected existing National Geodetic Survey (NGS) control stations. Existing NSRS control stations were utilized as checks to ensure that quality x, y, z coordinate values were computed for each of the newly established photogrammetric control stations.  Recovery information sheets for the existing NGS control stations can be found in Section 5 of this report. A control diagram can be found in Section 3 of this report.

## Accuracy Standards

The relative vertical accuracy of the LiDAR data will be 10-cm RMSEz with swath overlap (between adjacent swaths) and an absolute- vertical accuracy of 15-cm RMSE.

## GPS Equipment

Woolpert survey crews used the following GPS equipment:

- Trimble Navigation R8 model 4 dual-frequency GPS receivers

- Trimble R10 Model GNSS dual- frequency GPS receivers

- TSC7 data collector

## Methodology

### Static GPS

The field crew utilized Static GPS surveying throughout the ground control data collection process. The survey was conducted using a 5-second epoch rate with each observation lasting at least 30 minutes. Each station was occupied twice to ensure the required horizontal and vertical accuracies were met.

### Real-time Kinematic (RTK) GPS

The field crew utilized Real-Time Kinematic (RTK) GPS surveying throughout the ground control data collection process. The survey was conducted using a 1-second epoch rate, in a fixed solution RTK mode, with each observation lasting between 60 and 180 seconds. Each station was occupied twice to ensure the required horizontal and vertical accuracies were met.

### GPS Data Analysis and Processing

Woolpert processed all session baselines each day using Trimble Navigation’s Trimble Business Center (TBC) software, version 5.60 baseline processor, with the accompanying broadcast ephemeris. Daily processing ensured the integrity of the network as it was constructed and allowing the field crews to immediately reschedule observations of any poor baselines.

### Datum Reference and Final Coordinates

The spatial reference system for the project is NAD83 2011 (2010.00 epoch), Ohio State Plane North and/or Ohio State Plane South as appropriate, US survey feet horizontal and NAVD88 US feet survey feet vertical using GEOID18.

### Quality Assurance

Existing NGS published benchmarks were surveyed to assure that there were no discrepancies in the field observation data. Close examinations of the residuals showed no distortions in orientation or scale. Ground control data meets positional accuracies necessary to support 1.0-point per 0.3-meters squared (1-foot Ground Sampling Distance (GSD)) data at 95% confidence level as outlined in the Federal Geographic Data Committee “Geospatial Positioning Accuracy Standards, Part 3: National Standard for Spatial Data Accuracy (NSSDA)”, (FGDC-STD-007.3-1998).

# Ground Control/Geodetic Control Coordinates

## Geodetic Control

- Horizontal Datum: NAD 1983 (Conus) 2011

- Horizontal Projection: Ohio North & South

- Vertical Datum: NAVD88

- Geoid Model: Geoid18 (Conus)

- Units: US Survey Feet

Table 2.1 Geodetic Control -Ohio North

| Point Number | Northing (UsFT) | Easting (UsFT) | Orthometric Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| 4G4 A | 476976.14 | 2473970.08 | 1074.50 | AB6029 |
| CADIZ | 211690.58 | 2383782.84 | 1134.30 | KY3525 |
| CARPORT | 329307.42 | 2363466.31 | 1161.81 | KY3529 |
| CAUSEWAY | 310380.84 | 1973068.65 | 1102.80 | AB5587 |
| D 184 | 118236.77 | 2136809.09 | 815.26 | JX1427 |
| PHILPORT | 294619.81 | 2269369.60 | 888.72 | KY3547 |
| SAVAGE | 385225.49 | 2122569.14 | 1043.18 | KY1597 |
| Y 32 | 237000.96 | 2125060.49 | 780.69 | KY1491 |
| Y 337 | 433490.27 | 2248086.98 | 1077.87 | KY2913 |
| YODER | 350680.47 | 2110096.30 | 1197.84 | KY0956 |

Table 2.1 Geodetic Control -Ohio South

| Point Number | Northing (UsFT) | Easting (UsFT) | Orthometric Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| B 27 | 723713.72 | 1986530.54 | 911.79 | JY0005 |
| COM 1 | 515314.40 | 2263021.81 | 607.70 | JX2283 |
| EIGHTEEN AZ MK | 219216.33 | 2062169.82 | 555.15 | HY2186 |
| F 200 | 309751.91 | 2027135.23 | 698.96 | HY0076 |
| FAIRFIELD COUNTY 7 | 680253.13 | 1944398.48 | 917.10 | AH7726 |
| FORD | 743944.41 | 2207364.99 | 1142.00 | KY1500 |
| G 236 | 550042.63 | 2259259.28 | 645.75 | JX0797 |
| HOC GAR | 559571.83 | 1984309.01 | 771.53 | AB6092 |
| L 233 | 451562.61 | 2092392.26 | 956.15 | JY0252 |
| LANC | 622317.65 | 1944548.68 | 811.18 | DG7198 |
| R 312 | 836699.88 | 2496254.67 | 673.24 | KY2969 |
| RACINE | 336997.45 | 2136959.08 | 597.29 | HX2816 |
| WOODPORT | 651091.42 | 2361709.28 | 1204.11 | JX2260 |

## Ground Control

- Horizontal Datum: NAD 1983 (Conus) 2011

- Horizontal Projection: Ohio North & South

- Vertical Datum: NAVD88

- Geoid Model: Geoid18 (Conus)

- Units: US Survey Feet

Table 2.2 Ground Control -Ohio North

| Point Number | Northing (UsFT) | Easting (UsFT) | Orthometric Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| 2001_2020_OH | 329323.00 | 2256626.53 | 981.29 | NVA |
| 2002_2021_OH | 206772.00 | 2264175.16 | 1029.87 | NVA |
| 2003_2020_OH | 305950.19 | 2296692.54 | 943.74 | NVA |
| 2007_2020_OH | 288234.17 | 2346013.97 | 1234.87 | NVA |
| 2008_2020_OH | 221509.86 | 2217890.19 | 799.63 | NVA |
| 2009_2020_OH | 225291.16 | 2223980.06 | 805.33 | NVA |
| 2012_2021_OH | 242031.78 | 2250858.03 | 815.19 | NVA |
| 2014_2020_OH | 327174.94 | 2247264.75 | 907.14 | NVA |
| 2016_2020_OH | 308058.30 | 2308027.68 | 1136.91 | NVA |
| 2017_2020_OH | 267602.81 | 2218295.70 | 1145.89 | NVA |
| 2018_2020_OH | 258876.07 | 2275902.03 | 881.01 | NVA |
| 2020_2020_OH | 282575.33 | 2266290.09 | 934.31 | NVA |
| 2021_2021_OH | 191841.94 | 2425507.31 | 1055.17 | NVA |
| 2023_2020_OH | 226526.07 | 2360618.02 | 1231.87 | NVA |
| 2025_2021_OH | 321107.36 | 2377940.49 | 1231.59 | NVA |
| 2026_2021_OH | 197429.41 | 2401389.86 | 1074.74 | NVA |
| 2030_2021_OH | 284457.04 | 2448909.57 | 1255.60 | NVA |
| 2031_2021_OH | 205648.68 | 2404986.23 | 1090.06 | NVA |
| 2032_2020_OH | 242851.67 | 2409675.74 | 1278.00 | NVA |
| 2033_2021_OH | 256610.23 | 2453065.67 | 1106.84 | NVA |
| 2035_2020_OH | 226185.14 | 2417158.28 | 1105.84 | NVA |
| 2036_2021_OH | 214843.61 | 2460232.94 | 1191.05 | NVA |
| 2037_2021_OH | 280781.42 | 2469442.82 | 1215.36 | NVA |
| 2039_2020_OH | 463119.73 | 2362340.00 | 1051.25 | NVA |
| 2040_2020_OH | 419635.68 | 2311597.79 | 1112.58 | NVA |
| 2041_2020_OH | 460667.29 | 2352096.20 | 1115.01 | NVA |
| 2042_2020_OH | 377409.69 | 2310269.88 | 1006.08 | NVA |
| 2043_2020_OH | 415266.96 | 2286391.01 | 1039.83 | NVA |
| 2044_2020_OH | 349158.58 | 2253204.97 | 972.48 | NVA |
| 2045_2020_OH | 418993.40 | 2238160.37 | 957.46 | NVA |
| 2046_2020_OH | 396698.65 | 2296307.11 | 1264.12 | NVA |
| 2047_2020_OH | 411141.63 | 2271616.40 | 1071.04 | NVA |
| 2048_2020_OH | 428565.54 | 2244497.83 | 1084.73 | NVA |
| 2049_2020_OH | 429624.72 | 2266205.73 | 1071.42 | NVA |
| 2050_2020_OH | 411182.93 | 2235902.28 | 988.83 | NVA |
| 2051_2020_OH | 475141.51 | 2320116.06 | 1117.32 | NVA |
| 2052_2020_OH | 404379.17 | 2336294.26 | 1206.19 | NVA |
| 2053_2020_OH | 400284.25 | 2326831.35 | 1071.56 | NVA |
| 2054_2020_OH | 417306.38 | 2261236.31 | 1138.68 | NVA |
| 2055_2020_OH | 447642.78 | 2272353.33 | 1147.32 | NVA |
| 2056_2020_OH | 447359.41 | 2239351.29 | 1093.94 | NVA |
| 2057_2020_OH | 523482.07 | 2467018.49 | 1019.18 | NVA |
| 2058_2020_OH | 417338.59 | 2501057.45 | 933.59 | NVA |
| 2059_2020_OH | 484997.46 | 2391663.87 | 1076.52 | NVA |
| 2060_2020_OH | 465613.62 | 2403200.75 | 1257.53 | NVA |
| 2061_2020_OH | 493164.79 | 2478388.39 | 1126.53 | NVA |
| 2062_2020_OH | 391218.81 | 2394498.01 | 1125.18 | NVA |
| 2063_2020_OH | 528453.96 | 2467206.78 | 1022.20 | NVA |
| 2064_2020_OH | 401152.00 | 2394180.67 | 1286.94 | NVA |
| 2065_2021_OH | 329740.09 | 2385662.59 | 1094.28 | NVA |
| 2066_2020_OH | 389519.69 | 2475526.82 | 1035.79 | NVA |
| 2067_2020_OH | 528914.38 | 2486416.25 | 954.16 | NVA |
| 2068_2020_OH | 374800.83 | 2443695.38 | 1089.70 | NVA |
| 2069_2020_OH | 501528.96 | 2470399.25 | 1046.78 | NVA |
| 2070_2020_OH | 422614.56 | 2452080.69 | 1208.62 | NVA |
| 2071_2020_OH | 520896.60 | 2476352.29 | 1045.69 | NVA |
| 2072_2020_OH | 462175.26 | 2478994.75 | 1136.29 | NVA |
| 2073_2020_OH | 485150.54 | 2401866.34 | 1192.77 | NVA |
| 2074_2020_OH | 329516.43 | 2478387.77 | 933.87 | NVA |
| 2075_2020_OH | 396396.00 | 2460830.93 | 1126.78 | NVA |
| 2076_2020_OH | 527154.37 | 2477835.18 | 858.60 | NVA |
| 2077_2020_OH | 437007.59 | 2495506.40 | 1114.61 | NVA |
| 2078_2021_OH | 269298.13 | 1987536.20 | 1166.70 | NVA |
| 2079_2021_OH | 239077.23 | 1988191.31 | 1138.68 | NVA |
| 2082_2021_OH | 315723.53 | 2023066.98 | 1048.05 | NVA |
| 2083_2021_OH | 313019.63 | 1941895.35 | 1151.60 | NVA |
| 2086_2021_OH | 236723.56 | 1994709.99 | 1029.31 | NVA |
| 2087_2021_OH | 288583.25 | 2014107.26 | 1009.51 | NVA |
| 2089_2021_OH | 240669.88 | 1979336.50 | 1139.86 | NVA |
| 2090_2020_OH | 198808.47 | 2186511.77 | 800.83 | NVA |
| 2091_2020_OH | 269865.28 | 2112216.09 | 974.00 | NVA |
| 2092_2021_OH | 185592.78 | 2123373.82 | 795.50 | NVA |
| 2093_2021_OH | 213583.38 | 2148665.29 | 774.80 | NVA |
| 2094_2020_OH | 250280.22 | 2056879.91 | 933.09 | NVA |
| 2096_2021_OH | 229451.62 | 2076602.47 | 1042.76 | NVA |
| 2097_2020_OH | 300636.96 | 2187494.42 | 1154.88 | NVA |
| 2098_2021_OH | 202324.31 | 2117615.77 | 965.94 | NVA |
| 2099_2021_OH | 213708.31 | 2144362.66 | 749.06 | NVA |
| 2101_2020_OH | 220817.95 | 2175985.86 | 802.19 | NVA |
| 2103_2021_OH | 297023.42 | 2063459.76 | 1183.81 | NVA |
| 2104_2020_OH | 415171.69 | 2088349.98 | 1215.47 | NVA |
| 2105_2020_OH | 455489.08 | 2124268.89 | 1149.65 | NVA |
| 2106_2020_OH | 376129.11 | 2078283.03 | 1049.74 | NVA |
| 2107_2020_OH | 450453.83 | 2135050.91 | 1147.10 | NVA |
| 2108_2020_OH | 351835.00 | 2097897.63 | 1132.56 | NVA |
| 2109_2020_OH | 397137.55 | 2126302.13 | 916.57 | NVA |
| 2110_2020_OH | 348661.05 | 2108996.49 | 1151.02 | NVA |
| 2111_2020_OH | 413091.58 | 2100553.89 | 1132.56 | NVA |
| 2112_2020_OH | 341074.83 | 2137043.70 | 960.41 | NVA |
| 2113_2020_OH | 328245.64 | 2195845.66 | 978.00 | NVA |
| 2114_2020_OH | 471919.65 | 2138134.62 | 989.11 | NVA |
| 2115_2020_OH | 319984.38 | 2184249.91 | 1162.08 | NVA |
| 2116_2020_OH | 335909.88 | 2171997.08 | 1286.15 | NVA |
| 2117_2020_OH | 423125.04 | 2129787.17 | 1062.56 | NVA |
| 2210_2020_OH | 413530.06 | 2123007.35 | 923.65 | NVA |
| 2211_2020_OH | 428147.58 | 2290788.93 | 1100.26 | NVA |
| 2212_2020_OH | 387052.88 | 2240894.78 | 971.19 | NVA |
| 2216_2020_OH | 501905.06 | 2483534.56 | 1068.79 | NVA |
| 2217_2021_OH | 257557.85 | 1966138.13 | 1031.00 | NVA |
| 2219_2020_OH | 335173.65 | 2361816.07 | 1147.70 | NVA |
| 2223_2021_OH | 200439.21 | 2313941.91 | 987.81 | NVA |
| 2224_2020_OH | 226675.15 | 2385926.43 | 1216.28 | NVA |
| 2229_2020_OH | 302471.22 | 2111923.90 | 809.61 | NVA |
| 3002_2020_OH | 315513.30 | 2116776.84 | 1178.39 | VVA |
| 3003_2021_OH | 292610.30 | 2448663.56 | 905.14 | VVA |
| 3008_2021_OH | 292349.55 | 2459850.65 | 1238.84 | VVA |
| 3013_2020_OH | 285479.17 | 2084514.16 | 1161.69 | VVA |
| 3020_2020_OH | 466484.21 | 2485898.60 | 1183.85 | VVA |
| 3021_2020_OH | 418382.49 | 2497525.86 | 967.73 | VVA |
| 3024_2021_OH | 291757.15 | 2471609.86 | 1070.61 | VVA |
| 3025_2020_OH | 415612.90 | 2135432.31 | 942.75 | VVA |
| 3027_2021_OH | 243119.61 | 2448877.05 | 1195.19 | VVA |
| 3028_2020_OH | 406702.71 | 2453305.11 | 958.17 | VVA |
| 3030_2021_OH | 255568.19 | 2420333.50 | 1139.24 | VVA |
| 3034_2020_OH | 192611.17 | 2197951.36 | 802.62 | VVA |
| 3035_2020_OH | 474524.06 | 2379502.09 | 1077.39 | VVA |
| 3037_2021_OH | 257792.21 | 2011939.26 | 938.22 | VVA |
| 3038_2020_OH | 323695.58 | 2081847.09 | 1307.33 | VVA |
| 3039_2020_OH | 320781.72 | 2142036.03 | 1004.87 | VVA |
| 3044_2020_OH | 251203.26 | 2389625.66 | 1226.40 | VVA |
| 3046_2020_OH | 265351.32 | 2030718.06 | 875.86 | VVA |
| 3049_2020_OH | 303944.55 | 2067972.54 | 1012.46 | VVA |
| 3050_2020_OH | 341831.18 | 2254502.35 | 1134.11 | VVA |
| 3052_2021_OH | 234765.42 | 2260880.52 | 966.41 | VVA |
| 3056_2021_OH | 219405.98 | 2299983.84 | 1118.52 | VVA |
| 3061_2021_OH | 197406.00 | 2304083.11 | 885.32 | VVA |
| 3062_2021_OH | 199569.62 | 2466587.26 | 821.07 | VVA |
| 3064_2021_OH | 348518.10 | 2414203.95 | 1043.08 | VVA |
| 3065_2020_OH | 371313.96 | 2200408.53 | 1157.50 | VVA |
| 3067_2020_OH | 294146.95 | 2316404.88 | 983.56 | VVA |
| 3074_2020_OH | 453251.06 | 2501819.80 | 1050.46 | VVA |
| 3075_2020_OH | 267041.26 | 2342424.10 | 1237.29 | VVA |
| 3077_2020_OH | 396011.17 | 2177689.50 | 1172.89 | VVA |
| 3078_2020_OH | 424794.69 | 2401842.42 | 1264.55 | VVA |
| 3081_2020_OH | 264146.03 | 2374113.03 | 989.63 | VVA |
| 3082_2020_OH | 292624.30 | 2215662.87 | 1005.12 | VVA |
| 3084_2021_OH | 855409.907 | 2466172.347 | 773.547 | VVA |
| 3087_2020_OH | 243955.91 | 2013130.18 | 1075.27 | VVA |
| 3088_2020_OH | 351030.67 | 2214308.21 | 1158.33 | VVA |
| 3089_2021_OH | 315846.39 | 1959501.74 | 1176.17 | VVA |
| 3090_2020_OH | 440532.00 | 2211139.01 | 1019.36 | VVA |
| 3098_2020_OH | 297105.40 | 2178215.93 | 1110.49 | VVA |
| 3100_2021_OH | 287220.81 | 1943196.32 | 1084.73 | VVA |
| 3101_2020_OH | 339192.59 | 2096872.96 | 1001.86 | VVA |
| 3103_2020_OH | 409605.52 | 2210933.83 | 1139.89 | VVA |
| 3105_2020_OH | 219874.15 | 2187088.40 | 820.72 | VVA |
| 3108_2020_OH | 451367.23 | 2512050.62 | 1021.37 | VVA |
| 3109_2021_OH | 208929.52 | 2135906.51 | 1035.05 | VVA |
| 3112_2020_OH | 364197.46 | 2215884.56 | 1100.37 | VVA |
| 3114_2020_OH | 449725.81 | 2494278.91 | 1202.09 | VVA |
| 3118_2020_OH | 326427.54 | 2149081.04 | 1134.45 | VVA |
| 3119_2020_OH | 423316.49 | 2206741.61 | 1123.28 | VVA |
| 3121_2020_OH | 265729.99 | 2218970.84 | 1194.51 | VVA |
| 3123_2021_OH | 224831.20 | 2305836.99 | 892.41 | VVA |
| 3125_2021_OH | 277058.07 | 2416765.48 | 1223.93 | VVA |
| 3126_2020_OH | 488487.89 | 2498276.53 | 1114.44 | VVA |
| 3128_2021_OH | 303299.17 | 2390617.50 | 1226.99 | VVA |
| 3132_2020_OH | 306031.18 | 2105965.47 | 813.39 | VVA |
| 3134_2020_OH | 410539.71 | 2349321.43 | 1109.90 | VVA |
| 3135_2020_OH | 231880.79 | 2417954.26 | 1245.08 | VVA |
| 3136_2021_OH | 189982.52 | 2331214.54 | 1158.80 | VVA |
| 3137_2021_OH | 219706.10 | 2308668.17 | 895.57 | VVA |
| 3140_2021_OH | 310068.88 | 1968018.07 | 1108.28 | VVA |
| 3143_2020_OH | 306505.94 | 2092336.80 | 833.19 | VVA |
| 3146_2021_OH | 205390.11 | 2243504.51 | 1136.57 | VVA |
| 3148_2020_OH | 308292.02 | 2317793.57 | 917.08 | VVA |
| 3149_2020_OH | 449987.14 | 2216652.92 | 948.09 | VVA |
| 3155_2020_OH | 423113.24 | 2189721.44 | 1106.54 | VVA |
| 3156_2020_OH | 420287.24 | 2411655.59 | 1219.76 | VVA |
| 3157_2020_OH | 390272.16 | 2182793.40 | 1061.23 | VVA |
| 3160_2020_OH | 320470.44 | 2277144.33 | 1020.27 | VVA |
| 3161_2020_OH | 317262.16 | 2140744.22 | 882.62 | VVA |
| 3162_2020_OH | 311606.93 | 2098535.72 | 1111.94 | VVA |

Table 2.2 Ground Control -Ohio South

| Point Number | Northing (UsFT) | Easting (UsFT) | Orthometric Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| 1001_2021_OH | 307859.55 | 1960320.82 | 654.20 | LCP |
| 1002_2021_OH | 292311.01 | 1973429.06 | 941.26 | LCP |
| 1003_2021_OH | 317648.58 | 1995765.80 | 797.41 | LCP |
| 1004_2021_OH | 355739.26 | 2020127.15 | 801.20 | LCP |
| 1005_2021_OH | 301800.77 | 2061846.89 | 569.98 | LCP |
| 1006_2021_OH | 622592.19 | 1934107.28 | 831.89 | LCP |
| 1007_2021_OH | 624702.14 | 1947388.19 | 840.07 | LCP |
| 1008_2021_OH | 608937.73 | 1869832.70 | 1033.58 | LCP |
| 1009_2021_OH | 626978.65 | 2006148.85 | 808.03 | LCP |
| 1010_2021_OH | 878558.43 | 2499681.33 | 685.42 | LCP |
| 1011_2021_OH | 711607.80 | 1883903.69 | 870.30 | LCP |
| 1012_2021_OH | 792748.30 | 2473864.79 | 662.97 | LCP |
| 1013_2021_OH | 487775.86 | 2234215.43 | 608.87 | LCP |
| 1014_2021_OH | 577070.31 | 2144698.07 | 766.89 | LCP |
| 1015_2021_OH | 514103.52 | 1928721.61 | 675.36 | LCP |
| 1016_2021_OH | 958756.36 | 2128501.61 | 877.96 | LCP |
| 1017_2021_OH | 561133.03 | 1991081.43 | 728.15 | LCP |
| 1018_2021_OH | 450998.46 | 2187307.43 | 665.39 | LCP |
| 1019_2021_OH | 936043.17 | 2128993.19 | 936.87 | LCP |
| 1020_2021_OH | 500713.05 | 2246870.81 | 617.48 | LCP |
| 1021_2021_OH | 935549.07 | 2215044.57 | 1149.49 | LCP |
| 1022_2021_OH | 657401.39 | 2435316.02 | 640.84 | LCP |
| 1023_2021_OH | 709873.23 | 2445005.58 | 1212.28 | LCP |
| 1024_2021_OH | 735664.75 | 2432381.87 | 1164.75 | LCP |
| 1025_2021_OH | 762869.82 | 2419746.07 | 1254.98 | LCP |
| 1026_2021_OH | 796724.90 | 2429239.06 | 1269.57 | LCP |
| 1027_2020_OH | 826268.74 | 2394349.21 | 1024.05 | LCP |
| 1028_2020_OH | 862038.50 | 2400713.73 | 1225.13 | LCP |
| 1029_2020_OH | 896977.81 | 2362790.55 | 1059.52 | LCP |
| 1030_2020_OH | 935420.09 | 2350057.30 | 1056.73 | LCP |
| 1031_2021_OH | 730926.02 | 2346613.22 | 1295.96 | LCP |
| 1032_2021_OH | 772849.34 | 2331006.81 | 1071.09 | LCP |
| 1033_2021_OH | 819637.46 | 2311702.12 | 990.24 | LCP |
| 1034_2020_OH | 869987.28 | 2315070.87 | 870.08 | LCP |
| 1035_2020_OH | 934202.29 | 2464640.01 | 688.19 | LCP |
| 1036_2020_OH | 935262.67 | 2483541.43 | 723.81 | LCP |
| 1037_2021_OH | 768866.68 | 2281365.09 | 842.54 | LCP |
| 1038_2021_OH | 804343.08 | 2291438.90 | 990.29 | LCP |
| 1039_2020_OH | 855707.89 | 2229304.19 | 870.64 | LCP |
| 1040_2021_OH | 860033.40 | 2261888.02 | 824.50 | LCP |
| 1041_2021_OH | 726581.88 | 2254714.82 | 811.92 | LCP |
| 1042_2021_OH | 787167.78 | 2194212.22 | 831.92 | LCP |
| 1043_2020_OH | 850371.17 | 2184187.23 | 987.12 | LCP |
| 1044_2020_OH | 911615.44 | 2187475.90 | 1076.11 | LCP |
| 1045_2021_OH | 743712.15 | 2197129.93 | 1073.70 | LCP |
| 1046_2021_OH | 809006.44 | 2161569.38 | 1038.05 | LCP |
| 1047_2020_OH | 878194.24 | 2177774.04 | 979.29 | LCP |
| 1048_2021_OH | 711916.79 | 2102815.77 | 732.76 | LCP |
| 1049_2021_OH | 784891.79 | 2138599.32 | 1082.67 | LCP |
| 1050_2021_OH | 786124.07 | 2115563.98 | 730.82 | LCP |
| 1051_2020_OH | 931133.31 | 2102488.55 | 979.19 | LCP |
| 1052_2021_OH | 784338.47 | 2080403.14 | 1035.28 | LCP |
| 1053_2021_OH | 783762.92 | 2049471.57 | 975.08 | LCP |
| 1054_2021_OH | 778057.50 | 2066091.25 | 962.82 | LCP |
| 1055_2021_OH | 719123.46 | 2010571.60 | 973.99 | LCP |
| 1056_2020_OH | 818592.86 | 2013746.45 | 1095.96 | LCP |
| 1057_2021_OH | 919417.94 | 1975062.81 | 1116.62 | LCP |
| 1058_2020_OH | 789357.87 | 2033342.43 | 940.74 | LCP |
| 1059_2021_OH | 785218.62 | 1916790.70 | 1151.98 | LCP |
| 1060_2021_OH | 765769.39 | 2004164.75 | 1012.39 | LCP |
| 1061_2021_OH | 719306.01 | 1981416.46 | 917.36 | LCP |
| 1062_2021_OH | 746074.70 | 1971733.58 | 948.32 | LCP |
| 1063_2021_OH | 923047.32 | 1939344.24 | 1160.62 | LCP |
| 1064_2021_OH | 544625.43 | 1909530.98 | 771.26 | LCP |
| 1065_2021_OH | 709114.16 | 1896470.52 | 1003.45 | LCP |
| 1066_2021_OH | 571999.46 | 1964569.60 | 829.27 | LCP |
| 1067_2021_OH | 505520.59 | 1954033.28 | 759.33 | LCP |
| 1068_2021_OH | 713102.74 | 1948832.10 | 916.04 | LCP |
| 1069_2021_OH | 545724.51 | 2046273.89 | 705.30 | LCP |
| 1070_2021_OH | 684053.94 | 2033357.98 | 845.05 | LCP |
| 1071_2021_OH | 612949.64 | 2026898.13 | 1003.97 | LCP |
| 1072_2021_OH | 540298.90 | 2012992.06 | 918.03 | LCP |
| 1073_2021_OH | 690206.22 | 2004276.35 | 943.85 | LCP |
| 1074_2021_OH | 690523.34 | 1984508.77 | 977.39 | LCP |
| 1075_2021_OH | 703625.02 | 1880007.23 | 828.46 | LCP |
| 1076_2021_OH | 584119.66 | 2227351.85 | 964.37 | LCP |
| 1077_2021_OH | 688595.28 | 2207872.18 | 834.10 | LCP |
| 1078_2021_OH | 659705.24 | 2188146.39 | 1046.58 | LCP |
| 1079_2021_OH | 685876.88 | 2171884.01 | 770.04 | LCP |
| 1080_2021_OH | 536583.29 | 2168891.62 | 719.73 | LCP |
| 1081_2021_OH | 693558.59 | 2103441.36 | 731.52 | LCP |
| 1082_2021_OH | 656832.94 | 2139352.77 | 679.99 | LCP |
| 1083_2021_OH | 708964.75 | 2132851.63 | 835.07 | LCP |
| 1084_2021_OH | 519702.53 | 2142599.18 | 986.09 | LCP |
| 1085_2021_OH | 517536.12 | 2109858.68 | 996.43 | LCP |
| 1086_2021_OH | 649550.57 | 2083798.19 | 761.70 | LCP |
| 1087_2021_OH | 565421.08 | 2076709.38 | 728.39 | LCP |
| 1088_2021_OH | 509566.29 | 2256591.47 | 719.91 | LCP |
| 1089_2021_OH | 645755.42 | 2276752.92 | 779.39 | LCP |
| 1090_2021_OH | 635573.29 | 2295974.06 | 795.44 | LCP |
| 1091_2021_OH | 643449.19 | 2320588.06 | 1157.78 | LCP |
| 1092_2021_OH | 652714.61 | 2334159.32 | 1155.60 | LCP |
| 1093_2021_OH | 679538.01 | 2350078.92 | 1262.47 | LCP |
| 1094_2021_OH | 679514.15 | 2359495.71 | 1247.29 | LCP |
| 1095_2021_OH | 679241.09 | 2372219.89 | 1232.94 | LCP |
| 1096_2021_OH | 676893.15 | 2394384.97 | 1336.82 | LCP |
| 1097_2021_OH | 662699.81 | 2419671.62 | 1331.84 | LCP |
| 1098_2021_OH | 483555.86 | 2061269.52 | 867.61 | LCP |
| 1099_2021_OH | 353316.32 | 2153973.20 | 731.52 | LCP |
| 1100_2021_OH | 374318.73 | 2101055.73 | 560.71 | LCP |
| 1101_2021_OH | 330167.75 | 2134090.15 | 595.86 | LCP |
| 1102_2021_OH | 452519.24 | 2024943.41 | 757.65 | LCP |
| 1103_2021_OH | 455359.53 | 2167672.23 | 802.76 | LCP |
| 1104_2021_OH | 442619.31 | 2048378.51 | 761.92 | LCP |
| 1105_2021_OH | 376089.65 | 2121036.33 | 752.64 | LCP |
| 1106_2021_OH | 485252.62 | 2200734.59 | 795.05 | LCP |
| 1107_2021_OH | 321338.98 | 2069251.03 | 562.79 | LCP |
| 1108_2021_OH | 218338.54 | 2057800.70 | 561.60 | LCP |
| 1109_2021_OH | 218583.74 | 2044490.32 | 549.89 | LCP |
| 1110_2021_OH | 260487.07 | 2033823.87 | 577.67 | LCP |
| 1111_2021_OH | 316818.77 | 2032238.23 | 650.62 | LCP |
| 1112_2021_OH | 330409.80 | 2026740.95 | 650.99 | LCP |
| 1113_2021_OH | 309688.16 | 2044385.99 | 580.46 | LCP |
| 1114_2021_OH | 371929.92 | 2006818.93 | 703.81 | LCP |
| 1115_2021_OH | 379359.23 | 2011634.53 | 747.28 | LCP |
| 1116_2021_OH | 325266.75 | 1986849.85 | 655.69 | LCP |
| 1117_2021_OH | 539872.14 | 2147741.66 | 964.65 | LCP |
| 1118_2021_OH | 552739.93 | 2153397.49 | 943.31 | LCP |
| 1119_2021_OH | 562525.13 | 2182277.51 | 899.21 | LCP |
| 1120_2021_OH | 577143.04 | 2200660.31 | 681.71 | LCP |
| 1121_2021_OH | 590255.60 | 2141864.18 | 969.98 | LCP |
| 1122_2021_OH | 598371.67 | 2164421.88 | 1002.75 | LCP |
| 1123_2021_OH | 607554.51 | 2152763.98 | 793.44 | LCP |
| 1124_2021_OH | 616150.08 | 2159426.49 | 1034.30 | LCP |
| 1125_2021_OH | 630307.56 | 2135509.86 | 671.86 | LCP |
| 1126_2021_OH | 633632.76 | 2208192.21 | 1119.34 | LCP |
| 1127_2021_OH | 643863.39 | 1994797.22 | 1032.56 | LCP |
| 1128_2021_OH | 637175.44 | 1972175.77 | 1042.94 | LCP |
| 1129_2021_OH | 629192.99 | 1963169.70 | 870.23 | LCP |
| 1130_2021_OH | 640292.17 | 1978257.87 | 1004.53 | LCP |
| 1131_2021_OH | 625815.46 | 1956527.80 | 836.51 | LCP |
| 1132_2021_OH | 615610.83 | 1925416.90 | 985.73 | LCP |
| 1133_2021_OH | 604606.11 | 1900937.87 | 916.71 | LCP |
| 1134_2021_OH | 613622.82 | 1914484.11 | 1048.74 | LCP |
| 1135_2021_OH | 600290.32 | 1887806.36 | 949.52 | LCP |
| 1136_2021_OH | 284084.93 | 1944713.92 | 936.82 | LCP |
| 1137_2021_OH | 351639.65 | 2082762.41 | 561.21 | LCP |
| 1138_2021_OH | 929681.10 | 2131018.74 | 883.28 | LCP |
| 1139_2021_OH | 949557.58 | 2128640.63 | 831.76 | LCP |
| 1140_2021_OH | 969133.47 | 2123332.33 | 860.01 | LCP |
| 1141_2021_OH | 985589.65 | 2122818.62 | 1102.61 | LCP |
| 1142_2021_OH | 1002142.80 | 2131977.06 | 1075.12 | LCP |
| 1143_2021_OH | 1018867.02 | 2128846.33 | 876.30 | LCP |
| 1144_2021_OH | 1035602.40 | 2124669.73 | 1143.95 | LCP |
| 1145_2021_OH | 1052141.59 | 2115015.01 | 1163.45 | LCP |
| 1146_2021_OH | 1069139.90 | 2113444.35 | 1142.99 | LCP |
| 1147_2021_OH | 1089415.43 | 2107496.71 | 961.51 | LCP |
| 1148_2021_OH | 937189.00 | 2367097.58 | 1148.41 | LCP |
| 1149_2021_OH | 952989.54 | 2361743.26 | 1176.45 | LCP |
| 1150_2021_OH | 969012.73 | 2362646.01 | 1244.22 | LCP |
| 1151_2021_OH | 984722.84 | 2374253.11 | 1190.72 | LCP |
| 1152_2021_OH | 1000674.03 | 2359799.22 | 1065.38 | LCP |
| 1153_2021_OH | 1016278.80 | 2448355.27 | 1047.92 | LCP |
| 1154_2021_OH | 1031985.29 | 2441426.28 | 1231.83 | LCP |
| 1155_2021_OH | 1048113.98 | 2456391.94 | 1125.70 | LCP |
| 1156_2021_OH | 1063738.02 | 2464811.32 | 1068.63 | LCP |
| 1157_2021_OH | 1079784.50 | 2453386.30 | 1209.69 | LCP |
| 1158_2021_OH | 1095730.26 | 2448750.37 | 1151.91 | LCP |
| 1159_2021_OH | 1111833.71 | 2447070.02 | 1080.23 | LCP |
| 1160_2021_OH | 1128159.25 | 2446055.70 | 1134.07 | LCP |
| 1161_2021_OH | 1144311.32 | 2445618.98 | 1037.88 | LCP |
| 1162_2021_OH | 741467.71 | 1997669.36 | 941.11 | LCP |
| 1163_2021_OH | 753746.18 | 1961623.01 | 1002.10 | LCP |
| 1164_2021_OH | 719194.11 | 2093058.00 | 722.29 | LCP |
| 1165_2021_OH | 803234.67 | 2151786.66 | 869.68 | LCP |
| 1166_2021_OH | 734648.20 | 2213291.08 | 841.48 | LCP |
| 1167_2021_OH | 849480.17 | 2242375.87 | 815.91 | LCP |
| 1168_2021_OH | 832823.92 | 2384870.12 | 1212.94 | LCP |
| 1169_2021_OH | 743250.10 | 2460969.24 | 684.81 | LCP |
| 2004_2021_OH | 774157.31 | 2309743.63 | 911.32 | NVA |
| 2005_2021_OH | 781716.66 | 2289868.39 | 1136.32 | NVA |
| 2006_2021_OH | 749543.11 | 2297722.19 | 902.36 | NVA |
| 2010_2021_OH | 765649.52 | 2308855.66 | 935.57 | NVA |
| 2011_2021_OH | 795242.50 | 2249090.46 | 818.51 | NVA |
| 2013_2021_OH | 787565.97 | 2260497.18 | 826.55 | NVA |
| 2015_2021_OH | 743929.44 | 2263779.05 | 1004.81 | NVA |
| 2019_2021_OH | 767981.51 | 2336879.01 | 1198.52 | NVA |
| 2022_2021_OH | 757283.25 | 2416063.00 | 1163.25 | NVA |
| 2024_2021_OH | 731511.92 | 2425243.24 | 863.02 | NVA |
| 2027_2021_OH | 739187.38 | 2379057.70 | 1326.09 | NVA |
| 2028_2021_OH | 753048.03 | 2355172.46 | 973.93 | NVA |
| 2029_2021_OH | 734595.06 | 2404343.35 | 1092.25 | NVA |
| 2034_2021_OH | 754689.43 | 2374670.66 | 1202.37 | NVA |
| 2038_2021_OH | 741227.08 | 2389203.57 | 1053.93 | NVA |
| 2080_2021_OH | 776314.88 | 1945813.18 | 1178.82 | NVA |
| 2081_2021_OH | 809599.22 | 1993370.95 | 1032.67 | NVA |
| 2084_2021_OH | 776136.08 | 1966854.92 | 1025.34 | NVA |
| 2085_2020_OH | 804452.41 | 2040296.82 | 1095.51 | NVA |
| 2088_2021_OH | 804032.51 | 1971179.83 | 979.24 | NVA |
| 2095_2021_OH | 759704.78 | 2168456.53 | 806.89 | NVA |
| 2100_2021_OH | 776604.18 | 2107934.93 | 720.04 | NVA |
| 2102_2021_OH | 749473.56 | 2111602.64 | 867.96 | NVA |
| 2118_2021_OH | 496445.46 | 2177244.82 | 865.01 | NVA |
| 2119_2021_OH | 484810.38 | 2174021.96 | 798.24 | NVA |
| 2120_2021_OH | 421401.26 | 2171848.11 | 842.11 | NVA |
| 2121_2021_OH | 431010.99 | 2172213.42 | 710.67 | NVA |
| 2122_2021_OH | 412028.33 | 2144573.07 | 606.77 | NVA |
| 2123_2021_OH | 369854.60 | 2174577.34 | 621.18 | NVA |
| 2124_2021_OH | 471153.56 | 2189184.74 | 845.56 | NVA |
| 2125_2021_OH | 538619.94 | 2167525.57 | 810.37 | NVA |
| 2126_2021_OH | 717597.61 | 2234607.63 | 920.34 | NVA |
| 2127_2021_OH | 696262.60 | 2199139.12 | 877.55 | NVA |
| 2128_2021_OH | 581820.91 | 2285998.75 | 657.70 | NVA |
| 2129_2021_OH | 652047.21 | 2284259.42 | 1140.97 | NVA |
| 2130_2021_OH | 718674.77 | 2247747.20 | 1027.33 | NVA |
| 2131_2021_OH | 636107.44 | 2247322.66 | 729.46 | NVA |
| 2132_2021_OH | 538110.21 | 2240746.84 | 1019.03 | NVA |
| 2133_2021_OH | 716236.81 | 2304964.26 | 1032.15 | NVA |
| 2134_2021_OH | 554293.66 | 2161246.85 | 946.42 | NVA |
| 2135_2021_OH | 719370.03 | 2221348.23 | 1014.68 | NVA |
| 2136_2021_OH | 586655.87 | 2296352.08 | 889.07 | NVA |
| 2137_2021_OH | 619843.99 | 2285985.24 | 1088.84 | NVA |
| 2138_2021_OH | 608142.13 | 2204419.52 | 928.11 | NVA |
| 2139_2021_OH | 622614.67 | 2203848.76 | 973.77 | NVA |
| 2140_2021_OH | 618108.58 | 2217870.06 | 951.88 | NVA |
| 2141_2021_OH | 649566.51 | 2408194.55 | 686.23 | NVA |
| 2142_2021_OH | 611057.42 | 2308118.79 | 761.37 | NVA |
| 2143_2021_OH | 651667.80 | 2315646.15 | 966.11 | NVA |
| 2144_2021_OH | 687426.86 | 2416399.17 | 1267.03 | NVA |
| 2145_2021_OH | 700745.02 | 2331558.45 | 1323.34 | NVA |
| 2146_2021_OH | 654137.92 | 2368713.53 | 992.11 | NVA |
| 2147_2021_OH | 614783.29 | 2342166.25 | 1131.90 | NVA |
| 2148_2021_OH | 573474.41 | 2358000.88 | 813.81 | NVA |
| 2149_2021_OH | 642258.26 | 2349149.14 | 927.87 | NVA |
| 2150_2021_OH | 539906.35 | 2340106.70 | 1145.16 | NVA |
| 2151_2021_OH | 641819.65 | 2381840.43 | 1188.14 | NVA |
| 2152_2021_OH | 659116.78 | 2352648.16 | 1218.87 | NVA |
| 2153_2021_OH | 569207.90 | 2375888.33 | 734.93 | NVA |
| 2154_2021_OH | 533097.30 | 1986586.50 | 786.16 | NVA |
| 2155_2021_OH | 510580.32 | 1971855.75 | 1029.50 | NVA |
| 2156_2021_OH | 505692.86 | 2070110.99 | 661.17 | NVA |
| 2157_2021_OH | 417502.54 | 2031156.55 | 832.02 | NVA |
| 2158_2021_OH | 460746.58 | 2107497.50 | 943.31 | NVA |
| 2159_2021_OH | 383442.71 | 2074501.74 | 584.71 | NVA |
| 2160_2021_OH | 448509.86 | 2115130.13 | 911.64 | NVA |
| 2161_2021_OH | 331218.63 | 2044476.78 | 636.36 | NVA |
| 2162_2021_OH | 366792.30 | 2122700.12 | 765.30 | NVA |
| 2163_2021_OH | 410283.32 | 2132130.97 | 641.23 | NVA |
| 2164_2021_OH | 517483.46 | 2101448.70 | 862.48 | NVA |
| 2165_2021_OH | 390579.02 | 2022091.88 | 688.34 | NVA |
| 2166_2021_OH | 507331.79 | 2076153.27 | 655.43 | NVA |
| 2167_2021_OH | 475261.29 | 2080811.34 | 678.16 | NVA |
| 2168_2021_OH | 470187.02 | 2116112.53 | 668.42 | NVA |
| 2169_2021_OH | 693292.16 | 1972544.83 | 953.53 | NVA |
| 2170_2021_OH | 741138.98 | 1934792.60 | 1171.44 | NVA |
| 2171_2021_OH | 630806.96 | 1950620.22 | 891.28 | NVA |
| 2172_2021_OH | 588154.03 | 1899387.49 | 917.38 | NVA |
| 2173_2021_OH | 754667.84 | 1989532.15 | 856.47 | NVA |
| 2174_2021_OH | 637376.74 | 1902106.62 | 1091.16 | NVA |
| 2175_2021_OH | 542951.68 | 1986496.60 | 761.98 | NVA |
| 2176_2021_OH | 689282.90 | 1972549.06 | 1019.54 | NVA |
| 2177_2021_OH | 666015.33 | 2017219.59 | 1071.74 | NVA |
| 2178_2021_OH | 745216.56 | 1991315.73 | 851.23 | NVA |
| 2179_2021_OH | 572892.89 | 1901819.88 | 1115.65 | NVA |
| 2180_2021_OH | 569061.01 | 1927493.07 | 1163.60 | NVA |
| 2181_2021_OH | 608660.12 | 1939805.53 | 1098.33 | NVA |
| 2182_2021_OH | 670280.04 | 1889064.52 | 783.87 | NVA |
| 2183_2021_OH | 591902.06 | 1954981.75 | 779.13 | NVA |
| 2184_2021_OH | 647012.87 | 1988330.79 | 1017.45 | NVA |
| 2185_2021_OH | 745375.11 | 1998231.22 | 881.81 | NVA |
| 2186_2021_OH | 647196.11 | 1938563.20 | 993.11 | NVA |
| 2187_2021_OH | 653834.12 | 1901977.94 | 928.47 | NVA |
| 2188_2021_OH | 623590.46 | 1931169.15 | 930.01 | NVA |
| 2189_2021_OH | 531270.94 | 2043762.42 | 674.94 | NVA |
| 2190_2021_OH | 627257.05 | 2051857.62 | 874.05 | NVA |
| 2191_2021_OH | 532549.31 | 2064781.48 | 909.59 | NVA |
| 2192_2021_OH | 545528.68 | 2014169.18 | 708.91 | NVA |
| 2193_2021_OH | 556543.98 | 2068062.34 | 984.64 | NVA |
| 2194_2021_OH | 562269.38 | 2108766.23 | 1066.40 | NVA |
| 2195_2021_OH | 624116.76 | 2152006.53 | 836.19 | NVA |
| 2196_2021_OH | 610582.97 | 2143008.16 | 689.42 | NVA |
| 2197_2021_OH | 600866.27 | 2100020.91 | 933.62 | NVA |
| 2198_2021_OH | 615011.29 | 2116896.89 | 980.71 | NVA |
| 2199_2021_OH | 724355.47 | 2102801.09 | 839.67 | NVA |
| 2200_2021_OH | 659329.26 | 2040200.48 | 1044.20 | NVA |
| 2201_2021_OH | 675291.48 | 2066354.84 | 895.17 | NVA |
| 2202_2021_OH | 683265.89 | 2065333.31 | 988.90 | NVA |
| 2203_2021_OH | 697369.77 | 2059944.04 | 855.53 | NVA |
| 2204_2021_OH | 729791.20 | 2142487.51 | 913.77 | NVA |
| 2205_2021_OH | 710933.31 | 2144102.46 | 720.95 | NVA |
| 2206_2021_OH | 668284.46 | 2099416.13 | 955.46 | NVA |
| 2207_2021_OH | 637756.93 | 2147994.32 | 921.97 | NVA |
| 2208_2021_OH | 704384.03 | 2140808.18 | 905.68 | NVA |
| 2209_2021_OH | 695271.13 | 2176669.35 | 1042.11 | NVA |
| 2213_2021_OH | 738260.72 | 2223864.70 | 890.42 | NVA |
| 2214_2021_OH | 748532.94 | 2226401.24 | 803.94 | NVA |
| 2215_2021_OH | 560867.67 | 1995199.57 | 739.49 | NVA |
| 2218_2021_OH | 727028.00 | 1919703.99 | 1002.42 | NVA |
| 2220_2021_OH | 487137.63 | 2089569.95 | 638.67 | NVA |
| 2221_2021_OH | 784335.40 | 1916916.17 | 1158.25 | NVA |
| 2222_2021_OH | 727382.16 | 2182830.72 | 888.77 | NVA |
| 2225_2021_OH | 672312.48 | 1939609.33 | 860.08 | NVA |
| 2226_2021_OH | 760288.04 | 2398038.61 | 1215.30 | NVA |
| 2227_2021_OH | 756958.91 | 2044773.55 | 895.89 | NVA |
| 2228_2021_OH | 547260.35 | 2085684.67 | 686.05 | NVA |
| 2231_2021_OH | 578324.06 | 1888074.80 | 987.00 | NVA |
| 2233_2021_OH | 633120.08 | 2088986.41 | 827.22 | NVA |
| 2235_2021_OH | 630146.54 | 2135778.25 | 671.16 | NVA |
| 2236_2021_OH | 614738.17 | 1897416.39 | 936.62 | NVA |
| 2237_2021_OH | 568785.29 | 2131725.79 | 759.02 | NVA |
| 2238_2021_OH | 617039.32 | 1924164.08 | 946.50 | NVA |
| 2240_2021_OH | 539476.79 | 2157445.30 | 977.15 | NVA |
| 2241_2021_OH | 598805.95 | 1952634.11 | 788.58 | NVA |
| 2242_2021_OH | 567692.59 | 2110786.13 | 1109.46 | NVA |
| 2244_2021_OH | 554551.09 | 2119916.07 | 716.13 | NVA |
| 2245_2021_OH | 634761.99 | 1886769.39 | 972.80 | NVA |
| 2247_2021_OH | 608124.86 | 2169342.41 | 971.77 | NVA |
| 2249_2021_OH | 587839.55 | 2151723.97 | 977.21 | NVA |
| 2252_2021_OH | 613732.09 | 2104939.52 | 1010.00 | NVA |
| 2253_2021_OH | 678259.17 | 1900604.25 | 857.75 | NVA |
| 2254_2021_OH | 600779.08 | 1876661.58 | 1099.65 | NVA |
| 2256_2021_OH | 650366.63 | 1886575.69 | 1059.88 | NVA |
| 2257_2021_OH | 545841.32 | 2096771.68 | 907.86 | NVA |
| 2258_2021_OH | 331242.65 | 2056454.84 | 664.47 | NVA |
| 2259_2021_OH | 584815.38 | 2201948.52 | 910.67 | NVA |
| 2260_2021_OH | 362192.74 | 2064813.33 | 645.38 | NVA |
| 2261_2021_OH | 595533.93 | 2124833.12 | 1027.21 | NVA |
| 2262_2021_OH | 355202.63 | 2049875.60 | 664.65 | NVA |
| 2263_2021_OH | 656218.32 | 1927616.96 | 875.68 | NVA |
| 2264_2021_OH | 346519.27 | 2080528.85 | 572.06 | NVA |
| 2265_2021_OH | 597563.01 | 2106253.28 | 1015.50 | NVA |
| 2266_2021_OH | 349722.51 | 2053280.44 | 914.49 | NVA |
| 2267_2021_OH | 546827.37 | 2120749.17 | 691.76 | NVA |
| 2268_2021_OH | 305160.39 | 2064076.72 | 570.96 | NVA |
| 2270_2021_OH | 580594.45 | 2175121.88 | 942.99 | NVA |
| 2272_2021_OH | 616791.23 | 1933744.33 | 955.14 | NVA |
| 2273_2021_OH | 584332.58 | 1918989.65 | 1073.77 | NVA |
| 2274_2021_OH | 681264.13 | 1915014.26 | 881.50 | NVA |
| 2276_2021_OH | 632696.08 | 1932722.50 | 874.41 | NVA |
| 2279_2021_OH | 600951.97 | 2151735.78 | 700.04 | NVA |
| 2281_2021_OH | 689326.67 | 1959218.11 | 940.65 | NVA |
| 2283_2021_OH | 624370.90 | 1940656.60 | 886.94 | NVA |
| 2284_2021_OH | 348986.55 | 2026078.75 | 809.39 | NVA |
| 2286_2021_OH | 619787.81 | 2184820.27 | 998.58 | NVA |
| 2288_2021_OH | 636337.50 | 1918079.74 | 864.04 | NVA |
| 2289_2021_OH | 673211.41 | 1976281.50 | 1025.55 | NVA |
| 2291_2021_OH | 631973.20 | 2208037.00 | 1062.34 | NVA |
| 2292_2021_OH | 703337.13 | 1887190.98 | 830.22 | NVA |
| 2293_2021_OH | 564702.78 | 2168682.79 | 663.25 | NVA |
| 2294_2021_OH | 603107.73 | 2158982.08 | 1046.11 | NVA |
| 2296_2021_OH | 617207.05 | 2165359.31 | 709.85 | NVA |
| 2297_2021_OH | 576751.07 | 2112048.02 | 1083.09 | NVA |
| 2299_2021_OH | 639928.51 | 1977713.24 | 1017.83 | NVA |
| 2300_2021_OH | 334164.04 | 2031211.86 | 751.10 | NVA |
| 2301_2021_OH | 611607.72 | 1991343.92 | 796.02 | NVA |
| 3001_2021_OH | 495253.35 | 2130284.27 | 838.53 | VVA |
| 3004_2021_OH | 716243.58 | 2248605.27 | 876.21 | VVA |
| 3005_2021_OH | 519454.33 | 2021897.95 | 851.89 | VVA |
| 3006_2021_OH | 434022.91 | 2048857.74 | 743.07 | VVA |
| 3007_2021_OH | 616146.20 | 2376529.69 | 1042.95 | VVA |
| 3009_2021_OH | 616258.48 | 2248273.98 | 798.55 | VVA |
| 3010_2021_OH | 551513.26 | 2306117.62 | 685.93 | VVA |
| 3011_2021_OH | 531085.78 | 2080391.04 | 667.76 | VVA |
| 3012_2021_OH | 543058.76 | 2115761.31 | 995.12 | VVA |
| 3014_2021_OH | 610787.38 | 2032086.00 | 1117.83 | VVA |
| 3015_2021_OH | 619710.24 | 2120179.09 | 979.61 | VVA |
| 3016_2021_OH | 595670.76 | 1922956.56 | 1033.04 | VVA |
| 3017_2021_OH | 725289.94 | 2069279.56 | 762.11 | VVA |
| 3018_2021_OH | 521127.10 | 2061418.34 | 932.50 | VVA |
| 3019_2021_OH | 573427.39 | 2259058.06 | 1084.15 | VVA |
| 3022_2021_OH | 509588.11 | 2004198.85 | 772.14 | VVA |
| 3023_2021_OH | 582995.91 | 2161632.43 | 681.25 | VVA |
| 3026_2021_OH | 786261.17 | 2015752.91 | 1060.04 | VVA |
| 3029_2021_OH | 779466.23 | 2201657.76 | 951.15 | VVA |
| 3031_2021_OH | 740095.21 | 2442051.41 | 718.27 | VVA |
| 3032_2021_OH | 544424.41 | 2154476.42 | 894.35 | VVA |
| 3033_2021_OH | 379021.99 | 2128710.00 | 699.96 | VVA |
| 3036_2021_OH | 571215.00 | 1974657.74 | 765.28 | VVA |
| 3040_2021_OH | 533340.61 | 2167821.36 | 850.40 | VVA |
| 3041_2021_OH | 763218.34 | 2134325.52 | 999.19 | VVA |
| 3042_2021_OH | 501269.57 | 2218059.18 | 727.85 | VVA |
| 3043_2021_OH | 492022.07 | 2064694.23 | 852.47 | VVA |
| 3045_2021_OH | 636984.34 | 2213014.51 | 1042.25 | VVA |
| 3047_2021_OH | 573800.40 | 2341326.05 | 701.11 | VVA |
| 3048_2021_OH | 793807.01 | 2038865.96 | 940.35 | VVA |
| 3051_2021_OH | 585658.08 | 2331837.64 | 758.45 | VVA |
| 3053_2021_OH | 741248.29 | 2325592.52 | 1266.86 | VVA |
| 3054_2021_OH | 708815.90 | 2221968.48 | 888.98 | VVA |
| 3055_2021_OH | 789187.59 | 2297665.15 | 938.16 | VVA |
| 3057_2021_OH | 585043.56 | 2038727.44 | 745.37 | VVA |
| 3058_2021_OH | 608682.39 | 2308974.29 | 1034.30 | VVA |
| 3059_2021_OH | 689509.60 | 2313115.51 | 1211.76 | VVA |
| 3060_2021_OH | 651855.86 | 2270726.59 | 892.19 | VVA |
| 3063_2021_OH | 575870.32 | 1955550.60 | 1039.63 | VVA |
| 3066_2021_OH | 549740.30 | 2350044.04 | 1065.37 | VVA |
| 3068_2021_OH | 562885.00 | 1990404.02 | 748.42 | VVA |
| 3069_2021_OH | 723514.52 | 2260840.46 | 823.08 | VVA |
| 3070_2021_OH | 605793.51 | 2249440.49 | 1097.54 | VVA |
| 3071_2021_OH | 451495.16 | 2051649.79 | 761.74 | VVA |
| 3072_2021_OH | 716693.44 | 2192936.98 | 819.47 | VVA |
| 3073_2021_OH | 704127.65 | 2316289.08 | 1009.55 | VVA |
| 3076_2021_OH | 760191.36 | 2298081.35 | 1039.97 | VVA |
| 3079_2021_OH | 783279.00 | 2039086.55 | 931.34 | VVA |
| 3080_2021_OH | 514538.63 | 2187958.23 | 888.72 | VVA |
| 3083_2021_OH | 343205.26 | 2038600.26 | 606.52 | VVA |
| 3085_2021_OH | 645558.51 | 2128989.48 | 994.50 | VVA |
| 3086_2021_OH | 732895.01 | 1945082.93 | 1108.91 | VVA |
| 3091_2021_OH | 715079.50 | 2078670.67 | 977.05 | VVA |
| 3092_2021_OH | 723023.62 | 2000205.14 | 946.55 | VVA |
| 3093_2021_OH | 738629.83 | 2130560.28 | 984.19 | VVA |
| 3094_2021_OH | 747976.86 | 2199698.55 | 988.10 | VVA |
| 3095_2021_OH | 511193.33 | 2187900.53 | 947.01 | VVA |
| 3096_2021_OH | 575745.34 | 2004243.33 | 768.23 | VVA |
| 3097_2021_OH | 499462.30 | 2182412.04 | 768.55 | VVA |
| 3099_2021_OH | 641466.50 | 2412147.29 | 650.00 | VVA |
| 3102_2021_OH | 804289.04 | 1901465.39 | 1137.79 | VVA |
| 3104_2021_OH | 709287.24 | 2254209.26 | 856.01 | VVA |
| 3106_2021_OH | 423140.77 | 2054948.71 | 673.42 | VVA |
| 3107_2021_OH | 742642.71 | 2370291.70 | 1228.79 | VVA |
| 3110_2021_OH | 721340.34 | 2079464.31 | 982.33 | VVA |
| 3111_2021_OH | 649770.04 | 2187509.61 | 1001.09 | VVA |
| 3113_2021_OH | 443256.42 | 2167345.50 | 744.90 | VVA |
| 3115_2021_OH | 601251.75 | 2185489.57 | 855.58 | VVA |
| 3116_2021_OH | 476822.42 | 2221633.51 | 764.68 | VVA |
| 3117_2021_OH | 523433.86 | 1983275.16 | 1031.29 | VVA |
| 3120_2021_OH | 769299.31 | 2039252.14 | 821.50 | VVA |
| 3122_2021_OH | 373345.81 | 2070809.74 | 569.93 | VVA |
| 3124_2021_OH | 666279.74 | 2388818.83 | 1280.29 | VVA |
| 3127_2021_OH | 439841.38 | 2063561.59 | 735.48 | VVA |
| 3129_2021_OH | 439878.56 | 2063513.35 | 736.22 | VVA |
| 3130_2021_OH | 767896.11 | 2196729.82 | 1052.02 | VVA |
| 3131_2021_OH | 664559.58 | 1959091.11 | 901.27 | VVA |
| 3133_2021_OH | 736679.65 | 2370853.88 | 1172.63 | VVA |
| 3138_2021_OH | 719753.28 | 2029191.06 | 1131.08 | VVA |
| 3139_2021_OH | 771446.57 | 2121367.36 | 983.50 | VVA |
| 3141_2021_OH | 520239.97 | 2164077.59 | 901.49 | VVA |
| 3142_2021_OH | 664654.66 | 2326403.37 | 1027.85 | VVA |
| 3144_2021_OH | 585715.59 | 2173519.62 | 992.09 | VVA |
| 3145_2021_OH | 724027.02 | 2091055.06 | 842.97 | VVA |
| 3147_2021_OH | 678518.81 | 2146004.80 | 728.58 | VVA |
| 3150_2021_OH | 574838.51 | 2151031.69 | 1018.12 | VVA |
| 3151_2021_OH | 714881.53 | 2421219.24 | 1020.45 | VVA |
| 3152_2021_OH | 737545.42 | 2044049.18 | 819.63 | VVA |
| 3153_2021_OH | 702737.19 | 2352526.87 | 1249.45 | VVA |
| 3154_2021_OH | 774757.76 | 2232285.89 | 1038.95 | VVA |
| 3158_2021_OH | 641443.20 | 2271783.94 | 1142.16 | VVA |
| 3159_2021_OH | 736354.92 | 1957368.28 | 1012.79 | VVA |
| 3164_2021_OH | 637136.21 | 2149149.39 | 896.64 | VVA |
| 3165_2021_OH | 630293.63 | 2002703.43 | 800.51 | VVA |
| 3167_2021_OH | 613939.37 | 1983554.07 | 807.52 | VVA |
| 3169_2021_OH | 673971.67 | 1911208.06 | 856.37 | VVA |
| 3171_2021_OH | 542970.41 | 2097127.54 | 893.44 | VVA |
| 3172_2021_OH | 628975.96 | 1891034.64 | 960.98 | VVA |
| 3174_2021_OH | 625427.17 | 2128394.44 | 980.56 | VVA |
| 3175_2021_OH | 629120.84 | 1892723.91 | 978.72 | VVA |
| 3177_2021_OH | 687597.47 | 1972365.18 | 1033.42 | VVA |
| 3179_2021_OH | 590759.75 | 2171194.09 | 993.80 | VVA |
| 3180_2021_OH | 611331.81 | 1953465.22 | 798.36 | VVA |
| 3182_2021_OH | 680411.28 | 1965214.38 | 926.00 | VVA |
| 3184_2021_OH | 608826.40 | 2163235.03 | 986.18 | VVA |
| 3185_2021_OH | 668806.13 | 1893034.21 | 767.14 | VVA |
| 3187_2021_OH | 617556.58 | 2168593.38 | 696.87 | VVA |
| 3188_2021_OH | 699372.86 | 1978090.88 | 904.74 | VVA |
| 3190_2021_OH | 627295.30 | 2185206.67 | 1011.27 | VVA |
| 3191_2021_OH | 607136.15 | 1891679.83 | 905.40 | VVA |
| 3194_2021_OH | 559947.83 | 2179731.24 | 669.92 | VVA |
| 3195_2021_OH | 662388.56 | 1890707.76 | 826.90 | VVA |
| 3197_2021_OH | 620099.80 | 1974198.66 | 812.99 | VVA |
| 3199_2021_OH | 611212.25 | 2207885.78 | 810.56 | VVA |
| 3201_2021_OH | 607866.47 | 1922214.92 | 1091.47 | VVA |
| 3203_2021_OH | 589731.72 | 2118641.77 | 1095.24 | VVA |
| 3204_2021_OH | 666872.29 | 1935661.25 | 889.43 | VVA |
| 3205_2021_OH | 349928.60 | 2032197.07 | 750.56 | VVA |
| 3206_2021_OH | 565846.59 | 2129915.93 | 806.44 | VVA |
| 3207_2021_OH | 610979.82 | 1892799.66 | 934.71 | VVA |
| 3208_2021_OH | 335361.34 | 2045388.44 | 590.73 | VVA |
| 3209_2021_OH | 641538.48 | 1972205.79 | 991.84 | VVA |
| 3210_2021_OH | 373603.10 | 2014703.98 | 618.08 | VVA |
| 3211_2021_OH | 623810.07 | 2114833.38 | 994.92 | VVA |
| 3212_2021_OH | 690031.06 | 1910665.32 | 934.36 | VVA |
| 3214_2021_OH | 576890.67 | 2145308.06 | 774.14 | VVA |
| 3215_2021_OH | 630794.86 | 1941128.63 | 972.85 | VVA |
| 3216_2021_OH | 602138.67 | 1939804.50 | 1113.09 | VVA |
| 3217_2021_OH | 681646.05 | 1974759.00 | 967.35 | VVA |
| 3218_2021_OH | 621861.60 | 1999051.08 | 947.32 | VVA |
| 3219_2021_OH | 592767.24 | 1944036.06 | 1058.18 | VVA |

## Geodetic Control- Geodetic Coordinate System NAD83

- Horizontal Datum: NAD 1983 (2011)

- Vertical Datum: NAVD88

- Geoid Model: Geoid18 (Conus)

- Units: US Survey Feet

Table 2.3 Ground Control - Geodetic Coordinate System NAD83

| Point Number | NAD83 (Conus) Latitude (N) | NAD83 (Conus) Longitude (W) | Ellipsoid Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| 4G4 A | N40°57'40.75422" | W80°40'10.68930" | 962.87 | AB6029 |
| B 27 | N39°59'13.10113" | W82°26'08.35558" | 799.80 | JY0005 |
| CADIZ | N40°14'16.74831" | W81°00'45.09116" | 1023.42 | KY3525 |
| CARPORT | N40°33'42.17491" | W81°04'42.29895" | 1051.24 | KY3529 |
| CAUSEWAY | N40°31'06.95465" | W82°29'00.84357" | 992.37 | AB5587 |
| COM 1 | N39°24'36.68947" | W81°27'27.25553" | 495.29 | JX2283 |
| D 184 | N39°59'22.56360" | W81°53'57.84364" | 704.38 | JX1427 |
| EIGHTEEN AZ MK | N38°36'05.06902" | W82°10'20.12239" | 445.39 | HY2186 |
| F 200 | N38°51'00.98658" | W82°17'38.82065" | 588.20 | HY0076 |
| FAIRFIELD COUNTY 7 | N39°52'03.54531" | W82°35'09.10930" | 804.94 | AH7726 |
| FORD | N40°02'21.94067" | W81°38'48.85657" | 1031.04 | KY1500 |
| G 236 | N39°30'20.34378" | W81°28'10.14298" | 533.43 | JX0797 |
| HOC GAR | N39°32'10.87827" | W82°26'38.20547" | 659.62 | AB6092 |
| L 233 | N39°14'20.39664" | W82°03'45.26431" | 843.93 | JY0252 |
| LANC | N39°42'30.96549" | W82°35'06.48243" | 699.07 | DG7198 |
| PHILPORT | N40°28'12.78251" | W81°25'06.96724" | 778.87 | KY3547 |
| R 312 | N40°16'55.05466" | W80°36'30.34158" | 561.52 | KY2969 |
| RACINE | N38°55'25.49640" | W81°54'28.35947" | 485.63 | HX2816 |
| SAVAGE | N40°43'21.69268" | W81°56'38.91037" | 934.67 | KY1597 |
| WOODPORT | N39°46'45.34677" | W81°06'03.16825" | 1093.47 | JX2260 |
| Y 32 | N40°18'56.86967" | W81°56'18.94513" | 670.96 | KY1491 |
| Y 337 | N40°51'07.46150" | W81°29'21.50257" | 969.08 | KY2913 |
| YODER | N40°37'41.09142" | W81°59'23.53119" | 1089.09 | KY0956 |

## Ground Control-Geodetic Coordinate System NAD83

- Horizontal Datum: NAD 1983 (Conus)

- Vertical Datum: NAVD88

- Units: Meters

Table 2.4 Ground Control – Geodetic Coordinate System NAD83

| Point Number | NAD83 (Conus) Latitude (N) | NAD83 (Conus) Longitude (W) | Ellipsoid Height (UsFT) | Description |
| --- | --- | --- | --- | --- |
| 1001_2021_OH | N38°50'42.92935" | W82°31'43.38163" | 544.15 | LCP |
| 1002_2021_OH | N38°48'09.24788" | W82°28'57.73628" | 831.29 | LCP |
| 1003_2021_OH | N38°52'19.55909" | W82°24'15.23996" | 686.77 | LCP |
| 1004_2021_OH | N38°58'35.69576" | W82°19'06.23712" | 689.91 | LCP |
| 1005_2021_OH | N38°49'41.38010" | W82°10'20.41531" | 459.21 | LCP |
| 1006_2021_OH | N39°42'33.56033" | W82°37'20.09579" | 719.82 | LCP |
| 1007_2021_OH | N39°42'54.55699" | W82°34'30.17324" | 727.96 | LCP |
| 1008_2021_OH | N39°40'16.94573" | W82°51'01.88175" | 921.93 | LCP |
| 1009_2021_OH | N39°43'16.86736" | W82°21'58.15504" | 695.96 | LCP |
| 1010_2021_OH | N40°23'47.85622" | W80°35'34.73545" | 573.35 | LCP |
| 1011_2021_OH | N39°57'12.13087" | W82°48'06.31360" | 758.29 | LCP |
| 1012_2021_OH | N40°09'45.36884" | W80°41'30.51555" | 551.61 | LCP |
| 1013_2021_OH | N39°20'07.63817" | W81°33'37.92391" | 496.13 | LCP |
| 1014_2021_OH | N39°34'57.84767" | W81°52'29.40355" | 654.80 | LCP |
| 1015_2021_OH | N39°24'41.21788" | W82°38'26.84717" | 563.78 | LCP |
| 1016_2021_OH | N40°37'50.60053" | W81°55'25.15955" | 769.27 | LCP |
| 1017_2021_OH | N39°32'26.25832" | W82°25'11.74098" | 616.26 | LCP |
| 1018_2021_OH | N39°14'08.53514" | W81°43'38.95008" | 552.49 | LCP |
| 1019_2021_OH | N40°34'06.17402" | W81°55'20.66912" | 828.00 | LCP |
| 1020_2021_OH | N39°22'14.17040" | W81°30'55.06068" | 504.88 | LCP |
| 1021_2021_OH | N40°33'54.39673" | W81°36'45.91962" | 1040.55 | LCP |
| 1022_2021_OH | N39°47'35.37244" | W80°50'19.00850" | 530.60 | LCP |
| 1023_2021_OH | N39°56'12.07241" | W80°48'02.24107" | 1101.52 | LCP |
| 1024_2021_OH | N40°00'29.23095" | W80°50'38.20525" | 1053.84 | LCP |
| 1025_2021_OH | N40°05'00.29390" | W80°53'14.32149" | 1144.03 | LCP |
| 1026_2021_OH | N40°10'33.09828" | W80°51'04.25994" | 1158.44 | LCP |
| 1027_2020_OH | N40°15'31.05257" | W80°58'27.22921" | 913.06 | LCP |
| 1028_2020_OH | N40°21'23.37685" | W80°56'57.23025" | 1114.12 | LCP |
| 1029_2020_OH | N40°27'14.71228" | W81°04'59.90820" | 948.93 | LCP |
| 1030_2020_OH | N40°33'36.42792" | W81°07'37.02468" | 946.36 | LCP |
| 1031_2021_OH | N39°59'56.51356" | W81°09'01.22855" | 1185.15 | LCP |
| 1032_2021_OH | N40°06'53.02294" | W81°12'14.01154" | 960.31 | LCP |
| 1033_2021_OH | N40°14'37.99226" | W81°16'14.26073" | 879.66 | LCP |
| 1034_2020_OH | N40°22'55.00330" | W81°15'21.88093" | 759.74 | LCP |
| 1035_2020_OH | N40°33'04.60130" | W80°42'53.28964" | 576.15 | LCP |
| 1036_2020_OH | N40°33'11.31360" | W80°38'48.23711" | 611.54 | LCP |
| 1037_2021_OH | N40°06'20.22843" | W81°22'53.53680" | 731.64 | LCP |
| 1038_2021_OH | N40°12'09.51826" | W81°20'38.05917" | 879.70 | LCP |
| 1039_2020_OH | N40°20'44.14719" | W81°33'51.91038" | 760.73 | LCP |
| 1040_2021_OH | N40°21'23.34031" | W81°26'50.49960" | 714.42 | LCP |
| 1041_2021_OH | N39°59'25.51024" | W81°28'42.64163" | 700.67 | LCP |
| 1042_2021_OH | N40°09'30.25265" | W81°41'32.98604" | 721.36 | LCP |
| 1043_2020_OH | N40°19'55.63627" | W81°43'35.12085" | 877.31 | LCP |
| 1044_2020_OH | N40°30'00.46632" | W81°42'45.76652" | 967.14 | LCP |
| 1045_2021_OH | N40°02'20.58066" | W81°41'00.47083" | 962.78 | LCP |
| 1046_2021_OH | N40°13'08.72496" | W81°48'31.23157" | 927.81 | LCP |
| 1047_2020_OH | N40°24'31.07788" | W81°44'54.93253" | 869.91 | LCP |
| 1048_2021_OH | N39°57'13.05987" | W82°01'15.22033" | 621.76 | LCP |
| 1049_2021_OH | N40°09'12.07134" | W81°53'29.41317" | 972.21 | LCP |
| 1050_2021_OH | N40°09'25.67834" | W81°58'25.96909" | 620.35 | LCP |
| 1051_2020_OH | N40°33'19.20245" | W82°01'04.39254" | 870.14 | LCP |
| 1052_2021_OH | N40°09'09.81596" | W82°05'58.89912" | 924.76 | LCP |
| 1053_2021_OH | N40°09'05.29628" | W82°12'37.25976" | 864.30 | LCP |
| 1054_2021_OH | N40°08'08.33568" | W82°09'03.51992" | 852.13 | LCP |
| 1055_2021_OH | N39°58'27.45584" | W82°20'59.59050" | 862.26 | LCP |
| 1056_2020_OH | N40°14'50.35502" | W82°20'16.51622" | 985.17 | LCP |
| 1057_2021_OH | N40°31'26.97966" | W82°28'35.02818" | 1006.22 | LCP |
| 1058_2020_OH | N40°10'01.04467" | W82°16'04.78242" | 829.91 | LCP |
| 1059_2021_OH | N40°09'20.44116" | W82°41'05.94466" | 1040.24 | LCP |
| 1060_2021_OH | N40°06'08.52012" | W82°22'21.04039" | 900.91 | LCP |
| 1061_2021_OH | N39°58'29.57300" | W82°27'14.08674" | 805.32 | LCP |
| 1062_2021_OH | N40°02'54.14136" | W82°29'18.42043" | 836.36 | LCP |
| 1063_2021_OH | N40°32'02.67950" | W82°36'17.54804" | 1049.78 | LCP |
| 1064_2021_OH | N39°29'42.52558" | W82°42'32.26917" | 659.61 | LCP |
| 1065_2021_OH | N39°56'47.87155" | W82°45'24.85096" | 891.48 | LCP |
| 1066_2021_OH | N39°34'13.75468" | W82°30'50.19424" | 717.27 | LCP |
| 1067_2021_OH | N39°23'16.65021" | W82°33'04.26985" | 647.80 | LCP |
| 1068_2021_OH | N39°57'28.22604" | W82°34'12.57414" | 804.00 | LCP |
| 1069_2021_OH | N39°29'52.88991" | W82°13'27.79519" | 593.60 | LCP |
| 1070_2021_OH | N39°52'40.40119" | W82°16'08.05352" | 733.33 | LCP |
| 1071_2021_OH | N39°40'57.82661" | W82°17'33.01079" | 892.20 | LCP |
| 1072_2021_OH | N39°29'00.05337" | W82°20'32.50965" | 806.27 | LCP |
| 1073_2021_OH | N39°53'41.77344" | W82°22'20.97738" | 831.80 | LCP |
| 1074_2021_OH | N39°53'45.10711" | W82°26'34.59942" | 865.23 | LCP |
| 1075_2021_OH | N39°55'53.10900" | W82°48'55.98994" | 716.40 | LCP |
| 1076_2021_OH | N39°36'00.53485" | W81°34'52.80334" | 852.01 | LCP |
| 1077_2021_OH | N39°53'14.92828" | W81°38'49.05849" | 722.63 | LCP |
| 1078_2021_OH | N39°48'31.18704" | W81°43'05.31947" | 934.94 | LCP |
| 1079_2021_OH | N39°52'51.17108" | W81°46'31.01441" | 658.72 | LCP |
| 1080_2021_OH | N39°28'15.92728" | W81°47'24.45413" | 607.13 | LCP |
| 1081_2021_OH | N39°54'11.60187" | W82°01'08.44286" | 620.43 | LCP |
| 1082_2021_OH | N39°48'06.51226" | W81°53'30.81643" | 568.60 | LCP |
| 1083_2021_OH | N39°56'42.13576" | W81°54'49.77880" | 724.06 | LCP |
| 1084_2021_OH | N39°25'30.99220" | W81°53'01.21339" | 873.60 | LCP |
| 1085_2021_OH | N39°25'11.58092" | W81°59'58.61441" | 884.20 | LCP |
| 1086_2021_OH | N39°46'57.63718" | W82°05'23.06436" | 650.36 | LCP |
| 1087_2021_OH | N39°33'06.46644" | W82°06'58.44889" | 616.74 | LCP |
| 1088_2021_OH | N39°23'40.60449" | W81°28'50.00954" | 607.43 | LCP |
| 1089_2021_OH | N39°46'04.21841" | W81°24'12.12886" | 667.87 | LCP |
| 1090_2021_OH | N39°44'21.21583" | W81°20'07.66715" | 684.06 | LCP |
| 1091_2021_OH | N39°45'35.79246" | W81°14'51.19855" | 1046.69 | LCP |
| 1092_2021_OH | N39°47'05.45823" | W81°11'55.72047" | 1044.70 | LCP |
| 1093_2021_OH | N39°51'28.20722" | W81°08'26.64031" | 1151.73 | LCP |
| 1094_2021_OH | N39°51'26.55327" | W81°06'25.90325" | 1136.62 | LCP |
| 1095_2021_OH | N39°51'21.88388" | W81°03'42.81228" | 1122.33 | LCP |
| 1096_2021_OH | N39°50'55.09842" | W80°58'59.13153" | 1226.34 | LCP |
| 1097_2021_OH | N39°48'30.52564" | W80°53'38.19906" | 1221.56 | LCP |
| 1098_2021_OH | N39°19'37.92472" | W82°10'19.36968" | 755.59 | LCP |
| 1099_2021_OH | N38°58'05.63727" | W81°50'51.58228" | 619.59 | LCP |
| 1100_2021_OH | N39°01'36.47169" | W82°02'00.22225" | 448.82 | LCP |
| 1101_2021_OH | N38°54'18.17396" | W81°55'05.21803" | 484.32 | LCP |
| 1102_2021_OH | N39°14'32.20260" | W82°18'02.54331" | 645.86 | LCP |
| 1103_2021_OH | N39°14'53.22551" | W81°47'48.07182" | 689.87 | LCP |
| 1104_2021_OH | N39°12'53.73224" | W82°13'05.05248" | 649.92 | LCP |
| 1105_2021_OH | N39°01'52.87846" | W81°57'46.89548" | 640.58 | LCP |
| 1106_2021_OH | N39°19'45.92528" | W81°40'44.34119" | 682.13 | LCP |
| 1107_2021_OH | N38°52'54.22814" | W82°08'45.88567" | 451.71 | LCP |
| 1108_2021_OH | N38°35'56.54569" | W82°11'15.19422" | 451.87 | LCP |
| 1109_2021_OH | N38°35'59.39068" | W82°14'02.83783" | 440.31 | LCP |
| 1110_2021_OH | N38°42'53.87024" | W82°16'15.85195" | 467.68 | LCP |
| 1111_2021_OH | N38°52'10.71872" | W82°16'34.09499" | 539.74 | LCP |
| 1112_2021_OH | N38°54'25.18792" | W82°17'43.21266" | 539.95 | LCP |
| 1113_2021_OH | N38°50'59.91043" | W82°14'00.76577" | 469.65 | LCP |
| 1114_2021_OH | N39°01'15.96203" | W82°21'54.45473" | 592.47 | LCP |
| 1115_2021_OH | N39°02'29.32090" | W82°20'53.27726" | 635.83 | LCP |
| 1116_2021_OH | N38°53'34.93904" | W82°26'07.90811" | 545.06 | LCP |
| 1117_2021_OH | N39°28'49.98964" | W81°51'53.87018" | 852.25 | LCP |
| 1118_2021_OH | N39°30'56.76984" | W81°50'40.54437" | 830.95 | LCP |
| 1119_2021_OH | N39°32'31.24724" | W81°44'30.97765" | 786.69 | LCP |
| 1120_2021_OH | N39°34'54.12903" | W81°40'34.61840" | 569.20 | LCP |
| 1121_2021_OH | N39°37'08.35499" | W81°53'04.45213" | 858.04 | LCP |
| 1122_2021_OH | N39°38'26.94955" | W81°48'15.38244" | 890.64 | LCP |
| 1123_2021_OH | N39°39'58.56674" | W81°50'43.55777" | 681.52 | LCP |
| 1124_2021_OH | N39°41'23.03178" | W81°49'17.53224" | 922.40 | LCP |
| 1125_2021_OH | N39°43'44.61638" | W81°54'22.29026" | 560.31 | LCP |
| 1126_2021_OH | N39°44'11.72700" | W81°38'51.60851" | 1007.37 | LCP |
| 1127_2021_OH | N39°46'03.88409" | W82°24'23.21374" | 920.44 | LCP |
| 1128_2021_OH | N39°44'57.91785" | W82°29'12.93714" | 930.78 | LCP |
| 1129_2021_OH | N39°43'39.02254" | W82°31'08.22536" | 758.06 | LCP |
| 1130_2021_OH | N39°45'28.70517" | W82°27'55.04933" | 892.37 | LCP |
| 1131_2021_OH | N39°43'05.61909" | W82°32'33.21812" | 724.36 | LCP |
| 1132_2021_OH | N39°41'24.43031" | W82°39'11.14795" | 873.74 | LCP |
| 1133_2021_OH | N39°39'35.13989" | W82°44'23.92476" | 804.90 | LCP |
| 1134_2021_OH | N39°41'04.57570" | W82°41'30.95306" | 936.83 | LCP |
| 1135_2021_OH | N39°38'52.10674" | W82°47'11.66270" | 837.82 | LCP |
| 1136_2021_OH | N38°46'47.83321" | W82°35'00.36889" | 827.37 | LCP |
| 1137_2021_OH | N38°57'53.17327" | W82°05'53.31533" | 449.65 | LCP |
| 1138_2021_OH | N40°33'03.18976" | W81°54'54.96073" | 774.35 | LCP |
| 1139_2021_OH | N40°36'19.71312" | W81°55'24.11859" | 723.01 | LCP |
| 1140_2021_OH | N40°39'33.43960" | W81°56'31.35957" | 751.36 | LCP |
| 1141_2021_OH | N40°42'16.04284" | W81°56'36.70812" | 994.05 | LCP |
| 1142_2021_OH | N40°44'58.99395" | W81°54'36.41713" | 966.65 | LCP |
| 1143_2021_OH | N40°47'44.40491" | W81°55'15.69159" | 767.77 | LCP |
| 1144_2021_OH | N40°50'29.98259" | W81°56'08.62143" | 1035.40 | LCP |
| 1145_2021_OH | N40°53'13.93450" | W81°58'12.94024" | 1054.74 | LCP |
| 1146_2021_OH | N40°56'01.92974" | W81°58'32.09882" | 1034.05 | LCP |
| 1147_2021_OH | N40°59'22.53373" | W81°59'48.09320" | 852.11 | LCP |
| 1148_2021_OH | N40°33'51.28532" | W81°03'55.94474" | 1037.85 | LCP |
| 1149_2021_OH | N40°36'28.20691" | W81°05'02.09146" | 1066.02 | LCP |
| 1150_2021_OH | N40°39'06.34934" | W81°04'47.12357" | 1133.83 | LCP |
| 1151_2021_OH | N40°41'39.70406" | W81°02'13.28026" | 1080.18 | LCP |
| 1152_2021_OH | N40°44'19.53993" | W81°05'17.62537" | 955.13 | LCP |
| 1153_2021_OH | N40°46'38.44419" | W80°46'03.78675" | 936.17 | LCP |
| 1154_2021_OH | N40°49'14.87780" | W80°47'29.95315" | 1120.33 | LCP |
| 1155_2021_OH | N40°51'51.33093" | W80°44'11.28912" | 1014.07 | LCP |
| 1156_2021_OH | N40°54'23.99732" | W80°42'17.71904" | 956.98 | LCP |
| 1157_2021_OH | N40°57'04.68849" | W80°44'42.38192" | 1098.35 | LCP |
| 1158_2021_OH | N40°59'43.04911" | W80°45'38.76773" | 1040.70 | LCP |
| 1159_2021_OH | 41d02'22.39582" | W80°45'56.63800" | 969.08 | LCP |
| 1160_2021_OH | 41d05'03.80417" | W80°46'05.78336" | 1022.93 | LCP |
| 1161_2021_OH | 41d07'43.38495" | W80°46'07.44096" | 926.71 | LCP |
| 1162_2021_OH | N40°02'08.44972" | W82°23'44.98848" | 829.34 | LCP |
| 1163_2021_OH | N40°04'09.94519" | W82°31'28.45605" | 890.19 | LCP |
| 1164_2021_OH | N39°58'25.47044" | W82°03'20.05999" | 611.31 | LCP |
| 1165_2021_OH | N40°12'12.41358" | W81°50'37.87220" | 759.38 | LCP |
| 1166_2021_OH | N40°00'49.51689" | W81°37'33.82495" | 730.38 | LCP |
| 1167_2021_OH | N40°19'41.24276" | W81°31'03.98731" | 705.85 | LCP |
| 1168_2021_OH | N40°16'37.38275" | W81°00'28.07467" | 1102.04 | LCP |
| 1169_2021_OH | N40°01'38.83726" | W80°44'28.99314" | 573.66 | LCP |
| 2001_2020_OH | N40°33'57.20969" | W81°27'46.49810" | 871.97 | NVA |
| 2002_2021_OH | N40°13'45.40355" | W81°26'27.97284" | 919.44 | NVA |
| 2003_2020_OH | N40°30'01.23489" | W81°19'11.48589" | 833.85 | NVA |
| 2004_2021_OH | N40°07'08.87454" | W81°16'47.43759" | 800.51 | NVA |
| 2005_2021_OH | N40°08'26.14602" | W81°21'02.00895" | 1025.54 | NVA |
| 2006_2021_OH | N40°03'07.23243" | W81°19'26.28868" | 791.38 | NVA |
| 2007_2020_OH | N40°26'59.10335" | W81°08'36.67370" | 1124.45 | NVA |
| 2008_2020_OH | N40°16'16.14659" | W81°36'22.71481" | 689.42 | NVA |
| 2009_2020_OH | N40°16'52.88550" | W81°35'03.64526" | 695.15 | NVA |
| 2010_2021_OH | N40°05'44.92658" | W81°17'00.34184" | 824.72 | NVA |
| 2011_2021_OH | N40°10'44.60128" | W81°29'45.08907" | 707.89 | NVA |
| 2012_2021_OH | N40°19'35.36255" | W81°29'14.37168" | 705.10 | NVA |
| 2013_2021_OH | N40°09'27.46979" | W81°27'19.29073" | 715.82 | NVA |
| 2014_2020_OH | N40°33'37.06690" | W81°29'48.11599" | 797.87 | NVA |
| 2015_2021_OH | N40°02'15.90400" | W81°26'43.58959" | 893.71 | NVA |
| 2016_2020_OH | N40°30'20.52260" | W81°16'44.38731" | 1026.92 | NVA |
| 2017_2020_OH | N40°23'51.55200" | W81°36'11.36934" | 1036.33 | NVA |
| 2018_2020_OH | N40°22'18.79650" | W81°23'48.30149" | 770.89 | NVA |
| 2019_2021_OH | N40°06'04.08448" | W81°10'59.35202" | 1087.72 | NVA |
| 2020_2020_OH | N40°26'14.14808" | W81°25'48.72135" | 824.48 | NVA |
| 2021_2021_OH | N40°10'53.26415" | W80°51'51.99016" | 944.04 | NVA |
| 2022_2021_OH | N40°04'05.74269" | W80°54'02.97024" | 1052.36 | NVA |
| 2023_2020_OH | N40°16'47.12155" | W81°05'40.65693" | 1121.13 | NVA |
| 2024_2021_OH | N39°59'49.48146" | W80°52'10.89412" | 752.16 | NVA |
| 2025_2021_OH | N40°32'18.78284" | W81°01'36.58212" | 1120.88 | NVA |
| 2026_2021_OH | N40°11'52.81888" | W80°57'01.36943" | 963.73 | NVA |
| 2027_2021_OH | N40°01'13.14694" | W81°02'02.66272" | 1215.34 | NVA |
| 2028_2021_OH | N40°03'33.82581" | W81°07'06.91035" | 863.13 | NVA |
| 2029_2021_OH | N40°00'23.58934" | W80°56'38.71862" | 981.49 | NVA |
| 2030_2021_OH | N40°26'03.81858" | W80°46'26.94625" | 1144.08 | NVA |
| 2031_2021_OH | N40°13'13.39042" | W80°56'13.13492" | 978.99 | NVA |
| 2032_2020_OH | N40°19'20.11758" | W80°55'03.99466" | 1166.91 | NVA |
| 2033_2021_OH | N40°21'27.88639" | W80°45'40.38943" | 995.30 | NVA |
| 2034_2021_OH | N40°03'47.02233" | W81°02'55.84211" | 1091.61 | NVA |
| 2035_2020_OH | N40°16'34.10176" | W80°53'31.37637" | 994.68 | NVA |
| 2036_2021_OH | N40°14'33.82949" | W80°44'18.72335" | 1079.69 | NVA |
| 2037_2021_OH | N40°25'23.40465" | W80°42'02.44776" | 1103.59 | NVA |
| 2038_2021_OH | N40°01'31.65329" | W80°59'51.83452" | 943.16 | NVA |
| 2039_2020_OH | N40°55'44.44582" | W81°04'28.55552" | 941.04 | NVA |
| 2040_2020_OH | N40°48'42.47307" | W81°15'37.64512" | 1003.07 | NVA |
| 2041_2020_OH | N40°55'21.84833" | W81°06'42.51041" | 1004.94 | NVA |
| 2042_2020_OH | N40°41'45.44473" | W81°16'02.67488" | 896.42 | NVA |
| 2043_2020_OH | N40°48'02.71744" | W81°21'06.19499" | 930.59 | NVA |
| 2044_2020_OH | N40°37'13.60284" | W81°28'27.80761" | 863.33 | NVA |
| 2045_2020_OH | N40°48'45.33645" | W81°31'32.78062" | 848.72 | NVA |
| 2046_2020_OH | N40°44'57.94157" | W81°19'00.53866" | 1154.76 | NVA |
| 2047_2020_OH | N40°47'23.83378" | W81°24'18.97429" | 961.96 | NVA |
| 2048_2020_OH | N40°50'19.20906" | W81°30'08.93836" | 975.92 | NVA |
| 2049_2020_OH | N40°50'27.12469" | W81°25'26.35028" | 962.39 | NVA |
| 2050_2020_OH | N40°47'28.41057" | W81°32'03.27183" | 880.11 | NVA |
| 2051_2020_OH | N40°57'49.67727" | W81°13'36.33300" | 1007.60 | NVA |
| 2052_2020_OH | N40°46'08.14070" | W81°10'19.50998" | 1096.33 | NVA |
| 2053_2020_OH | N40°45'29.08654" | W81°12'23.27744" | 961.81 | NVA |
| 2054_2020_OH | N40°48'26.01145" | W81°26'32.94775" | 1029.71 | NVA |
| 2055_2020_OH | N40°53'24.39700" | W81°24'03.41385" | 1038.20 | NVA |
| 2056_2020_OH | N40°53'25.48331" | W81°31'13.14874" | 985.11 | NVA |
| 2057_2020_OH | N41°05'21.62057" | W80°41'28.71328" | 907.85 | NVA |
| 2058_2020_OH | N40°47'45.82928" | W80°34'34.83485" | 821.19 | NVA |
| 2059_2020_OH | N40°59'15.68085" | W80°58'01.55351" | 965.93 | NVA |
| 2060_2020_OH | N40°56'02.14406" | W80°55'35.69404" | 1146.87 | NVA |
| 2061_2020_OH | N41°00'19.76213" | W80°39'08.63933" | 1014.97 | NVA |
| 2062_2020_OH | N40°43'48.66734" | W80°57'46.15408" | 1014.36 | NVA |
| 2063_2020_OH | N41°06'10.69898" | W80°41'24.90743" | 910.87 | NVA |
| 2064_2020_OH | N40°45'26.86236" | W80°57'48.00221" | 1176.18 | NVA |
| 2065_2021_OH | N40°33'42.77439" | W80°59'54.65366" | 983.46 | NVA |
| 2066_2020_OH | N40°43'16.42800" | W80°40'14.31171" | 923.54 | NVA |
| 2067_2020_OH | N41°06'11.23321" | W80°37'13.90197" | 842.72 | NVA |
| 2068_2020_OH | N40°40'57.40129" | W80°47'11.41069" | 977.88 | NVA |
| 2069_2020_OH | N41°01'44.05132" | W80°40'50.54950" | 935.35 | NVA |
| 2070_2020_OH | N40°48'48.13610" | W80°45'10.15597" | 1096.93 | NVA |
| 2071_2020_OH | N41°04'54.14865" | W80°39'27.55112" | 934.28 | NVA |
| 2072_2020_OH | N40°55'13.48793" | W80°39'09.29358" | 1024.49 | NVA |
| 2073_2020_OH | N40°59'15.40005" | W80°55'48.49629" | 1082.10 | NVA |
| 2074_2020_OH | N40°33'23.03855" | W80°39'53.55893" | 821.65 | NVA |
| 2075_2020_OH | N40°44'27.36399" | W80°43'23.32122" | 1014.77 | NVA |
| 2076_2020_OH | N41°05'55.65825" | W80°39'06.45910" | 747.19 | NVA |
| 2077_2020_OH | N40°51'01.34448" | W80°35'41.40921" | 1002.41 | NVA |
| 2078_2021_OH | N40°24'20.92221" | W82°25'53.92967" | 1056.15 | NVA |
| 2079_2021_OH | N40°19'22.28694" | W82°25'45.77775" | 1027.79 | NVA |
| 2080_2021_OH | N40°07'52.87837" | W82°34'52.07240" | 1067.08 | NVA |
| 2081_2021_OH | N40°13'21.76461" | W82°24'39.38637" | 921.46 | NVA |
| 2082_2021_OH | N40°31'59.14643" | W82°18'13.29349" | 938.29 | NVA |
| 2083_2021_OH | N40°31'32.89020" | W82°35'44.52349" | 1040.78 | NVA |
| 2084_2021_OH | N40°07'51.21166" | W82°30'21.17875" | 913.61 | NVA |
| 2085_2020_OH | N40°12'30.01745" | W82°14'34.65186" | 984.93 | NVA |
| 2086_2021_OH | N40°18'58.96818" | W82°24'21.65197" | 918.48 | NVA |
| 2087_2021_OH | N40°27'31.14028" | W82°20'09.99258" | 899.43 | NVA |
| 2088_2021_OH | N40°12'26.87654" | W82°29'25.46171" | 867.70 | NVA |
| 2089_2021_OH | N40°19'38.07972" | W82°27'40.08752" | 1028.88 | NVA |
| 2090_2020_OH | N40°12'34.80894" | W81°43'10.11310" | 690.46 | NVA |
| 2091_2020_OH | N40°24'22.39882" | W81°59'02.24050" | 864.47 | NVA |
| 2092_2021_OH | N40°10'28.99613" | W81°56'44.93479" | 685.11 | NVA |
| 2093_2021_OH | N40°15'03.85963" | W81°51'16.47600" | 664.69 | NVA |
| 2094_2020_OH | N40°21'11.48124" | W82°10'58.46098" | 823.18 | NVA |
| 2095_2021_OH | N40°05'01.01937" | W81°47'07.48539" | 696.19 | NVA |
| 2096_2021_OH | N40°17'44.87653" | W82°06'44.91747" | 932.73 | NVA |
| 2097_2020_OH | N40°29'20.90587" | W81°42'45.60100" | 1045.86 | NVA |
| 2098_2021_OH | N40°13'14.68007" | W81°57'57.78789" | 855.77 | NVA |
| 2099_2021_OH | N40°15'05.40504" | W81°52'11.95176" | 638.95 | NVA |
| 2100_2021_OH | N40°07'52.03343" | W82°00'04.89822" | 609.48 | NVA |
| 2101_2020_OH | N40°16'13.19627" | W81°45'23.35332" | 692.08 | NVA |
| 2102_2021_OH | N40°03'23.72868" | W81°59'19.65816" | 757.25 | NVA |
| 2103_2021_OH | N40°28'53.12977" | W82°09'31.11170" | 1074.31 | NVA |
| 2104_2020_OH | N40°48'19.51567" | W82°04'01.41852" | 1106.76 | NVA |
| 2105_2020_OH | N40°54'55.87388" | W81°56'10.93653" | 1040.86 | NVA |
| 2106_2020_OH | N40°41'54.19338" | W82°06'14.63658" | 940.87 | NVA |
| 2107_2020_OH | N40°54'05.40766" | W81°53'50.94603" | 1038.41 | NVA |
| 2108_2020_OH | N40°37'53.17455" | W82°02'01.66176" | 1023.71 | NVA |
| 2109_2020_OH | N40°45'19.15911" | W81°55'49.41611" | 808.04 | NVA |
| 2110_2020_OH | N40°37'21.20010" | W81°59'37.94712" | 1042.22 | NVA |
| 2111_2020_OH | N40°47'58.33237" | W82°01'22.86442" | 1023.93 | NVA |
| 2112_2020_OH | N40°36'04.46914" | W81°53'34.90752" | 851.73 | NVA |
| 2113_2020_OH | N40°33'52.95262" | W81°40'54.16638" | 869.21 | NVA |
| 2114_2020_OH | N40°57'37.30051" | W81°53'08.81827" | 880.08 | NVA |
| 2115_2020_OH | N40°32'32.36906" | W81°43'25.36303" | 1053.29 | NVA |
| 2116_2020_OH | N40°35'10.78004" | W81°46'02.33486" | 1177.57 | NVA |
| 2117_2020_OH | N40°49'35.72059" | W81°55'01.87400" | 954.04 | NVA |
| 2118_2021_OH | N39°21'38.55692" | W81°45'42.11758" | 752.16 | NVA |
| 2119_2021_OH | N39°19'43.81687" | W81°46'24.34355" | 685.35 | NVA |
| 2120_2021_OH | N39°09'17.25800" | W81°46'58.41736" | 729.42 | NVA |
| 2121_2021_OH | N39°10'52.21198" | W81°46'52.80892" | 597.91 | NVA |
| 2122_2021_OH | N39°07'46.61233" | W81°52'45.49075" | 494.20 | NVA |
| 2123_2021_OH | N39°00'47.55099" | W81°46'29.02632" | 509.01 | NVA |
| 2124_2021_OH | N39°17'27.58553" | W81°43'12.87849" | 732.59 | NVA |
| 2125_2021_OH | N39°28'36.16245" | W81°47'41.67157" | 697.79 | NVA |
| 2126_2021_OH | N39°57'58.89712" | W81°33'02.17754" | 809.01 | NVA |
| 2127_2021_OH | N39°54'31.49986" | W81°40'40.19433" | 766.20 | NVA |
| 2128_2021_OH | N39°35'31.25177" | W81°22'23.94368" | 545.84 | NVA |
| 2129_2021_OH | N39°47'05.48419" | W81°22'34.99841" | 1029.55 | NVA |
| 2130_2021_OH | N39°58'08.14264" | W81°30'13.27685" | 916.02 | NVA |
| 2131_2021_OH | N39°44'32.23510" | W81°30'30.37431" | 617.60 | NVA |
| 2132_2021_OH | N39°28'24.43421" | W81°32'07.95449" | 906.55 | NVA |
| 2133_2021_OH | N39°57'37.17360" | W81°17'58.86147" | 921.05 | NVA |
| 2134_2021_OH | N39°31'11.55099" | W81°49'00.23334" | 834.00 | NVA |
| 2135_2021_OH | N39°58'17.75566" | W81°35'52.23899" | 903.41 | NVA |
| 2136_2021_OH | N39°36'17.73699" | W81°20'10.89802" | 777.36 | NVA |
| 2137_2021_OH | N39°41'47.02454" | W81°22'18.04683" | 977.26 | NVA |
| 2138_2021_OH | N39°40'00.15465" | W81°39'42.92990" | 815.88 | NVA |
| 2139_2021_OH | N39°42'23.23832" | W81°39'48.51411" | 861.68 | NVA |
| 2140_2021_OH | N39°41'37.38325" | W81°36'49.67686" | 839.75 | NVA |
| 2141_2021_OH | N39°46'22.73935" | W80°56'08.16438" | 575.93 | NVA |
| 2142_2021_OH | N39°40'17.36258" | W81°17'36.40163" | 649.92 | NVA |
| 2143_2021_OH | N39°46'57.68318" | W81°15'53.03646" | 855.00 | NVA |
| 2144_2021_OH | N39°52'35.43628" | W80°54'14.52697" | 1156.48 | NVA |
| 2145_2021_OH | N39°55'00.45840" | W81°12'20.21496" | 1212.47 | NVA |
| 2146_2021_OH | N39°47'14.36984" | W81°04'32.84629" | 881.52 | NVA |
| 2147_2021_OH | N39°40'49.46137" | W81°10'20.30648" | 1020.85 | NVA |
| 2148_2021_OH | N39°33'58.88181" | W81°07'05.87591" | 702.70 | NVA |
| 2149_2021_OH | N39°45'19.94979" | W81°08'45.72996" | 817.08 | NVA |
| 2150_2021_OH | N39°28'29.78236" | W81°11'00.61002" | 1033.82 | NVA |
| 2151_2021_OH | N39°45'10.56301" | W81°01'47.27549" | 1077.68 | NVA |
| 2152_2021_OH | N39°48'06.02452" | W81°07'57.66263" | 1108.16 | NVA |
| 2153_2021_OH | N39°33'13.95120" | W81°03'18.35400" | 624.02 | NVA |
| 2154_2021_OH | N39°27'49.19419" | W82°26'09.37408" | 674.40 | NVA |
| 2155_2021_OH | N39°24'06.69897" | W82°29'17.24764" | 917.89 | NVA |
| 2156_2021_OH | N39°23'16.39408" | W82°08'25.72835" | 549.16 | NVA |
| 2157_2021_OH | N39°08'45.94526" | W82°16'44.65691" | 720.18 | NVA |
| 2158_2021_OH | N39°15'50.40435" | W82°00'32.63774" | 830.95 | NVA |
| 2159_2021_OH | N39°03'07.87668" | W82°07'36.23850" | 472.90 | NVA |
| 2160_2021_OH | N39°13'49.03434" | W81°58'56.48228" | 799.21 | NVA |
| 2161_2021_OH | N38°54'32.72485" | W82°13'58.81340" | 525.26 | NVA |
| 2162_2021_OH | N39°00'20.88266" | W81°57'26.51877" | 653.33 | NVA |
| 2163_2021_OH | N39°07'30.17988" | W81°55'23.52795" | 528.76 | NVA |
| 2164_2021_OH | N39°25'11.50730" | W82°01'45.78740" | 750.32 | NVA |
| 2165_2021_OH | N39°04'20.02686" | W82°18'40.43517" | 576.72 | NVA |
| 2166_2021_OH | N39°23'32.34835" | W82°07'08.67743" | 543.38 | NVA |
| 2167_2021_OH | N39°18'15.16544" | W82°06'11.13793" | 566.00 | NVA |
| 2168_2021_OH | N39°17'23.23662" | W81°58'42.40610" | 555.97 | NVA |
| 2169_2021_OH | N39°54'12.51748" | W82°29'08.09716" | 841.35 | NVA |
| 2170_2021_OH | N40°02'05.14533" | W82°37'13.34856" | 1059.63 | NVA |
| 2171_2021_OH | N39°43'54.91709" | W82°33'48.86721" | 779.17 | NVA |
| 2172_2021_OH | N39°36'52.49524" | W82°44'43.17799" | 805.70 | NVA |
| 2173_2021_OH | N40°04'18.97593" | W82°25'29.46199" | 744.67 | NVA |
| 2174_2021_OH | N39°44'59.05199" | W82°44'10.07570" | 979.18 | NVA |
| 2175_2021_OH | N39°29'26.59412" | W82°26'10.43162" | 650.16 | NVA |
| 2176_2021_OH | N39°53'32.89488" | W82°29'08.05109" | 907.36 | NVA |
| 2177_2021_OH | N39°49'42.48515" | W82°19'35.50779" | 959.81 | NVA |
| 2178_2021_OH | N40°02'45.56135" | W82°25'06.62944" | 739.39 | NVA |
| 2179_2021_OH | N39°34'21.72480" | W82°44'11.58401" | 1004.06 | NVA |
| 2180_2021_OH | N39°33'44.38861" | W82°38'43.62891" | 1051.90 | NVA |
| 2181_2021_OH | N39°40'15.93463" | W82°36'06.97873" | 986.29 | NVA |
| 2182_2021_OH | N39°50'23.86588" | W82°46'58.38054" | 671.70 | NVA |
| 2183_2021_OH | N39°37'30.43138" | W82°32'52.77352" | 667.03 | NVA |
| 2184_2021_OH | N39°46'35.06888" | W82°25'45.99716" | 905.30 | NVA |
| 2185_2021_OH | N40°02'47.05713" | W82°23'37.70590" | 770.05 | NVA |
| 2186_2021_OH | N39°46'36.78190" | W82°36'23.44838" | 881.04 | NVA |
| 2187_2021_OH | N39°47'41.69865" | W82°44'12.27561" | 816.40 | NVA |
| 2188_2021_OH | N39°42'43.38560" | W82°37'57.71142" | 817.96 | NVA |
| 2189_2021_OH | N39°27'30.10806" | W82°14'00.37960" | 563.18 | NVA |
| 2190_2021_OH | N39°43'18.54277" | W82°12'13.14726" | 762.49 | NVA |
| 2191_2021_OH | N39°27'42.04434" | W82°09'32.32043" | 797.77 | NVA |
| 2192_2021_OH | N39°29'51.72303" | W82°20'17.37611" | 597.14 | NVA |
| 2193_2021_OH | N39°31'39.07702" | W82°08'49.29077" | 872.99 | NVA |
| 2194_2021_OH | N39°32'33.77035" | W82°00'09.39560" | 954.54 | NVA |
| 2195_2021_OH | N39°42'42.30737" | W81°50'51.71404" | 724.44 | NVA |
| 2196_2021_OH | N39°40'29.17801" | W81°52'48.04921" | 577.64 | NVA |
| 2197_2021_OH | N39°38'55.70273" | W82°01'58.48548" | 822.14 | NVA |
| 2198_2021_OH | N39°41'14.58342" | W81°58'21.67211" | 869.24 | NVA |
| 2199_2021_OH | N39°59'15.98427" | W82°01'14.56102" | 728.75 | NVA |
| 2200_2021_OH | N39°48'35.86780" | W82°14'41.18014" | 932.54 | NVA |
| 2201_2021_OH | N39°51'12.75681" | W82°09'05.23340" | 783.82 | NVA |
| 2202_2021_OH | N39°52'31.60499" | W82°09'17.94129" | 877.59 | NVA |
| 2203_2021_OH | N39°54'51.18720" | W82°10'26.41457" | 744.23 | NVA |
| 2204_2021_OH | N40°00'07.31191" | W81°52'44.22192" | 802.93 | NVA |
| 2205_2021_OH | N39°57'00.84319" | W81°52'25.15105" | 609.92 | NVA |
| 2206_2021_OH | N39°50'02.03363" | W82°02'01.76758" | 844.25 | NVA |
| 2207_2021_OH | N39°44'57.39667" | W81°51'41.82156" | 810.38 | NVA |
| 2208_2021_OH | N39°55'56.34445" | W81°53'08.02263" | 794.61 | NVA |
| 2209_2021_OH | N39°54'23.62510" | W81°45'28.64106" | 930.86 | NVA |
| 2210_2020_OH | N40°48'01.34856" | W81°56'30.86645" | 815.09 | NVA |
| 2211_2020_OH | N40°50'09.40913" | W81°20'06.77495" | 990.97 | NVA |
| 2212_2020_OH | N40°43'29.43321" | W81°31'01.90073" | 862.36 | NVA |
| 2213_2021_OH | N40°01'24.18207" | W81°35'17.46782" | 779.29 | NVA |
| 2214_2021_OH | N40°03'05.43525" | W81°34'43.51748" | 692.90 | NVA |
| 2215_2021_OH | N39°32'23.59621" | W82°24'19.17521" | 627.61 | NVA |
| 2216_2020_OH | N41°01'45.02410" | W80°37'59.08907" | 957.23 | NVA |
| 2217_2021_OH | N40°22'24.98221" | W82°30'30.51577" | 920.07 | NVA |
| 2218_2021_OH | N39°59'45.45269" | W82°40'26.98020" | 890.57 | NVA |
| 2219_2020_OH | N40°34'40.40022" | W81°05'02.44390" | 1037.21 | NVA |
| 2220_2021_OH | N39°20'12.15606" | W82°04'18.99324" | 526.46 | NVA |
| 2221_2021_OH | N40°09'11.71591" | W82°41'04.30560" | 1046.50 | NVA |
| 2222_2021_OH | N39°59'40.44565" | W81°44'06.08748" | 777.76 | NVA |
| 2223_2021_OH | N40°12'36.35956" | W81°15'47.58668" | 877.16 | NVA |
| 2224_2020_OH | N40°16'44.43477" | W81°00'14.14156" | 1105.38 | NVA |
| 2225_2021_OH | N39°50'45.01936" | W82°36'10.41550" | 747.92 | NVA |
| 2226_2021_OH | N40°04'38.52296" | W80°57'54.11648" | 1104.49 | NVA |
| 2227_2021_OH | N40°04'40.56555" | W82°13'38.80170" | 784.79 | NVA |
| 2228_2021_OH | N39°30'06.57955" | W82°05'04.92582" | 574.21 | NVA |
| 2229_2020_OH | N40°29'44.60913" | W81°59'03.52526" | 700.34 | NVA |
| 2231_2021_OH | N39°35'15.01174" | W82°47'07.34314" | 875.45 | NVA |
| 2233_2021_OH | N39°44'15.01548" | W82°04'17.60438" | 715.86 | NVA |
| 2235_2021_OH | N39°43'43.00759" | W81°54'18.86867" | 559.61 | NVA |
| 2236_2021_OH | N39°41'15.18383" | W82°45'09.31642" | 824.76 | NVA |
| 2237_2021_OH | N39°33'36.81815" | W81°55'15.77356" | 646.99 | NVA |
| 2238_2021_OH | N39°41'38.52726" | W82°39'27.20663" | 834.51 | NVA |
| 2240_2021_OH | N39°28'45.38939" | W81°49'50.14734" | 864.66 | NVA |
| 2241_2021_OH | N39°38'38.65259" | W82°33'22.83319" | 676.47 | NVA |
| 2242_2021_OH | N39°33'27.26021" | W81°59'43.22361" | 997.64 | NVA |
| 2244_2021_OH | N39°31'16.85505" | W81°57'47.64836" | 604.09 | NVA |
| 2245_2021_OH | N39°44'32.76781" | W82°47'26.33875" | 860.86 | NVA |
| 2247_2021_OH | N39°40'02.96235" | W81°47'11.49703" | 859.71 | NVA |
| 2249_2021_OH | N39°36'43.79341" | W81°50'58.67329" | 865.13 | NVA |
| 2252_2021_OH | N39°41'02.60309" | W82°00'54.71835" | 898.59 | NVA |
| 2253_2021_OH | N39°51'43.05390" | W82°44'30.71340" | 745.62 | NVA |
| 2254_2021_OH | N39°38'56.56369" | W82°49'34.16929" | 988.04 | NVA |
| 2256_2021_OH | N39°47'06.98407" | W82°47'29.46345" | 947.85 | NVA |
| 2257_2021_OH | N39°29'52.02646" | W82°02'43.56807" | 795.95 | NVA |
| 2258_2021_OH | N38°54'32.58461" | W82°11'27.27830" | 553.32 | NVA |
| 2259_2021_OH | N39°36'09.83978" | W81°40'17.26581" | 798.22 | NVA |
| 2260_2021_OH | N38°59'38.21183" | W82°09'40.06612" | 533.82 | NVA |
| 2261_2021_OH | N39°38'01.61337" | W81°56'41.68415" | 915.49 | NVA |
| 2262_2021_OH | N38°58'29.62977" | W82°12'49.55122" | 553.23 | NVA |
| 2263_2021_OH | N39°48'05.79815" | W82°38'43.84043" | 763.57 | NVA |
| 2264_2021_OH | N38°57'02.65882" | W82°06'21.87700" | 460.56 | NVA |
| 2265_2021_OH | N39°38'22.72931" | W82°00'39.03395" | 903.95 | NVA |
| 2266_2021_OH | N38°57'35.35305" | W82°12'06.66568" | 803.13 | NVA |
| 2267_2021_OH | N39°30'00.46772" | W81°57'37.60574" | 579.65 | NVA |
| 2268_2021_OH | N38°50'14.50716" | W82°09'52.08051" | 460.13 | NVA |
| 2270_2021_OH | N39°35'30.41629" | W81°46'00.44733" | 830.65 | NVA |
| 2272_2021_OH | N39°41'36.22273" | W82°37'24.63858" | 843.10 | NVA |
| 2273_2021_OH | N39°36'15.17744" | W82°40'32.59075" | 962.02 | NVA |
| 2274_2021_OH | N39°52'13.09238" | W82°41'25.99745" | 769.41 | NVA |
| 2276_2021_OH | N39°44'13.40093" | W82°37'37.99781" | 762.34 | NVA |
| 2279_2021_OH | N39°38'53.38606" | W81°50'57.31482" | 588.06 | NVA |
| 2281_2021_OH | N39°53'33.31393" | W82°31'59.08569" | 828.47 | NVA |
| 2283_2021_OH | N39°42'51.21920" | W82°35'56.31450" | 774.85 | NVA |
| 2284_2021_OH | N38°57'28.82382" | W82°17'51.06298" | 698.14 | NVA |
| 2286_2021_OH | N39°41'56.97239" | W81°43'52.29541" | 886.52 | NVA |
| 2288_2021_OH | N39°44'49.14420" | W82°40'45.53601" | 751.98 | NVA |
| 2289_2021_OH | N39°50'54.05430" | W82°28'20.22756" | 913.38 | NVA |
| 2291_2021_OH | N39°43'55.34040" | W81°38'53.79576" | 950.35 | NVA |
| 2292_2021_OH | N39°55'50.50196" | W82°47'23.76027" | 718.17 | NVA |
| 2293_2021_OH | N39°32'53.86204" | W81°47'24.29794" | 550.82 | NVA |
| 2294_2021_OH | N39°39'14.16543" | W81°49'24.46717" | 934.10 | NVA |
| 2296_2021_OH | N39°41'33.03047" | W81°48'01.53361" | 597.90 | NVA |
| 2297_2021_OH | N39°34'56.71969" | W81°59'26.45917" | 971.33 | NVA |
| 2299_2021_OH | N39°45'25.11304" | W82°28'02.02507" | 905.66 | NVA |
| 2300_2021_OH | N38°55'02.19252" | W82°16'46.53701" | 640.01 | NVA |
| 2301_2021_OH | N39°40'45.12554" | W82°25'07.81113" | 683.86 | NVA |
| 3001_2021_OH | N39°21'30.14248" | W81°55'40.12939" | 726.01 | VVA |
| 3002_2020_OH | N40°31'53.19510" | W81°57'59.67798" | 1069.34 | VVA |
| 3003_2021_OH | N40°27'24.41812" | W80°46'28.04115" | 793.57 | VVA |
| 3004_2021_OH | N39°57'44.02412" | W81°30'02.60202" | 764.88 | VVA |
| 3005_2021_OH | N39°25'33.85917" | W82°18'39.47315" | 740.18 | VVA |
| 3006_2021_OH | N39°11'28.74864" | W82°12'59.30632" | 631.08 | VVA |
| 3007_2021_OH | N39°40'57.70666" | W81°03'00.54435" | 932.30 | VVA |
| 3008_2021_OH | N40°27'19.62961" | W80°44'03.41924" | 1127.15 | VVA |
| 3009_2021_OH | N39°41'15.97041" | W81°30'20.99368" | 686.56 | VVA |
| 3010_2021_OH | N39°30'29.16667" | W81°18'12.08431" | 574.20 | VVA |
| 3011_2021_OH | N39°27'26.94927" | W82°06'13.36817" | 555.83 | VVA |
| 3012_2021_OH | N39°29'23.50979" | W81°58'41.51720" | 883.03 | VVA |
| 3013_2020_OH | N40°26'58.15082" | W82°04'59.36138" | 1052.21 | VVA |
| 3014_2021_OH | N39°40'36.33321" | W82°16'26.72051" | 1006.14 | VVA |
| 3015_2021_OH | N39°42'00.83242" | W81°57'39.32749" | 868.14 | VVA |
| 3016_2021_OH | N39°38'07.31186" | W82°39'42.16530" | 921.19 | VVA |
| 3017_2021_OH | N39°59'26.75794" | W82°08'25.17721" | 651.04 | VVA |
| 3018_2021_OH | N39°25'49.27233" | W82°10'15.73389" | 820.63 | VVA |
| 3019_2021_OH | N39°34'11.48008" | W81°28'09.30445" | 972.00 | VVA |
| 3020_2020_OH | N40°55'54.60151" | W80°37'38.17273" | 1072.00 | VVA |
| 3021_2020_OH | N40°47'56.90909" | W80°35'20.44613" | 855.36 | VVA |
| 3022_2021_OH | N39°23'56.64731" | W82°22'25.21378" | 660.46 | VVA |
| 3023_2021_OH | N39°35'55.19788" | W81°48'52.53256" | 569.01 | VVA |
| 3024_2021_OH | N40°27'11.39649" | W80°41'31.49270" | 958.76 | VVA |
| 3025_2020_OH | N40°48'21.11233" | W81°53'49.11643" | 834.26 | VVA |
| 3026_2021_OH | N40°09'30.82980" | W82°19'51.42223" | 948.95 | VVA |
| 3027_2021_OH | N40°19'15.42547" | W80°46'37.92575" | 1083.76 | VVA |
| 3028_2020_OH | N40°46'10.69268" | W80°44'58.38774" | 846.32 | VVA |
| 3029_2021_OH | N40°08'13.48164" | W81°39'58.01400" | 840.50 | VVA |
| 3030_2021_OH | N40°21'23.82151" | W80°52'43.37223" | 1028.04 | VVA |
| 3031_2021_OH | N40°01'11.23595" | W80°48'32.89532" | 607.25 | VVA |
| 3032_2021_OH | N39°29'34.50541" | W81°50'27.55114" | 781.91 | VVA |
| 3033_2021_OH | N39°02'21.40002" | W81°56'09.41684" | 587.82 | VVA |
| 3034_2020_OH | N40°11'32.53507" | W81°40'43.43243" | 692.15 | VVA |
| 3035_2020_OH | N40°57'34.28728" | W81°00'42.45348" | 966.95 | VVA |
| 3036_2021_OH | N39°34'05.99701" | W82°28'41.36343" | 653.27 | VVA |
| 3037_2021_OH | N40°22'26.91549" | W82°20'38.75139" | 827.83 | VVA |
| 3038_2020_OH | N40°33'15.91066" | W82°05'31.54737" | 1198.18 | VVA |
| 3039_2020_OH | N40°32'43.59756" | W81°52'32.07006" | 896.03 | VVA |
| 3040_2021_OH | N39°27'43.96099" | W81°47'38.42577" | 737.79 | VVA |
| 3041_2021_OH | N40°05'38.18119" | W81°54'26.28287" | 888.59 | VVA |
| 3042_2021_OH | N39°22'22.61563" | W81°37'01.87099" | 615.08 | VVA |
| 3043_2021_OH | N39°21'01.47978" | W82°09'35.37974" | 740.46 | VVA |
| 3044_2020_OH | N40°20'46.14455" | W80°59'20.93979" | 1115.49 | VVA |
| 3045_2021_OH | N39°44'44.39556" | W81°37'49.46707" | 930.30 | VVA |
| 3046_2020_OH | N40°23'41.20780" | W82°16'35.87421" | 765.76 | VVA |
| 3047_2021_OH | N39°34'04.57123" | W81°10'38.72986" | 589.81 | VVA |
| 3048_2021_OH | N40°10'44.86356" | W82°14'53.47582" | 829.64 | VVA |
| 3049_2020_OH | N40°30'01.34247" | W82°08'32.34436" | 903.04 | VVA |
| 3050_2020_OH | N40°36'01.05040" | W81°28'12.10628" | 1024.92 | VVA |
| 3051_2021_OH | N39°36'03.11221" | W81°12'37.72494" | 647.12 | VVA |
| 3052_2021_OH | N40°18'22.39445" | W81°27'06.10752" | 856.24 | VVA |
| 3053_2021_OH | N40°01'41.53031" | W81°13'29.43553" | 1155.99 | VVA |
| 3054_2021_OH | N39°56'33.39776" | W81°35'45.62974" | 777.62 | VVA |
| 3055_2021_OH | N40°09'38.97353" | W81°19'20.37336" | 827.45 | VVA |
| 3056_2021_OH | N40°15'45.67879" | W81°18'44.14101" | 1008.04 | VVA |
| 3057_2021_OH | N39°36'21.72262" | W82°15'02.68436" | 633.78 | VVA |
| 3058_2021_OH | N39°39'53.77820" | W81°17'25.86819" | 922.84 | VVA |
| 3059_2021_OH | N39°53'11.97816" | W81°16'18.85764" | 1100.69 | VVA |
| 3060_2021_OH | N39°47'05.22179" | W81°25'28.37173" | 780.66 | VVA |
| 3061_2021_OH | N40°12'07.75144" | W81°17'55.17937" | 774.69 | VVA |
| 3062_2021_OH | N40°12'01.65495" | W80°43'00.82863" | 709.72 | VVA |
| 3063_2021_OH | N39°34'51.98336" | W82°32'45.39882" | 927.65 | VVA |
| 3064_2021_OH | N40°36'43.27830" | W80°53'40.43401" | 931.81 | VVA |
| 3065_2020_OH | N40°40'58.08884" | W81°39'49.69608" | 1049.00 | VVA |
| 3066_2021_OH | N39°30'05.51743" | W81°08'52.01199" | 954.15 | VVA |
| 3067_2020_OH | N40°28'01.89713" | W81°14'58.53048" | 873.42 | VVA |
| 3068_2021_OH | N39°32'43.58012" | W82°25'20.36906" | 636.53 | VVA |
| 3069_2021_OH | N39°58'54.50712" | W81°27'24.40004" | 711.81 | VVA |
| 3070_2021_OH | N39°39'32.41994" | W81°30'07.55176" | 985.51 | VVA |
| 3071_2021_OH | N39°14'21.35997" | W82°12'23.12078" | 649.73 | VVA |
| 3072_2021_OH | N39°57'53.95274" | W81°41'57.45761" | 708.32 | VVA |
| 3073_2021_OH | N39°55'36.00226" | W81°15'35.57298" | 898.54 | VVA |
| 3074_2020_OH | N40°53'40.44244" | W80°34'14.59612" | 938.30 | VVA |
| 3075_2020_OH | N40°23'30.25740" | W81°09'27.32237" | 1126.82 | VVA |
| 3076_2021_OH | N40°04'52.40497" | W81°19'19.88453" | 929.07 | VVA |
| 3077_2020_OH | N40°45'04.17290" | W81°44'41.81117" | 1064.58 | VVA |
| 3078_2020_OH | N40°49'19.10348" | W80°56'02.93821" | 1153.79 | VVA |
| 3079_2021_OH | N40°09'00.82235" | W82°14'51.01330" | 820.48 | VVA |
| 3080_2021_OH | N39°24'36.49555" | W81°43'23.73206" | 775.95 | VVA |
| 3081_2020_OH | N40°22'56.62817" | W81°02'38.46530" | 878.87 | VVA |
| 3082_2020_OH | N40°27'59.05766" | W81°36'42.10266" | 895.83 | VVA |
| 3083_2021_OH | N38°56'31.37124" | W82°15'12.74337" | 495.28 | VVA |
| 3084_2021_OH | N40°20'05.94431" | W80°42'53.62004" | 661.90 | VVA |
| 3085_2021_OH | N39°46'15.75704" | W81°55'44.51390" | 883.12 | VVA |
| 3086_2021_OH | N40°00'43.79075" | W82°35'00.95614" | 997.02 | VVA |
| 3087_2020_OH | N40°20'10.16990" | W82°20'23.69244" | 964.78 | VVA |
| 3088_2020_OH | N40°37'36.31215" | W81°36'51.95577" | 1049.59 | VVA |
| 3089_2021_OH | N40°32'00.95086" | W82°31'56.53881" | 1065.58 | VVA |
| 3090_2020_OH | N40°52'20.99080" | W81°37'21.37896" | 910.77 | VVA |
| 3091_2021_OH | N39°57'45.46747" | W82°06'25.09195" | 866.01 | VVA |
| 3092_2021_OH | N39°59'06.14841" | W82°23'12.68467" | 834.71 | VVA |
| 3093_2021_OH | N40°01'35.43946" | W81°55'16.75728" | 873.44 | VVA |
| 3094_2021_OH | N40°03'02.49294" | W81°40'26.94601" | 877.21 | VVA |
| 3095_2021_OH | N39°24'03.43682" | W81°43'24.83373" | 834.22 | VVA |
| 3096_2021_OH | N39°34'50.53254" | W82°22'23.46353" | 656.40 | VVA |
| 3097_2021_OH | N39°22'07.95177" | W81°44'36.00612" | 655.70 | VVA |
| 3098_2020_OH | N40°28'46.81980" | W81°44'46.07956" | 1001.47 | VVA |
| 3099_2021_OH | N39°45'02.01802" | W80°55'19.35871" | 539.76 | VVA |
| 3100_2021_OH | N40°27'17.97037" | W82°35'27.32804" | 973.81 | VVA |
| 3101_2020_OH | N40°35'48.30318" | W82°02'15.82096" | 892.91 | VVA |
| 3102_2021_OH | N40°12'28.53257" | W82°44'23.96547" | 1026.04 | VVA |
| 3103_2020_OH | N40°47'15.42866" | W81°37'28.09119" | 1031.42 | VVA |
| 3104_2021_OH | N39°56'34.66294" | W81°28'51.64400" | 744.64 | VVA |
| 3105_2020_OH | N40°16'02.91024" | W81°43'00.24322" | 710.59 | VVA |
| 3106_2021_OH | N39°09'40.99046" | W82°11'42.40690" | 561.41 | VVA |
| 3107_2021_OH | N40°01'48.68051" | W81°03'54.61652" | 1118.03 | VVA |
| 3108_2020_OH | N40°53'19.57477" | W80°32'01.95823" | 909.08 | VVA |
| 3109_2021_OH | N40°14'18.77350" | W81°54'01.42838" | 924.94 | VVA |
| 3110_2021_OH | N39°58'47.30549" | W82°06'14.54693" | 871.33 | VVA |
| 3111_2021_OH | N39°46'53.05486" | W81°43'14.57796" | 889.35 | VVA |
| 3112_2020_OH | N40°39'46.25530" | W81°36'29.76655" | 991.68 | VVA |
| 3113_2021_OH | N39°12'53.62407" | W81°47'53.42096" | 632.06 | VVA |
| 3114_2020_OH | N40°53'07.25292" | W80°35'53.77048" | 1090.00 | VVA |
| 3115_2021_OH | N39°38'53.72317" | W81°43'45.75633" | 743.32 | VVA |
| 3116_2021_OH | N39°18'20.64153" | W81°36'19.43941" | 651.86 | VVA |
| 3117_2021_OH | N39°26'13.70276" | W82°26'51.66928" | 919.59 | VVA |
| 3118_2020_OH | N40°33'38.87717" | W81°51'00.26957" | 1025.73 | VVA |
| 3119_2020_OH | N40°49'31.31868" | W81°38'20.83164" | 1014.81 | VVA |
| 3120_2021_OH | N40°06'42.67226" | W82°14'49.38518" | 710.47 | VVA |
| 3121_2020_OH | N40°23'32.97785" | W81°36'02.89336" | 1084.91 | VVA |
| 3122_2021_OH | N39°01'28.22360" | W82°08'23.55114" | 458.22 | VVA |
| 3123_2021_OH | N40°16'38.48849" | W81°17'27.67997" | 781.91 | VVA |
| 3124_2021_OH | N39°49'11.13900" | W81°00'12.74476" | 1169.85 | VVA |
| 3125_2021_OH | N40°24'56.79232" | W80°53'24.32859" | 1112.74 | VVA |
| 3126_2020_OH | N40°59'29.31582" | W80°34'50.62830" | 1002.65 | VVA |
| 3127_2021_OH | N39°12'25.76155" | W82°09'52.26694" | 623.41 | VVA |
| 3128_2021_OH | N40°29'20.68740" | W80°58'56.42494" | 1116.09 | VVA |
| 3129_2021_OH | N39°12'26.13086" | W82°09'52.87795" | 624.15 | VVA |
| 3130_2021_OH | N40°06'19.59592" | W81°41'02.80546" | 941.29 | VVA |
| 3131_2021_OH | N39°49'28.54328" | W82°32'00.59727" | 789.10 | VVA |
| 3132_2020_OH | N40°30'20.12760" | W82°00'20.38813" | 704.13 | VVA |
| 3133_2021_OH | N40°00'49.67078" | W81°03'48.60997" | 1061.86 | VVA |
| 3134_2020_OH | N40°47'07.01374" | W81°07'28.95140" | 999.89 | VVA |
| 3135_2020_OH | N40°17'30.22886" | W80°53'19.75150" | 1133.92 | VVA |
| 3136_2021_OH | N40°10'50.56269" | W81°12'06.99707" | 1048.06 | VVA |
| 3137_2021_OH | N40°15'47.46022" | W81°16'52.08084" | 785.03 | VVA |
| 3138_2021_OH | N39°58'33.30651" | W82°17'00.40498" | 1019.58 | VVA |
| 3139_2021_OH | N40°07'00.29889" | W81°57'12.37351" | 872.94 | VVA |
| 3140_2021_OH | N40°31'03.87617" | W82°30'06.24009" | 997.77 | VVA |
| 3141_2021_OH | N39°25'34.76569" | W81°48'27.44201" | 788.84 | VVA |
| 3142_2021_OH | N39°49'04.54338" | W81°13'32.90650" | 916.89 | VVA |
| 3143_2020_OH | N40°30'25.54446" | W82°03'16.78874" | 723.90 | VVA |
| 3144_2021_OH | N39°36'21.15793" | W81°46'20.38781" | 879.80 | VVA |
| 3145_2021_OH | N39°59'13.32765" | W82°03'45.48661" | 732.02 | VVA |
| 3146_2021_OH | N40°13'34.14255" | W81°30'54.64936" | 1026.17 | VVA |
| 3147_2021_OH | N39°51'40.37634" | W81°52'03.63916" | 617.31 | VVA |
| 3148_2020_OH | N40°30'21.46148" | W81°14'37.92916" | 806.97 | VVA |
| 3149_2020_OH | N40°53'53.86130" | W81°36'08.33822" | 839.37 | VVA |
| 3150_2021_OH | N39°34'35.34882" | W81°51'08.71068" | 905.95 | VVA |
| 3151_2021_OH | N39°57'05.87283" | W80°53'06.41248" | 909.73 | VVA |
| 3152_2021_OH | N40°01'28.73954" | W82°13'48.86616" | 708.39 | VVA |
| 3153_2021_OH | N39°55'17.08794" | W81°07'50.74179" | 1138.66 | VVA |
| 3154_2021_OH | N40°07'23.97691" | W81°33'24.32663" | 928.14 | VVA |
| 3155_2020_OH | N40°49'30.91116" | W81°42'02.25002" | 998.14 | VVA |
| 3156_2020_OH | N40°48'32.81018" | W80°53'56.38984" | 1108.79 | VVA |
| 3157_2020_OH | N40°44'07.02308" | W81°43'36.15611" | 952.89 | VVA |
| 3158_2021_OH | N39°45'22.19458" | W81°25'16.42150" | 1030.60 | VVA |
| 3159_2021_OH | N40°01'18.06564" | W82°32'23.08440" | 900.82 | VVA |
| 3160_2020_OH | N40°32'27.24258" | W81°23'22.11966" | 910.67 | VVA |
| 3161_2020_OH | N40°32'08.91056" | W81°52'49.12707" | 773.73 | VVA |
| 3162_2020_OH | N40°31'15.62911" | W82°01'56.18382" | 1002.72 | VVA |
| 3164_2021_OH | N39°44'51.18118" | W81°51'27.08945" | 785.05 | VVA |
| 3165_2021_OH | N39°43'49.67832" | W82°22'42.19362" | 688.41 | VVA |
| 3167_2021_OH | N39°41'08.22754" | W82°26'47.43081" | 695.32 | VVA |
| 3169_2021_OH | N39°51'00.93995" | W82°42'14.60352" | 744.25 | VVA |
| 3171_2021_OH | N39°29'23.63354" | W82°02'39.21312" | 781.51 | VVA |
| 3172_2021_OH | N39°43'35.71550" | W82°46'31.50817" | 849.06 | VVA |
| 3174_2021_OH | N39°42'56.83568" | W81°55'53.75772" | 869.05 | VVA |
| 3175_2021_OH | N39°43'37.19772" | W82°46'09.89228" | 866.80 | VVA |
| 3177_2021_OH | N39°53'16.23843" | W82°29'10.41351" | 921.24 | VVA |
| 3179_2021_OH | N39°37'11.19479" | W81°46'49.58706" | 881.57 | VVA |
| 3180_2021_OH | N39°40'42.45619" | W82°33'12.30279" | 686.22 | VVA |
| 3182_2021_OH | N39°52'05.21951" | W82°30'42.13942" | 813.81 | VVA |
| 3184_2021_OH | N39°40'10.36551" | W81°48'29.53023" | 874.18 | VVA |
| 3185_2021_OH | N39°50'09.41926" | W82°46'07.43230" | 654.98 | VVA |
| 3187_2021_OH | N39°41'36.23524" | W81°47'20.12585" | 584.90 | VVA |
| 3188_2021_OH | N39°55'12.59651" | W82°27'56.90112" | 792.59 | VVA |
| 3190_2021_OH | N39°43'11.13594" | W81°43'46.53235" | 899.30 | VVA |
| 3191_2021_OH | N39°39'59.88545" | W82°46'22.40542" | 793.61 | VVA |
| 3194_2021_OH | N39°32'05.98483" | W81°45'03.75396" | 557.39 | VVA |
| 3195_2021_OH | N39°49'05.92539" | W82°46'37.00365" | 714.76 | VVA |
| 3197_2021_OH | N39°42'09.15107" | W82°28'47.08597" | 700.78 | VVA |
| 3199_2021_OH | N39°40'30.17634" | W81°38'58.23428" | 698.35 | VVA |
| 3201_2021_OH | N39°40'07.83427" | W82°39'51.92927" | 979.55 | VVA |
| 3203_2021_OH | N39°37'04.63686" | W81°58'01.26047" | 983.54 | VVA |
| 3204_2021_OH | N39°49'51.20690" | W82°37'00.94466" | 777.29 | VVA |
| 3205_2021_OH | N38°57'37.99267" | W82°16'33.57681" | 639.26 | VVA |
| 3206_2021_OH | N39°33'07.88754" | W81°55'39.12098" | 694.41 | VVA |
| 3207_2021_OH | N39°40'37.90709" | W82°46'08.23109" | 822.89 | VVA |
| 3208_2021_OH | N38°55'13.64652" | W82°13'47.12314" | 479.58 | VVA |
| 3209_2021_OH | N39°45'41.03856" | W82°29'12.54464" | 879.69 | VVA |
| 3210_2021_OH | N39°01'32.37203" | W82°20'14.50390" | 506.65 | VVA |
| 3211_2021_OH | N39°42'41.66158" | W81°58'47.42005" | 883.51 | VVA |
| 3212_2021_OH | N39°53'39.63967" | W82°42'22.03266" | 822.31 | VVA |
| 3214_2021_OH | N39°34'56.03045" | W81°52'21.62821" | 662.04 | VVA |
| 3215_2021_OH | N39°43'54.71397" | W82°35'50.36247" | 860.77 | VVA |
| 3216_2021_OH | N39°39'11.48031" | W82°36'06.89771" | 1001.08 | VVA |
| 3217_2021_OH | N39°52'17.41702" | W82°28'39.72199" | 855.16 | VVA |
| 3218_2021_OH | N39°42'26.38820" | W82°23'29.07372" | 835.21 | VVA |
| 3219_2021_OH | N39°37'38.90162" | W82°35'12.67817" | 946.18 | VVA |

# GPS Control Diagram

Image 3.1 Overview of the Lidar Control Network

Image 3.2 Overview of the Lidar NVA Network

Image 3.3 Overview of the Lidar VVA Network

# NGS Datasheets

Below are the published National Geodetic Survey (NGS) datasheets for those existing monumented control stations used to establish 3-dimensional coordinates for each of the newly established project ground control survey points.

## NGS Data Sheet

AB6029  CBN         -  This is a Cooperative Base Network Control Station.

AB6029  PACS        -  This is a Primary Airport Control Station.

AB6029  DESIGNATION -  4G4 A

AB6029  PID         -  AB6029

AB6029  STATE/COUNTY-  OH/MAHONING

AB6029  COUNTRY     -  US

AB6029  USGS QUAD   -  COLUMBIANA (2016)

AB6029

AB6029                         *CURRENT SURVEY CONTROL

AB6029  ______________________________________________________________________

AB6029* NAD 83(2011) POSITION- 40 57 40.75422(N) 080 40 10.68930(W)   ADJUSTED

AB6029* NAD 83(2011) ELLIP HT-   293.474 (meters)        (06/27/12)   ADJUSTED

AB6029* NAD 83(2011) EPOCH   -  2010.00

AB6029* NAVD 88 ORTHO HEIGHT -   327.54  (meters)     1074.6   (feet) GPS OBS

AB6029  ______________________________________________________________________

AB6029  NAVD 88 orthometric height was determined with geoid model    GEOID93

AB6029  GEOID HEIGHT    -        -34.119 (meters)                     GEOID93

AB6029  GEOID HEIGHT    -        -34.025 (meters)                     GEOID18

AB6029  NAD 83(2011) X  -    782,039.567 (meters)                     COMP

AB6029  NAD 83(2011) Y  - -4,759,812.697 (meters)                     COMP

AB6029  NAD 83(2011) Z  -  4,159,372.795 (meters)                     COMP

AB6029  LAPLACE CORR    -         -1.89  (seconds)                    DEFLEC18

AB6029

AB6029  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

AB6029  Standards:

AB6029         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

AB6029            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

AB6029  -------------------------------------------------------------------

AB6029  NETWORK    1.26   2.43           0.57   0.44   1.24     -0.02975332

AB6029  -------------------------------------------------------------------

AB6029  Click here for local accuracies and other accuracy information.

AB6029

AB6029

AB6029.This mark is at Youngstown Elser Metro Airport (4G4)

AB6029

AB6029.The horizontal coordinates were established by GPS observations

AB6029.and adjusted by the National Geodetic Survey in June 2012.

AB6029

AB6029.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

AB6029.been affixed to the stable North American tectonic plate. See

AB6029.NA2011 for more information.

AB6029

AB6029.The horizontal coordinates are valid at the epoch date displayed above

AB6029.which is a decimal equivalence of Year/Month/Day.

AB6029

AB6029.The orthometric height was determined by GPS observations and a

AB6029.high-resolution geoid model.

AB6029

AB6029.GPS derived orthometric heights for airport stations designated as

AB6029.PACS or SACS are published to 2 decimal places.  This maintains

AB6029.centimeter relative accuracy between the PACS and SACS.  It does

AB6029.not indicate centimeter accuracy relative to other marks which are

AB6029.part of the NAVD 88 network.

AB6029

AB6029.Significant digits in the geoid height do not necessarily reflect accuracy.

AB6029.GEOID18 height accuracy estimate available here.

AB6029

AB6029.Click photographs - Photos may exist for this station.

AB6029

AB6029.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AB6029

AB6029.The Laplace correction was computed from DEFLEC18 derived deflections.

AB6029

AB6029.The ellipsoidal height was determined by GPS observations

AB6029.and is referenced to NAD 83.

AB6029

AB6029. The following values were computed from the NAD 83(2011) position.

AB6029

AB6029;                    North         East     Units Scale Factor Converg.

AB6029;SPC OH N     -   145,382.619   754,067.589   MT  0.99994086   +1 12 08.8

AB6029;SPC OH N     -   476,976.14  2,473,970.08   sFT  0.99994086   +1 12 08.8

AB6029;UTM  17      - 4,534,515.839   527,800.383   MT  0.99960951   +0 12 59.7

AB6029

AB6029!             -  Elev Factor  x  Scale Factor =   Combined Factor

AB6029!SPC OH N     -   0.99995397  x   0.99994086  =   0.99989483

AB6029!UTM  17      -   0.99995397  x   0.99960951  =   0.99956350

AB6029

AB6029:                Primary Azimuth Mark                     Grid Az

AB6029:SPC OH N     -  4G4 B                                    270 17 05.4

AB6029:UTM  17      -  4G4 B                                    271 16 14.5

AB6029

AB6029_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TNF2780034515(NAD 83)

AB6029

AB6029|---------------------------------------------------------------------|

AB6029| PID    Reference Object                     Distance      Geod. Az  |

AB6029|                                                           dddmmss.s |

AB6029| AC8319 4G4 B                               441.903 METERS 2712914.2 |

AB6029|---------------------------------------------------------------------|

AB6029

AB6029                          SUPERSEDED SURVEY CONTROL

AB6029

AB6029  NAD 83(2007)-  40 57 40.75439(N)    080 40 10.69005(W) AD(2002.00) 0

AB6029  ELLIP H (02/10/07)  293.488  (m)                       GP(2002.00)

AB6029  ELLIP H (03/08/05)  293.498  (m)                       GP(       ) 4 2

AB6029  NAD 83(1995)-  40 57 40.75408(N)    080 40 10.68998(W) AD(       ) B

AB6029  ELLIP H (08/20/96)  293.490  (m)                       GP(       ) 4 2

AB6029

AB6029.Superseded values are not recommended for survey control.

AB6029

AB6029.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AB6029.See file dsdata.pdf to determine how the superseded data were derived.

AB6029

AB6029_MARKER: F = FLANGE-ENCASED ROD

AB6029_SETTING: 59 = STAINLESS STEEL ROD IN SLEEVE (10 FT.+)

AB6029_STAMPING: 4G4 A 1995

AB6029_MARK LOGO: NGS

AB6029_PROJECTION: FLUSH

AB6029_MAGNETIC: N = NO MAGNETIC MATERIAL

AB6029_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

AB6029_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AB6029+SATELLITE: SATELLITE OBSERVATIONS - June 16, 2009

AB6029_ROD/PIPE-DEPTH: 5.4  meters

AB6029_SLEEVE-DEPTH  : 0.9 meters

AB6029

AB6029  HISTORY     - Date     Condition        Report By

AB6029  HISTORY     - 1995     MONUMENTED       NGS

AB6029  HISTORY     - 20090616 GOOD             GEOCAC

AB6029

AB6029                          STATION DESCRIPTION

AB6029

AB6029'DESCRIBED BY NATIONAL GEODETIC SURVEY 1995 (AJL)

AB6029'NOTE--THIS IS THE PAC STATION.  THE STATION IS LOCATED ABOUT 11 KM

AB6029'(6.85 MI) SOUTH OF YOUNGSTOWN, 2 KM (1.25 MI) NORTHWEST OF NORTH LIMA,

AB6029'2 KM (1.25 MI) SOUTHWEST OF INTERSTATE HIGHWAY 76 AT THE

AB6029'YOUNGSTOWN-ELSER METRO AIRPORT, ON THE HIGHEST SPOT OF THE AIRFIELD,

AB6029'BETWEEN THE CONNECTOR TAXI TO RUNWAY END 28 AND THE AIRPORT PERIMETER

AB6029'FENCE.  OWNERSHIP--MICHAEL E.  STANKO, 10800 SHARROTT ROAD, NORTH

AB6029'LIMA, OH.  44452.  PHONE 330-549-0337.  TO REACH FROM THE UNDERPASS AT

AB6029'THE JUNCTION OF INTERSTATE HIGHWAY 76 AND STATE HIGHWAY 7 ABOUT 1.8 KM

AB6029'(1.10 MI) NORTH OF NORTH LIMA, GO SOUTH ON HIGHWAY 7 FOR 0.88 KM (0.55

AB6029'MI) TO A PAVED ROAD RIGHT.  TURN RIGHT, WEST, ON MIDDLETOWN ROAD FOR

AB6029'0.85 KM (0.50 MI) TO THE T-JUNCTION OF SHARROTT ROAD.  TURN RIGHT,

AB6029'NORTH, ON SHARROTT ROAD FOR 0.19 KM (0.10 MI) TO THE AIRPORT ENTRANCE

AB6029'ROAD ON THE LEFT.  TURN LEFT, WEST, ON THE ROAD FOR 0.29 KM (0.20 MI)

AB6029'TO THE APRON ON THE RIGHT.  TURN RIGHT, NORTH, ACROSS THE APRON FOR

AB6029'0.08 KM (0.05 MI) TO THE PARALLEL TAXI.  TURN RIGHT, EAST, ALONG THE

AB6029'TAXI FOR 0.32 KM (0.20 MI) TO THE STATION ON THE LEFT.  THE STATION IS

AB6029'A PUNCH HOLE TOP CENTER OF A STAINLESS ROD IN A 25 CM GREASE FILLED

AB6029'SLEEVE 90 CM LONG ENCASED IN A 12.7 CM PVC PIPE WITH A LOGO CAP

AB6029'SURROUNDED BY CONCRETE SET FLUSH WITH THE GROUND.  IT IS 52.5 M (172.2

AB6029'FT) SOUTHEAST OF THE CENTER OF THE RUNWAY END, 43.3 M (142.1 FT) EAST

AB6029'OF THE CENTER OF THE CONNECTOR, 28.9 M (94.8 FT) NORTH-NORTHEAST OF

AB6029'THE NORTHEAST CORNER OF THE EAST MOST HANGAR, 16.6 M (54.5 FT) WEST OF

AB6029'THE CENTER OF, AND ABOUT 0.5 M (1.6 FT) LOWER THAN THE CENTER OF

AB6029'SHARROTT ROAD, AND 3.0 M (9.8 FT) WEST OF A FIBERGLASS WITNESS POST IN

AB6029'THE WOODEN PERIMETER FENCE.  NOTE--SLEEVE DEPTH DOES NOT MEET CLASS A

AB6029'CRITERIA.

AB6029

AB6029                          STATION RECOVERY (2009)

AB6029

AB6029'RECOVERY NOTE BY GEOCACHING 2009 (RLM)

AB6029'RECOVERED IN GOOD CONDITION.

AB6029'

JY0005  DESIGNATION -  B 27

JY0005  PID         -  JY0005

JY0005  STATE/COUNTY-  OH/LICKING

JY0005  COUNTRY     -  US

JY0005  USGS QUAD   -  THORNVILLE (2016)

JY0005

JY0005                         *CURRENT SURVEY CONTROL

JY0005  ______________________________________________________________________

JY0005* NAD 83(2011) POSITION- 39 59 13.10113(N) 082 26 08.35558(W)   ADJUSTED

JY0005* NAD 83(2011) ELLIP HT-   243.814 (meters)        (06/27/12)   ADJUSTED

JY0005* NAD 83(2011) EPOCH   -  2010.00

JY0005* NAVD 88 ORTHO HEIGHT -   277.934 (meters)      911.86  (feet) ADJUSTED

JY0005  ______________________________________________________________________

JY0005  GEOID HEIGHT    -        -34.133 (meters)                     GEOID18

JY0005  NAD 83(2011) X  -    644,220.778 (meters)                     COMP

JY0005  NAD 83(2011) Y  - -4,851,236.301 (meters)                     COMP

JY0005  NAD 83(2011) Z  -  4,077,034.063 (meters)                     COMP

JY0005  LAPLACE CORR    -          1.48  (seconds)                    DEFLEC18

JY0005  DYNAMIC HEIGHT  -        277.778 (meters)      911.34  (feet) COMP

JY0005  MODELED GRAVITY -    980,057.7   (mgal)                       NAVD 88

JY0005

JY0005  VERT ORDER      -  SECOND    CLASS 0

JY0005

JY0005  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

JY0005  Standards:

JY0005         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

JY0005            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

JY0005  -------------------------------------------------------------------

JY0005  NETWORK    1.68   3.10           0.80   0.48   1.58     -0.25436154

JY0005  -------------------------------------------------------------------

JY0005  Click here for local accuracies and other accuracy information.

JY0005

JY0005

JY0005.The horizontal coordinates were established by GPS observations

JY0005.and adjusted by the National Geodetic Survey in June 2012.

JY0005

JY0005.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

JY0005.been affixed to the stable North American tectonic plate. See

JY0005.NA2011 for more information.

JY0005

JY0005.The horizontal coordinates are valid at the epoch date displayed above

JY0005.which is a decimal equivalence of Year/Month/Day.

JY0005

JY0005.The orthometric height was determined by differential leveling and

JY0005.adjusted by the NATIONAL GEODETIC SURVEY

JY0005.in June 1991.

JY0005

JY0005.Significant digits in the geoid height do not necessarily reflect accuracy.

JY0005.GEOID18 height accuracy estimate available here.

JY0005

JY0005.Click photographs - Photos may exist for this station.

JY0005

JY0005.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JY0005

JY0005.The Laplace correction was computed from DEFLEC18 derived deflections.

JY0005

JY0005.The ellipsoidal height was determined by GPS observations

JY0005.and is referenced to NAD 83.

JY0005

JY0005.The dynamic height is computed by dividing the NAVD 88

JY0005.geopotential number by the normal gravity value computed on the

JY0005.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0005.degrees latitude (g = 980.6199 gals.).

JY0005

JY0005.The modeled gravity was interpolated from observed gravity values.

JY0005

JY0005. The following values were computed from the NAD 83(2011) position.

JY0005

JY0005;                    North         East     Units Scale Factor Converg.

JY0005;SPC OH S     -   220,588.383   605,495.720   MT  0.99999116   +0 02 27.0

JY0005;SPC OH S     -   723,713.72  1,986,530.54   sFT  0.99999116   +0 02 27.0

JY0005;UTM  17      - 4,427,298.238   377,427.431   MT  0.99978496   -0 55 21.7

JY0005

JY0005!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY0005!SPC OH S     -   0.99996175  x   0.99999116  =   0.99995291

JY0005!UTM  17      -   0.99996175  x   0.99978496  =   0.99974672

JY0005

JY0005_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLE7742727298(NAD 83)

JY0005

JY0005                          SUPERSEDED SURVEY CONTROL

JY0005

JY0005  NAD 83(2007)-  39 59 13.10122(N)    082 26 08.35628(W) AD(2002.00) 0

JY0005  ELLIP H (02/10/07)  243.841  (m)                       GP(2002.00)

JY0005  ELLIP H (10/07/05)  243.817  (m)                       GP(       ) 4 2

JY0005  NAD 83(1995)-  39 59 13.10135(N)    082 26 08.35592(W) AD(       ) 1

JY0005  ELLIP H (02/25/99)  243.834  (m)                       GP(       ) 4 2

JY0005  NAVD 88             277.93   (m)          911.8    (f) LEVELING    3

JY0005  NGVD 29 (??/??/92)  278.133  (m)          912.51   (f) ADJ UNCH    2 0

JY0005

JY0005.Superseded values are not recommended for survey control.

JY0005

JY0005.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0005.See file dsdata.pdf to determine how the superseded data were derived.

JY0005

JY0005_MARKER: DB = BENCH MARK DISK

JY0005_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0005_STAMPING: B 27 1934

JY0005_MARK LOGO: CGS

JY0005_PROJECTION: FLUSH

JY0005_MAGNETIC: O = OTHER; SEE DESCRIPTION

JY0005_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0005+STABILITY: SURFACE MOTION

JY0005_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY0005+SATELLITE: SATELLITE OBSERVATIONS - March 17, 1998

JY0005

JY0005  HISTORY     - Date     Condition        Report By

JY0005  HISTORY     - 1934     MONUMENTED       CGS

JY0005  HISTORY     - 19980317 GOOD             GEOMET

JY0005

JY0005                          STATION DESCRIPTION

JY0005

JY0005'DESCRIBED BY COAST AND GEODETIC SURVEY 1934

JY0005'1.8 MI N FROM ATHERTON.

JY0005'1.8 MILES NORTH ALONG THE BALTIMORE AND OHIO RAILROAD FROM

JY0005'ATHERTON, LICKING COUNTY, 130 FEET WEST OF A WHITE FARMHOUSE,

JY0005'100 FEET NORTHWEST OF A 32-INCH ELM TREE, AT A DIRT-ROAD

JY0005'CROSSING, 26 FEET SOUTH OF THE CENTERLINE OF THE ROAD, 20 FEE

JY0005'WEST OF THE WEST RAIL, 22 FEET SOUTHEAST OF A 12-INCH ELM TREE,

JY0005'11 FEET SOUTHEAST OF A FENCE CORNER, 3.2 FEET EAST OF THE WEST

JY0005'FENCE, AND LEVEL WITH THE TRACK.  A STANDARD DISK, STAMPED B 27

JY0005'1934 AND SET IN THE TOP OF A CONCRETE POST PROJECTING ABOUT 6

JY0005'INCHES ABOVE GROUND.

JY0005

JY0005                          STATION RECOVERY (1998)

JY0005

JY0005'RECOVERY NOTE BY GEOMETRICS GPS INCORPORATED 1998 (DAR)

JY0005'THE STATION IS LOCATED IN LICKING TOWNSHIP IN LICKING COUNTY, OHIO

JY0005'ABOUT 2.5 MILES (4.0 KM) NORTHWEST OF THE TOWN OF JACKSONTOWN.  TO

JY0005'REACH THE STATION FROM THE INTERSECTION OF US 40 AND JACKSONTOWN ROAD

JY0005'(SR 13) PROCEED NORTH ALONG JACKSONTOWN ROAD (SR 13) A DISTANCE OF

JY0005'1.85 MILES (2.98 KM) TO ITS INTERSECTION WITH DAVIS DRIVE (T-309) .

JY0005'TURN LEFT AND PROCEED WEST ALONG DAVIS DRIVE (T-309) A DISTANCE OF 1.4

JY0005'MILES (2.3 KM) TO A RAILROAD CROSSING AND THE STATION ON THE LEFT.

KY3525  CBN         -  This is a Cooperative Base Network Control Station.

KY3525  PACS        -  This is a Primary Airport Control Station.

KY3525  DESIGNATION -  CADIZ

KY3525  PID         -  KY3525

KY3525  STATE/COUNTY-  OH/HARRISON

KY3525  COUNTRY     -  US

KY3525  USGS QUAD   -  FLUSHING (2016)

KY3525

KY3525                         *CURRENT SURVEY CONTROL

KY3525  ______________________________________________________________________

KY3525* NAD 83(2011) POSITION- 40 14 16.74831(N) 081 00 45.09116(W)   ADJUSTED

KY3525* NAD 83(2011) ELLIP HT-   311.980 (meters)        (06/27/12)   ADJUSTED

KY3525* NAD 83(2011) EPOCH   -  2010.00

KY3525* NAVD 88 ORTHO HEIGHT -   345.84  (meters)     1134.6   (feet) GPS OBS

KY3525  ______________________________________________________________________

KY3525  NAVD 88 orthometric height was determined with geoid model    GEOID93

KY3525  GEOID HEIGHT    -        -33.943 (meters)                     GEOID93

KY3525  GEOID HEIGHT    -        -33.803 (meters)                     GEOID18

KY3525  NAD 83(2011) X  -    761,708.810 (meters)                     COMP

KY3525  NAD 83(2011) Y  - -4,816,053.966 (meters)                     COMP

KY3525  NAD 83(2011) Z  -  4,098,394.637 (meters)                     COMP

KY3525  LAPLACE CORR    -         -1.56  (seconds)                    DEFLEC18

KY3525

KY3525  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY3525  Standards:

KY3525         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY3525            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY3525  -------------------------------------------------------------------

KY3525  NETWORK    1.52   2.96           0.69   0.53   1.51      0.01559897

KY3525  -------------------------------------------------------------------

KY3525  Click here for local accuracies and other accuracy information.

KY3525

KY3525

KY3525.This mark is at Harrison County Airport (8G6)

KY3525

KY3525.The horizontal coordinates were established by GPS observations

KY3525.and adjusted by the National Geodetic Survey in June 2012.

KY3525

KY3525.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY3525.been affixed to the stable North American tectonic plate. See

KY3525.NA2011 for more information.

KY3525

KY3525.The horizontal coordinates are valid at the epoch date displayed above

KY3525.which is a decimal equivalence of Year/Month/Day.

KY3525

KY3525.The orthometric height was determined by GPS observations and a

KY3525.high-resolution geoid model.

KY3525

KY3525.GPS derived orthometric heights for airport stations designated as

KY3525.PACS or SACS are published to 2 decimal places.  This maintains

KY3525.centimeter relative accuracy between the PACS and SACS.  It does

KY3525.not indicate centimeter accuracy relative to other marks which are

KY3525.part of the NAVD 88 network.

KY3525

KY3525.Significant digits in the geoid height do not necessarily reflect accuracy.

KY3525.GEOID18 height accuracy estimate available here.

KY3525

KY3525.Click photographs - Photos may exist for this station.

KY3525

KY3525.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY3525

KY3525.The Laplace correction was computed from DEFLEC18 derived deflections.

KY3525

KY3525.The ellipsoidal height was determined by GPS observations

KY3525.and is referenced to NAD 83.

KY3525

KY3525. The following values were computed from the NAD 83(2011) position.

KY3525

KY3525;                    North         East     Units Scale Factor Converg.

KY3525;SPC OH N     -    64,523.417   726,578.462   MT  1.00004315   +0 58 37.9

KY3525;SPC OH N     -   211,690.58  2,383,782.84   sFT  1.00004315   +0 58 37.9

KY3525;UTM  17      - 4,454,171.917   498,934.563   MT  0.99960001   -0 00 29.1

KY3525

KY3525!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY3525!SPC OH N     -   0.99995106  x   1.00004315  =   0.99999421

KY3525!UTM  17      -   0.99995106  x   0.99960001  =   0.99955109

KY3525

KY3525:                Primary Azimuth Mark                     Grid Az

KY3525:SPC OH N     -  CADIZ AZ MK                              305 16 36.2

KY3525:UTM  17      -  CADIZ AZ MK                              306 15 43.2

KY3525

KY3525_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME9893454171(NAD 83)

KY3525

KY3525|---------------------------------------------------------------------|

KY3525| PID    Reference Object                     Distance      Geod. Az  |

KY3525|                                                           dddmmss.s |

KY3525| AD9976 8G6 A                               APPROX. 0.5 KM 1371537.5 |

KY3525| KY3528 CADIZ AZ MK                         APPROX. 0.6 KM 3061514.1 |

KY3525|---------------------------------------------------------------------|

KY3525

KY3525                          SUPERSEDED SURVEY CONTROL

KY3525

KY3525  NAD 83(2007)-  40 14 16.74849(N)    081 00 45.09187(W) AD(2002.00) 0

KY3525  ELLIP H (02/10/07)  311.995  (m)                       GP(2002.00)

KY3525  ELLIP H (03/08/05)  311.993  (m)                       GP(       ) 4 2

KY3525  NAD 83(1995)-  40 14 16.74840(N)    081 00 45.09177(W) AD(       ) B

KY3525  ELLIP H (08/20/96)  312.013  (m)                       GP(       ) 4 2

KY3525  NAD 83(1986)-  40 14 16.75409(N)    081 00 45.10070(W) AD(       ) 3

KY3525  NAD 27      -  40 14 16.51527(N)    081 00 45.74305(W) AD(       ) 3

KY3525  NGVD 29 (02/23/89)  346.07   (m)  RAPSU86 model used   GPS OBS

KY3525

KY3525.Superseded values are not recommended for survey control.

KY3525

KY3525.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY3525.See file dsdata.pdf to determine how the superseded data were derived.

KY3525

KY3525_MARKER: DH = HORIZONTAL CONTROL DISK

KY3525_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY3525_STAMPING: CADIZ 1986

KY3525_MARK LOGO: NGS

KY3525_PROJECTION: FLUSH

KY3525_MAGNETIC: N = NO MAGNETIC MATERIAL

KY3525_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY3525+STABILITY: SURFACE MOTION

KY3525_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY3525+SATELLITE: SATELLITE OBSERVATIONS - December 08, 2012

KY3525

KY3525  HISTORY     - Date     Condition        Report By

KY3525  HISTORY     - 1986     MONUMENTED       NGS

KY3525  HISTORY     - 19871008 GOOD

KY3525  HISTORY     - 19950829 GOOD             NGS

KY3525  HISTORY     - 20121208 GOOD             GEOCAC

KY3525

KY3525                          STATION DESCRIPTION

KY3525

KY3525'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

KY3525'THE STATION IS LOCATED AT THE CADIZ - HARRISON COUNTY AIRPORT 2.5 MILE

KY3525'SOUTH OF CADIZ.  AT THE SOUTHWEST EDGE OF THE RUNWAY AND JUST NORTH

KY3525'OF THE ADMINISTRATION BUILDING.

KY3525'STATION MARKS ARE STANDARD DISKS STAMPED --CADIZ 1986--.  THE SURFACE

KY3525'MARK IS SET IN A 25 CM ( 10 INCH ) DIAMETER FLUSH CONCRETE MONUMENT

KY3525'AND IS 86.0 METERS (282 FEET) NORTH OF THE EAST CORNER OF THE METAL

KY3525'HANGER AT THE ADMINISTRATION BUILDING, 47.2 METERS (155 FEET) NORTH

KY3525'OF TWIN GAS PUMPS, 10.2 METERS (33.5 FEET) NORTHEAST OF THE NORTH-

KY3525'EAST EDGE OF A TAXIWAY AND 4.9 METERS (16 FEET) SOUTHWEST OF THE

KY3525'SOUTHWEST EDGE OF THE RUNWAY.  THE UNDERGROUND MARK IS SET 91 CM

KY3525'(36 INCHES) BELOW GROUND LEVEL.

KY3525'TYPED BY JAMES MALONEY (/04/87.

KY3525

KY3525                          STATION RECOVERY (1987)

KY3525

KY3525'RECOVERED 1987

KY3525'RECOVERED IN GOOD CONDITION.

KY3525

KY3525                          STATION RECOVERY (1995)

KY3525

KY3525'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY3525'THE STATION IS LOCATED ABOUT 4.0 KM (2.50 MI) SOUTH OF CADIZ AT THE

KY3525'HARRISON COUNTY AIRPORT.  IN THE GRASS, NEAR THE ROUNDED NORTH CORNER

KY3525'OF THE GRASS MEDIAN, BETWEEN THE RUNWAY AND TAXI, IN FRONT OF THE

KY3525'OFFICE.  OWNERSHIP--HARRISON COUNTY, 43000 AIRPORT ROAD, CADIZ, OH.

KY3525'43907.  AIRPORT MANAGER MICHAEL A.  MIHELARAKIS, PHONE 614-942-8313 OR

KY3525'614-937-2652.  NOTE--CONTACT 24 HOURS IN ADVANCE.  TO REACH THE

KY3525'STATION FROM THE JUNCTION OF U.S.  HIGHWAY 250 AND STATE HIGHWAY 9 IN

KY3525'CADIZ, GO SOUTH 4.58 KM (2.85 MI) ALONG HIGHWAY 9 TO THE AIRPORT

KY3525'ENTRANCE ROAD ON THE FAR RIGHT.  TURN RIGHT, WESTERLY, 0.96 KM (0.60

KY3525'MI) ALONG HARRISON COUNTY 38, CROSSING A HAUL ROAD, TO A GATE AND THE

KY3525'OFFICE ON THE LEFT.  PASS THROUGH GATE 0.04 KM (0.00 MI) ACROSS APRON

KY3525'TO A RAMP.  TURN RIGHT, NORTHEAST, 0.03 KM (0.00 MI) ACROSS TAXI TO

KY3525'THE RUNWAY AND THE STATION ON THE RIGHT.  STATION IS 20.3 M (66.6 FT)

KY3525'SOUTHEAST OF THE RAMP CENTERLINE, 16.3 M (53.5 FT) SOUTHWEST OF THE

KY3525'RUNWAY CENTERLINE, 14.2 M (46.6 FT) NORTHEAST OF THE TAXI CENTERLINE,

KY3525'10.5 M (34.4 FT) NORTHWEST OF A RUNWAY LIGHT, 5.3 M (17.4 FT)

KY3525'SOUTHEAST OF A TAXI LIGHT, 0.4 M (1.3 FT) SOUTH OF A WITNESS POST, AND

KY3525'THE MONUMENT IS 0.3 M (1.0 FT) BELOW THE LEVEL OF THE RUNWAY AND FLUSH

KY3525'WITH THE GROUND SURFACE.  BY R.G.  HAYES

KY3525

KY3529  CBN         -  This is a Cooperative Base Network Control Station.

KY3529  PACS        -  This is a Primary Airport Control Station.

KY3529  DESIGNATION -  CARPORT

KY3529  PID         -  KY3529

KY3529  STATE/COUNTY-  OH/CARROLL

KY3529  COUNTRY     -  US

KY3529  USGS QUAD   -  CARROLLTON (2016)

KY3529

KY3529                         *CURRENT SURVEY CONTROL

KY3529  ______________________________________________________________________

KY3529* NAD 83(2011) POSITION- 40 33 42.17491(N) 081 04 42.29895(W)   ADJUSTED

KY3529* NAD 83(2011) ELLIP HT-   320.422 (meters)        (06/27/12)   ADJUSTED

KY3529* NAD 83(2011) EPOCH   -  2010.00

KY3529* NAVD 88 ORTHO HEIGHT -   354.12  (meters)     1161.8   (feet) GPS OBS

KY3529  ______________________________________________________________________

KY3529  NAVD 88 orthometric height was determined with geoid model    GEOID93

KY3529  GEOID HEIGHT    -        -33.779 (meters)                     GEOID93

KY3529  GEOID HEIGHT    -        -33.688 (meters)                     GEOID18

KY3529  NAD 83(2011) X  -    752,557.425 (meters)                     COMP

KY3529  NAD 83(2011) Y  - -4,793,915.529 (meters)                     COMP

KY3529  NAD 83(2011) Z  -  4,125,777.035 (meters)                     COMP

KY3529  LAPLACE CORR    -         -2.02  (seconds)                    DEFLEC18

KY3529

KY3529  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY3529  Standards:

KY3529         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY3529            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY3529  -------------------------------------------------------------------

KY3529  NETWORK    1.23   2.43           0.57   0.41   1.24     -0.01870386

KY3529  -------------------------------------------------------------------

KY3529  Click here for local accuracies and other accuracy information.

KY3529

KY3529

KY3529.This mark is at Carroll Co-Tolson Airport (TSO)

KY3529

KY3529.The horizontal coordinates were established by GPS observations

KY3529.and adjusted by the National Geodetic Survey in June 2012.

KY3529

KY3529.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY3529.been affixed to the stable North American tectonic plate. See

KY3529.NA2011 for more information.

KY3529

KY3529.The horizontal coordinates are valid at the epoch date displayed above

KY3529.which is a decimal equivalence of Year/Month/Day.

KY3529

KY3529.The orthometric height was determined by GPS observations and a

KY3529.high-resolution geoid model.

KY3529

KY3529.GPS derived orthometric heights for airport stations designated as

KY3529.PACS or SACS are published to 2 decimal places.  This maintains

KY3529.centimeter relative accuracy between the PACS and SACS.  It does

KY3529.not indicate centimeter accuracy relative to other marks which are

KY3529.part of the NAVD 88 network.

KY3529

KY3529.Significant digits in the geoid height do not necessarily reflect accuracy.

KY3529.GEOID18 height accuracy estimate available here.

KY3529

KY3529.Click photographs - Photos may exist for this station.

KY3529

KY3529.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY3529

KY3529.The Laplace correction was computed from DEFLEC18 derived deflections.

KY3529

KY3529.The ellipsoidal height was determined by GPS observations

KY3529.and is referenced to NAD 83.

KY3529

KY3529. The following values were computed from the NAD 83(2011) position.

KY3529

KY3529;                    North         East     Units Scale Factor Converg.

KY3529;SPC OH N     -   100,373.103   720,385.972   MT  0.99997788   +0 56 02.1

KY3529;SPC OH N     -   329,307.42  2,363,466.31   sFT  0.99997788   +0 56 02.1

KY3529;UTM  17      - 4,490,108.106   493,361.571   MT  0.99960054   -0 03 03.6

KY3529

KY3529!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY3529!SPC OH N     -   0.99994974  x   0.99997788  =   0.99992762

KY3529!UTM  17      -   0.99994974  x   0.99960054  =   0.99955030

KY3529

KY3529:                Primary Azimuth Mark                     Grid Az

KY3529:SPC OH N     -  CARPORT AZ MK                            243 26 36.1

KY3529:UTM  17      -  CARPORT AZ MK                            244 25 41.8

KY3529

KY3529_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME9336190108(NAD 83)

KY3529

KY3529|---------------------------------------------------------------------|

KY3529| PID    Reference Object                     Distance      Geod. Az  |

KY3529|                                                           dddmmss.s |

KY3529| KY3532 CARPORT AZ MK                       APPROX. 0.6 KM 2442238.2 |

KY3529|---------------------------------------------------------------------|

KY3529

KY3529                          SUPERSEDED SURVEY CONTROL

KY3529

KY3529  NAD 83(2007)-  40 33 42.17509(N)    081 04 42.29967(W) AD(2002.00) 0

KY3529  ELLIP H (02/10/07)  320.436  (m)                       GP(2002.00)

KY3529  ELLIP H (03/08/05)  320.441  (m)                       GP(       ) 4 2

KY3529  NAD 83(1995)-  40 33 42.17491(N)    081 04 42.29961(W) AD(       ) B

KY3529  ELLIP H (08/20/96)  320.449  (m)                       GP(       ) 4 2

KY3529  NAD 83(1986)-  40 33 42.18242(N)    081 04 42.30533(W) AD(       ) 3

KY3529  NAD 27      -  40 33 41.94519(N)    081 04 42.91058(W) AD(       ) 3

KY3529  NGVD 29 (02/23/89)  354.17   (m)  RAPSU86 model used   GPS OBS

KY3529

KY3529.Superseded values are not recommended for survey control.

KY3529

KY3529.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY3529.See file dsdata.pdf to determine how the superseded data were derived.

KY3529

KY3529_MARKER: DH = HORIZONTAL CONTROL DISK

KY3529_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY3529_STAMPING: CARPORT 1986

KY3529_MARK LOGO: NGS

KY3529_MAGNETIC: N = NO MAGNETIC MATERIAL

KY3529_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY3529+STABILITY: SURFACE MOTION

KY3529_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY3529+SATELLITE: SATELLITE OBSERVATIONS - January 20, 2008

KY3529

KY3529  HISTORY     - Date     Condition        Report By

KY3529  HISTORY     - 1986     MONUMENTED       NGS

KY3529  HISTORY     - 19871009 GOOD

KY3529  HISTORY     - 19950809 GOOD             NGS

KY3529  HISTORY     - 20080120 GOOD             GEOCAC

KY3529

KY3529                          STATION DESCRIPTION

KY3529

KY3529'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

KY3529'THE STATION IS LOCATED AT THE CARROLLTON/CARROLL COUNTY AIRPORT,

KY3529'ABOUT 1 MILE SOUTHEAST OF THE TOWN OF CARROLLTON.  OWNERSHIP CARROLL

KY3529'COUNTY, C/O AIRPORT MANAGER WALT YOUNG, 253 AIRPORT ROAD, CARROLLTON,

KY3529'OHIO 44615, PHONE 216-627-5250.

KY3529'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 43, 9  AND

KY3529'39 ABOUT 0.5 MILES SOUTHEAST OF CARROLLTON.  GO SOUTHEAST ON STATE

KY3529'HIGHWAYS 43 AND 9 FOR 0.45 MILE TO AIRPORT ROAD ON THE RIGHT.  TURN

KY3529'RIGHT, SOUTH, AND GO FOR 0.4 MILE TO  A FORK.  BEAR LEFT AND GO

KY3529'FOR 0.1 MILE TO THE AIRPORT ADMINISTRATION BUILDING ON THE RIGHT

KY3529'CONTINUE AHEAD FOR ABOUT 30 METERS TO A GATE ON THE LEFT.  TURN

KY3529'LEFT THUR THE GATE AND GO SOUTH ACROSS THE TARMAC FOR 0.1 MILE

KY3529'TO THE RUNWAY AND THE STATION ON THE LEFT.  TO REACH THE AZIMUTH FROM

KY3529'THE STATION, GO SOUTHWEST PARALLELING THE RUNWAY (IN THE GRASS) FOR

KY3529'0.35 MILES TO THE AZIMUTH MARK ON THE SOUTHWEST SIDE OF A TURN AROUND.

KY3529'THE STATION MARKS ARE STANDARD NGS DISKS STAMPED --CARPORT 1986--. THE

KY3529'SURFACE MARK IS SET IN THE TOP OF A 30 CM ROUND CONCRETE POST FLUSH

KY3529'WITH THE GROUND.  IT IS 17.7 METERS (58.0 FEET) NORTH-NORTHWEST OF THE

KY3529'RUNWAY CENTERLINE, 16.2 METERS (53.0 FEET) EAST-NORTHEAST OF THE RAMP

KY3529'CENTERLINE, 38.7 METERS (127 FEET) SOUTH SOUTHWEST OF A CHAINLINK

KY3529'FENCE CORNER AND 0.3 METER (1.0 FOOT) WEST OF A FIBERGLASS WITNESS

KY3529'POST.  THE UNDERGROUND MARK IS SET IN A IRREGULAR MASS OF CONCRETE

KY3529'ABOUT 1.0 METER BELOW THE GROUND LEVEL.

KY3529'DESCRIBED BY B.L. LAMBERT.  TYPED BY JAMES MALONEY 09/04/87.

KY3529

KY3529                          STATION RECOVERY (1987)

KY3529

KY3529'RECOVERED 1987

KY3529'RECOVERED IN GOOD CONDITION.

KY3529

KY3529                          STATION RECOVERY (1995)

KY3529

KY3529'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY3529'THE STATION IS LOCATED ABOUT 1.6 KM (1.00 MI) SOUTHEAST OF CARROLLTON,

KY3529'AT THE CARROLL COUNTY AIRPORT.  IN THE GRASS, NORTHEAST OF THE TAXI

KY3529'AND NORTHWEST OF THE RUNWAY.  OWNERSHIP--CARROLL COUNTY, 253 AIRPORT

KY3529'ROAD, CARROLLTON, OHIO, 44615.  AIRPORT MANAGER ALAN MILLER, PHONE

KY3529'330-627-5250.  TO REACH THE STATION FROM THE JUNCTION OF STATE

KY3529'HIGHWAYS 43, 39 AND 9 ABOUT 0.8 KM (0.50 MI) SOUTHEAST OF CARROLLTON,

KY3529'GO SOUTHEAST 0.72 KM (0.45 MI) ON HIGHWAYS 43 AND 9 TO AIRPORT ROAD ON

KY3529'THE RIGHT.  TURN RIGHT, SOUTH, 0.64 KM (0.40 MI) TO A FORK.  BEAR

KY3529'RIGHT, WEST, 0.16 KM (0.10 MI) ON ALLEGHANY ROAD TO THE ENTRANCE AND

KY3529'ADMINISTRATION BUILDING ON THE LEFT.  TURN LEFT, SOUTH, ABOUT 30 M

KY3529'(98.4 FT) TO A GATE ON THE RIGHT.  TURN RIGHT, SOUTHWEST, THROUGH THE

KY3529'GATE, THEN SOUTHEAST, 0.16 KM (0.10 MI) ACROSS APRON AND ALONG TAXI TO

KY3529'THE STATION ON THE LEFT.  STATION IS 38.7 M (127.0 FT) SOUTH-SOUTHEAST

KY3529'OF A CHAIN LINK FENCE END, 17.6 M (57.7 FT) NORTH-NORTHWEST OF THE

KY3529'RUNWAY CENTERLINE, 16.1 M (52.8 FT) EAST-NORTHEAST OF THE TAXI

KY3529'CENTERLINE, 0.3 M (1.0 FT) WEST OF A WITNESS POST, AND THE MONUMENT IS

KY3529'LEVEL WITH THE TAXI AND FLUSH WITH THE GROUND SURFACE.

KY3529

KY3529                          STATION RECOVERY (2008)

KY3529

KY3529'RECOVERY NOTE BY GEOCACHING 2008 (RLM)

KY3529'RECOVERED IN GOOD CONDITION.

AB5587  FBN         -  This is a Federal Base Network Control Station.

AB5587  DESIGNATION -  CAUSEWAY

AB5587  PID         -  AB5587

AB5587  STATE/COUNTY-  OH/KNOX

AB5587  COUNTRY     -  US

AB5587  USGS QUAD   -  BUTLER (2016)

AB5587

AB5587                         *CURRENT SURVEY CONTROL

AB5587  ______________________________________________________________________

AB5587* NAD 83(2011) POSITION- 40 31 06.95465(N) 082 29 00.84357(W)   ADJUSTED

AB5587* NAD 83(2011) ELLIP HT-   302.490 (meters)        (06/27/12)   ADJUSTED

AB5587* NAD 83(2011) EPOCH   -  2010.00

AB5587* NAVD 88 ORTHO HEIGHT -   336.172 (meters)     1102.92  (feet) ADJUSTED

AB5587  ______________________________________________________________________

AB5587  GEOID HEIGHT    -        -33.661 (meters)                     GEOID18

AB5587  NAD 83(2011) X  -    635,179.774 (meters)                     COMP

AB5587  NAD 83(2011) Y  - -4,814,000.146 (meters)                     COMP

AB5587  NAD 83(2011) Z  -  4,122,126.635 (meters)                     COMP

AB5587  LAPLACE CORR    -          2.61  (seconds)                    DEFLEC18

AB5587  DYNAMIC HEIGHT  -        336.001 (meters)     1102.36  (feet) COMP

AB5587  MODELED GRAVITY -    980,108.8   (mgal)                       NAVD 88

AB5587

AB5587  VERT ORDER      -  SECOND    CLASS II

AB5587

AB5587  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

AB5587  Standards:

AB5587         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

AB5587            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

AB5587  -------------------------------------------------------------------

AB5587  NETWORK    0.39   0.98           0.18   0.13   0.50      0.10199609

AB5587  -------------------------------------------------------------------

AB5587  Click here for local accuracies and other accuracy information.

AB5587

AB5587

AB5587.The horizontal coordinates were established by GPS observations

AB5587.and adjusted by the National Geodetic Survey in June 2012.

AB5587

AB5587.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

AB5587.been affixed to the stable North American tectonic plate. See

AB5587.NA2011 for more information.

AB5587

AB5587.The horizontal coordinates are valid at the epoch date displayed above

AB5587.which is a decimal equivalence of Year/Month/Day.

AB5587

AB5587.The orthometric height was determined by differential leveling and

AB5587.adjusted by the NATIONAL GEODETIC SURVEY

AB5587.in August 1996.

AB5587

AB5587.Significant digits in the geoid height do not necessarily reflect accuracy.

AB5587.GEOID18 height accuracy estimate available here.

AB5587

AB5587.Click photographs - Photos may exist for this station.

AB5587

AB5587.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AB5587

AB5587.The Laplace correction was computed from DEFLEC18 derived deflections.

AB5587

AB5587.The ellipsoidal height was determined by GPS observations

AB5587.and is referenced to NAD 83.

AB5587

AB5587.The dynamic height is computed by dividing the NAVD 88

AB5587.geopotential number by the normal gravity value computed on the

AB5587.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

AB5587.degrees latitude (g = 980.6199 gals.).

AB5587

AB5587.The modeled gravity was interpolated from observed gravity values.

AB5587

AB5587. The following values were computed from the NAD 83(2011) position.

AB5587

AB5587;                    North         East     Units Scale Factor Converg.

AB5587;SPC OH N     -    94,604.269   601,392.527   MT  0.99998476   +0 00 38.9

AB5587;SPC OH N     -   310,380.84  1,973,068.65   sFT  0.99998476   +0 00 38.9

AB5587;UTM  17      - 4,486,376.396   374,324.096   MT  0.99979442   -0 57 50.4

AB5587

AB5587!             -  Elev Factor  x  Scale Factor =   Combined Factor

AB5587!SPC OH N     -   0.99995255  x   0.99998476  =   0.99993731

AB5587!UTM  17      -   0.99995255  x   0.99979442  =   0.99974698

AB5587

AB5587_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TLE7432486376(NAD 83)

AB5587

AB5587                          SUPERSEDED SURVEY CONTROL

AB5587

AB5587  NAD 83(2007)-  40 31 06.95477(N)    082 29 00.84432(W) AD(2002.00) 0

AB5587  ELLIP H (02/10/07)  302.503  (m)                       GP(2002.00)

AB5587  ELLIP H (09/23/04)  302.512  (m)                       GP(       ) 4 1

AB5587  NAD 83(1995)-  40 31 06.95506(N)    082 29 00.84459(W) AD(       ) B

AB5587  ELLIP H (08/20/96)  302.510  (m)                       GP(       ) 4 2

AB5587  NAVD 88             336.17   (m)         1102.9    (f) LEVELING    3

AB5587

AB5587.Superseded values are not recommended for survey control.

AB5587

AB5587.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AB5587.See file dsdata.pdf to determine how the superseded data were derived.

AB5587

AB5587_MARKER: DH = HORIZONTAL CONTROL DISK

AB5587_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

AB5587_STAMPING: CAUSEWAY 1987

AB5587_MARK LOGO: NGS

AB5587_PROJECTION: FLUSH

AB5587_MAGNETIC: R = STEEL ROD IMBEDDED IN MONUMENT

AB5587_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

AB5587+STABILITY: SURFACE MOTION

AB5587_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AB5587+SATELLITE: SATELLITE OBSERVATIONS - January 29, 2020

AB5587

AB5587  HISTORY     - Date     Condition        Report By

AB5587  HISTORY     - 1987     MONUMENTED       OHDNR

AB5587  HISTORY     - 19950914 GOOD             NGS

AB5587  HISTORY     - 20030710 GOOD             OHDT

AB5587  HISTORY     - 20170313 GOOD             OHDNR

AB5587  HISTORY     - 20200129 GOOD             WOOLPT

AB5587

AB5587                          STATION DESCRIPTION

AB5587

AB5587'DESCRIBED BY OHIO DEPARTMENT OF NATURAL RESOURCES 1987 (RLS)

AB5587'STATION DESCRIBED 1995.  THIS STATION WAS SET IN 1987 BY THE OHIO

AB5587'DEPARTMENT OF NATURAL RESOURCES.  THE STATION IS LOCATED ABOUT 6.4 KM

AB5587'(3.95 MI) NORTHEAST OF FREDERICKTOWN ON THE CAUSEWAY AT THE NORTHEAST

AB5587'END OF KNOX LAKE.  TO REACH THE STATION FROM THE FREDERICKTOWN

AB5587'MUNICIPAL BUILDING AT THE JUNCTION OF STATE ROUTE 13 AND 95, GO NORTH

AB5587'0.24 KM (0.15 MI) ON ROUTE 13 AND 95 TO A JUNCTION, TURN RIGHT

AB5587'(CROSSING NEW BRIDGE OVER THE NORTH BRANCH KOKOSING RIVER) AND GO 5.5

AB5587'KM (3.40 MI) EAST-NORTHEAST ON COUNTY ROAD 14 TO THE JUNCTION WITH

AB5587'KNOX LAKE ROAD, TURN LEFT AND GO 2.4 KM (1.50 MI) ON KNOX LAKE ROAD TO

AB5587'THE CAUSEWAY AND THE STATION ON THE RIGHT, 6.1 M (20.0 FT) WEST OF THE

AB5587'WATERS EDGE.  THE STATION MARKS ARE STANDARD NGS DISKS STAMPED

AB5587'--CAUSEWAY 1987--.  THE SURFACE DISK IS SET IN TOP OF A 0.3 M (1.0 FT)

AB5587'DIAMETER, FLUSH CONCRETE MONUMENT WITH A STEEL ROD EMBEDDED IN IT.  IT

AB5587'IS 85.6 M (280.8 FT) SOUTH OF THE EAST END OF A 1.8 M (5.9 FT)

AB5587'DIAMETER STEEL CULVERT PIPE UNDER THE ROAD, 10.3 M (33.8 FT) NORTH OF

AB5587'A UTILITY POLE, 5.6 M (18.4 FT) EAST OF THE ROAD CENTER, 0.3 M (1.0

AB5587'FT) BELOW SAME, AND 3.0 M (9.8 FT) WEST OF A FIBERGLASS WITNESS POST.

AB5587'THE UNDERGROUND MARK IS SET IN AN IRREGULAR MASS OF CONCRETE RECESSED

AB5587'1 M (3.3 FT) .  STATIONS RAMP 1987 AND MAPLE 1987 ARE BOTH VISIBLE AT

AB5587'EYE LEVEL FROM THIS STATION.

AB5587

AB5587                          STATION RECOVERY (1995)

AB5587

AB5587'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

AB5587'STATION IS ABOUT 6.4 KM (3.95 MI) NORTHEAST OF FREDERICKTOWN ON THE

AB5587'CAUSEWAY AT THE NORTHEAST END OF KNOX LAKE. TO REACH THE STATION FROM

AB5587'THE FREDERICKTOWN MUNICIPAL BUILDING AT THE JUNCTION OF STATE ROUTE 13

AB5587'AND 95, GO NORTH 0.24 KM (0.15 MI) ON ROUTE 13 AND 95 TO A JUNCTION,

AB5587'TURN RIGHT (CROSSING NEW BRIDGE OVER THE NORTH BRANCH KOKOSING RIVER)

AB5587'AND GO 5.5 KM (3.40 MI) EAST-NORTHEAST ON COUNTY ROAD 14 TO THE

AB5587'JUNCTION WITH KNOX LAKE ROAD, TURN LEFT AND GO 2.4 KM (1.50 MI) ON

AB5587'KNOX LAKE ROAD TO THE CAUSEWAY AND THE STATION ON THE RIGHT, 6.1 M

AB5587'(20.0 FT) WEST OF THE WATERS EDGE.  THE STATION MARKS ARE STANDARD NGS

AB5587'DISKS STAMPED --CAUSEWAY 1987--.  THE SURFACE DISK IS SET IN TOP OF A

AB5587'0.3 M (1.0 FT) DIAMETER, FLUSH CONCRETE MONUMENT WITH A STEEL ROD

AB5587'EMBEDDED IN IT.  IT IS 85.6 M (280.8 FT) SOUTH OF THE EAST END OF A

AB5587'1.8 M (5.9 FT) DIAMETER STEEL CULVERT PIPE UNDER THE ROAD, 10.3 M

AB5587'(33.8 FT) NORTH OF A UTILITY POLE, 5.6 M (18.4 FT) EAST OF THE ROAD

AB5587'CENTER, 0.3 M (1.0 FT) BELOW SAME, AND 3.0 M (9.8 FT) WEST OF A

AB5587'FIBERGLASS WITNESS POST.  THE UNDERGROUND MARK IS SET IN AN IRREGULAR

AB5587'MASS OF CONCRETE RECESSED 1 M (3.3 FT) .  STATIONS RAMP 1987 AND MAPLE

AB5587'1987 ARE BOTH VISIBLE AT EYE LEVEL FROM THIS STATION.

AB5587

AB5587                          STATION RECOVERY (2003)

AB5587

AB5587'RECOVERY NOTE BY OHIO DEPARTMENT OF TRANSPORTATION 2003 (JAS)

AB5587'RECOVERED AS DESCRIBED.

AB5587

AB5587                          STATION RECOVERY (2017)

AB5587

AB5587'RECOVERY NOTE BY OHIO DEPARTMENT OF NATURAL RESOURCES 2017 (WAM)

AB5587'FOR MORE INFORMATION CONTACT

AB5587'OHIO DEPARTMENT OF NATURAL RESOURCES

AB5587'OFFICE OF REAL ESTATE

AB5587'SURVEY SECTION

AB5587'2045 MORSE RD. BLDG. E2

AB5587'COLUMBUS, OH 43229

AB5587

AB5587                          STATION RECOVERY (2020)

AB5587

AB5587'RECOVERY NOTE BY WOOLPERT CONSULTANTS 2020 (GS)

AB5587'RECOVERED IN GOOD CONDITION.

JX2283  DESIGNATION -  COM 1

JX2283  PID         -  JX2283

JX2283  STATE/COUNTY-  OH/WASHINGTON

JX2283  COUNTRY     -  US

JX2283  USGS QUAD   -  MARIETTA (2016)

JX2283

JX2283                         *CURRENT SURVEY CONTROL

JX2283  ______________________________________________________________________

JX2283* NAD 83(1995) POSITION- 39 24 36.68902(N) 081 27 27.25832(W)   ADJUSTED

JX2283* NAVD 88 ORTHO HEIGHT -   185.27  (meters)      607.8   (feet) LEVELING

JX2283  ______________________________________________________________________

JX2283  GEOID HEIGHT    -        -34.265 (meters)                     GEOID18

JX2283  LAPLACE CORR    -          1.49  (seconds)                    DEFLEC18

JX2283  HORZ ORDER      -  FIRST

JX2283  VERT ORDER      -  THIRD ?

JX2283

JX2283.The horizontal coordinates were established by GPS observations

JX2283.and adjusted by the National Geodetic Survey in April 1998.

JX2283

JX2283.The orthometric height was determined by differential leveling.

JX2283.The vertical network tie was performed by a horz. field party for horz.

JX2283.obs reductions. Reset procedures were used to establish the elevation.

JX2283

JX2283.Significant digits in the geoid height do not necessarily reflect accuracy.

JX2283.GEOID18 height accuracy estimate available here.

JX2283

JX2283.Click photographs - Photos may exist for this station.

JX2283

JX2283.The Laplace correction was computed from DEFLEC18 derived deflections.

JX2283

JX2283. The following values were computed from the NAD 83(1995) position.

JX2283

JX2283;                    North         East     Units Scale Factor Converg.

JX2283;SPC OH S     -   157,068.128   689,770.359   MT  0.99993601   +0 39 41.2

JX2283;SPC OH S     -   515,314.35  2,263,021.59   sFT  0.99993601   +0 39 41.2

JX2283;SPC WV N     -   102,883.176   431,428.217   MT  0.99994780   -1 14 54.6

JX2283;SPC WV N     -   337,542.55  1,415,444.08   sFT  0.99994780   -1 14 54.6

JX2283;UTM  17      - 4,362,397.433   460,607.768   MT  0.99961911   -0 17 25.8

JX2283

JX2283!             -  Elev Factor  x  Scale Factor =   Combined Factor

JX2283!SPC OH S     -   0.99997631  x   0.99993601  =   0.99991232

JX2283!SPC WV N     -   0.99997631  x   0.99994780  =   0.99992411

JX2283!UTM  17      -   0.99997631  x   0.99961911  =   0.99959543

JX2283

JX2283_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SMD6060762397(NAD 83)

JX2283

JX2283                          SUPERSEDED SURVEY CONTROL

JX2283

JX2283  NAD 83(1986)-  39 24 36.69319(N)    081 27 27.26633(W) AD(       ) 1

JX2283

JX2283.Superseded values are not recommended for survey control.

JX2283

JX2283.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JX2283.See file dsdata.pdf to determine how the superseded data were derived.

JX2283

JX2283_MARKER: DD = SURVEY DISK

JX2283_SETTING: 9 = SET IN PREFABRICATED CONCRETE POST IMBEDDED IN GROUND

JX2283_STAMPING: COM 1 1992

JX2283_MARK LOGO: OH4730

JX2283_MAGNETIC: R = STEEL ROD IMBEDDED IN MONUMENT

JX2283_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JX2283+STABILITY: SURFACE MOTION

JX2283_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JX2283+SATELLITE: SATELLITE OBSERVATIONS - 1992

JX2283

JX2283  HISTORY     - Date     Condition        Report By

JX2283  HISTORY     - 1992     MONUMENTED       GEOMET

JX2283  HISTORY     - 20080816 GOOD             GEOCAC

JX2283

JX2283                          STATION DESCRIPTION

JX2283

JX2283'DESCRIBED BY GEOMETRICS GPS INCORPORATED 1992

JX2283'THE STATION IS LOCATED IN WASHINGTON COUNTY, OHIO ABOUT 0.7 MILES

JX2283'SOUTHWEST OF THE CITY OF MARIETTA ON FORT STREET.

JX2283'TO REACH THE STATION FROM THE INTERSECTION OF STATE ROUTE 7 (WEST END

JX2283'OF BRIDGE) AND FORT STREET PROCEED SOUTHEAST ALONG FORT STREET 0.65

JX2283'MILESTO ITS INTERSECTION WITH RAILROAD OVERPASS. CONTINUE AND PROCEED

JX2283'SOUTHEAST ALONG FORT STREET 450.0 FEETTO THE STATION ON THE LEFT

JX2283'OPPOSITE AND ACROSS STREET FROM HOUSE NUMBER 407 LEVI BARBER.

JX2283'THE STATION IS AN ALUMINUM DISK SET IN CONCRETE FLUSH WITH THE GROUND

JX2283'STAMPED COM 1 1992.  THE STATION IS 9.55 FEETEAST FROM THE EDGE OF

JX2283'PAVEMENT OF FORT STREET, APPROXIMATELY 75.0 FEETEAST FROM THE

JX2283'SOUTHEAST CORNER OF LEVI BARBER HOUSE 1829, 34.6 FEETSOUTHWEST FROM

JX2283'POWER POLE NUMBER 16501, 13.0 FEETWEST FROM THE TOP OF BANK AND

JX2283'APPROXIMATELY 435.0 FEETSOUTH FROM USC AND GS BM T235 WHICH WAS

JX2283'LEVELED TO.

JX2283

JX2283                          STATION RECOVERY (2008)

JX2283

JX2283'RECOVERY NOTE BY GEOCACHING 2008 (BP)

JX2283'15.75 FEET NORTHWEST FROM A HISTORICAL MARKER.

JX1427  DESIGNATION -  D 184

JX1427  PID         -  JX1427

JX1427  STATE/COUNTY-  OH/MUSKINGUM

JX1427  COUNTRY     -  US

JX1427  USGS QUAD   -  ZANESVILLE EAST (2016)

JX1427

JX1427                         *CURRENT SURVEY CONTROL

JX1427  ______________________________________________________________________

JX1427* NAD 83(2011) POSITION- 39 59 22.56127(N) 081 53 57.85098(W)   ADJUSTED

JX1427* NAD 83(2011) ELLIP HT-   214.695 (meters)        (06/27/12)   ADJUSTED

JX1427* NAD 83(2011) EPOCH   -  2010.00

JX1427* NAVD 88 ORTHO HEIGHT -   248.492 (meters)      815.26  (feet) ADJUSTED

JX1427  ______________________________________________________________________

JX1427  GEOID HEIGHT    -        -33.797 (meters)                     GEOID18

JX1427  NAD 83(2011) X  -    689,566.753 (meters)                     COMP

JX1427  NAD 83(2011) Y  - -4,844,786.696 (meters)                     COMP

JX1427  NAD 83(2011) Z  -  4,077,238.911 (meters)                     COMP

JX1427  LAPLACE CORR    -         -0.33  (seconds)                    DEFLEC18

JX1427  DYNAMIC HEIGHT  -        248.358 (meters)      814.82  (feet) COMP

JX1427  MODELED GRAVITY -    980,082.9   (mgal)                       NAVD 88

JX1427

JX1427  VERT ORDER      -  FIRST     CLASS II

JX1427

JX1427  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

JX1427  Standards:

JX1427         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

JX1427            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

JX1427  -------------------------------------------------------------------

JX1427  NETWORK    0.48   2.72           0.20   0.19   1.39     -0.02359497

JX1427  -------------------------------------------------------------------

JX1427  Click here for local accuracies and other accuracy information.

JX1427

JX1427

JX1427.The horizontal coordinates were established by GPS observations

JX1427.and adjusted by the National Geodetic Survey in June 2012.

JX1427

JX1427.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

JX1427.been affixed to the stable North American tectonic plate. See

JX1427.NA2011 for more information.

JX1427

JX1427.The horizontal coordinates are valid at the epoch date displayed above

JX1427.which is a decimal equivalence of Year/Month/Day.

JX1427

JX1427.The orthometric height was determined by differential leveling and

JX1427.adjusted by the NATIONAL GEODETIC SURVEY

JX1427.in June 1991.

JX1427

JX1427.Significant digits in the geoid height do not necessarily reflect accuracy.

JX1427.GEOID18 height accuracy estimate available here.

JX1427

JX1427.Click photographs - Photos may exist for this station.

JX1427

JX1427.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JX1427

JX1427.The Laplace correction was computed from DEFLEC18 derived deflections.

JX1427

JX1427.The ellipsoidal height was determined by GPS observations

JX1427.and is referenced to NAD 83.

JX1427

JX1427.The dynamic height is computed by dividing the NAVD 88

JX1427.geopotential number by the normal gravity value computed on the

JX1427.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JX1427.degrees latitude (g = 980.6199 gals.).

JX1427

JX1427.The modeled gravity was interpolated from observed gravity values.

JX1427

JX1427. The following values were computed from the NAD 83(2011) position.

JX1427

JX1427;                    North         East     Units Scale Factor Converg.

JX1427;SPC OH S     -   221,048.788   651,294.263   MT  0.99999164   +0 22 51.9

JX1427;SPC OH S     -   725,224.23  2,136,787.93   sFT  0.99999164   +0 22 51.9

JX1427;UTM  17      - 4,426,990.282   423,215.025   MT  0.99967258   -0 34 40.9

JX1427

JX1427!             -  Elev Factor  x  Scale Factor =   Combined Factor

JX1427!SPC OH S     -   0.99996632  x   0.99999164  =   0.99995796

JX1427!UTM  17      -   0.99996632  x   0.99967258  =   0.99963891

JX1427

JX1427_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SME2321526990(NAD 83)

JX1427

JX1427|---------------------------------------------------------------------|

JX1427| PID    Reference Object                     Distance      Geod. Az  |

JX1427|                                                           dddmmss.s |

JX1427| AA5318 GPS 86                               15.326 METERS 14137     |

JX1427|---------------------------------------------------------------------|

JX1427

JX1427                          SUPERSEDED SURVEY CONTROL

JX1427

JX1427  NAD 83(2007)-  39 59 22.56148(N)    081 53 57.85224(W) AD(2002.00) 1

JX1427  ELLIP H (01/27/09)  214.688  (m)                       GP(2002.00) 3 1

JX1427  NAVD 88             248.49   (m)          815.3    (f) LEVELING    3

JX1427  NGVD 29 (??/??/92)  248.715  (m)          815.99   (f) ADJ UNCH    1 2

JX1427

JX1427.Superseded values are not recommended for survey control.

JX1427

JX1427.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JX1427.See file dsdata.pdf to determine how the superseded data were derived.

JX1427

JX1427_MARKER: DB = BENCH MARK DISK

JX1427_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JX1427_STAMPING: D 184 1954

JX1427_MARK LOGO: CGS

JX1427_PROJECTION: PROJECTING 30 CENTIMETERS

JX1427_MAGNETIC: O = OTHER; SEE DESCRIPTION

JX1427_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JX1427+STABILITY: SURFACE MOTION

JX1427_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JX1427+SATELLITE: SATELLITE OBSERVATIONS - October 17, 2008

JX1427

JX1427  HISTORY     - Date     Condition        Report By

JX1427  HISTORY     - 1954     MONUMENTED       CGS

JX1427  HISTORY     - 20080915 GOOD             OH-119

JX1427  HISTORY     - 20081017 GOOD             OH-119

JX1427

JX1427                          STATION DESCRIPTION

JX1427

JX1427'DESCRIBED BY COAST AND GEODETIC SURVEY 1954

JX1427'7.9 MI NE FROM ZANESVILLE.

JX1427'7.9 MILES NORTHEAST AND EAST ALONG THE BALTIMORE AND OHIO

JX1427'RAILROAD FROM THE STATION AT ZANESVILLE, ABOUT 0.5 MILE

JX1427'NORTHEAST OF SONORA, AT AN EAST-WEST BLACK TOP ROAD CROSSING,

JX1427'IN THE Y OF JUNCTION OF A GRAVEL ROAD LEADING NORTHEAST

JX1427'PARALLEL TO THE TRACK, NORTHWEST AND ACROSS TRACK FROM

JX1427'TELEPHONE POLE NO. 70/21, 68 FEET NORTHWEST AND ACROSS ROAD FROM

JX1427'THE NORTHWEST RAIL, 23 1/2 FEET NORTH OF CENTER LINE OF EAST-WEST

JX1427'ROAD, 20 FEET SOUTHEAST OF CENTER LINE OF ROAD NORTHEAST

JX1427'PARALLEL TO TRACK, 4.3 FEET NORTH-NORTHWEST OF TELEPHONE POLE

JX1427'NO. 86-10, 52 1/2 FEET NORTHWEST OF CROSSING WARNING SIGN NO.

JX1427'5185, ABOUT 4 FEET ABOVE LEVEL OF TRACK, ABOUT LEVEL WITH ROAD

JX1427'NORTHEAST AND SET IN THE TOP OF A CONCRETE POST PROJECTING 3

JX1427'INCHES.

JX1427

JX1427                          STATION RECOVERY (2008)

JX1427

JX1427'RECOVERY NOTE BY MUSKINGUM COUNTY OHIO 2008 (DPS)

JX1427'RECOVERED IN GOOD CONDITION.

JX1427

JX1427                          STATION RECOVERY (2008)

JX1427

JX1427'RECOVERY NOTE BY MUSKINGUM COUNTY OHIO 2008 (DPS)

JX1427'RECOVERED AS DESCRIBED.

HY2186  DESIGNATION -  EIGHTEEN AZ MK

HY2186  PID         -  HY2186

HY2186  STATE/COUNTY-  WV/MASON

HY2186  COUNTRY     -  US

HY2186  USGS QUAD   -  GLENWOOD (2016)

HY2186

HY2186                         *CURRENT SURVEY CONTROL

HY2186  ______________________________________________________________________

HY2186* NAD 83(1995) POSITION- 38 36 05.07119(N) 082 10 20.12847(W)   ADJUSTED

HY2186* NAVD 88 ORTHO HEIGHT -   169.211 (meters)      555.15  (feet) ADJUSTED

HY2186  ______________________________________________________________________

HY2186  GEOID HEIGHT    -        -33.456 (meters)                     GEOID18

HY2186  LAPLACE CORR    -         -0.76  (seconds)                    DEFLEC18

HY2186  DYNAMIC HEIGHT  -        169.100 (meters)      554.79  (feet) COMP

HY2186  MODELED GRAVITY -    979,970.1   (mgal)                       NAVD 88

HY2186

HY2186  HORZ ORDER      -  SECOND

HY2186  VERT ORDER      -  FIRST     CLASS II

HY2186

HY2186.The horizontal coordinates were established by classical geodetic methods

HY2186.and adjusted by the National Geodetic Survey in April 1998.

HY2186.

HY2186.The orthometric height was determined by differential leveling and

HY2186.adjusted by the NATIONAL GEODETIC SURVEY

HY2186.in June 1991.

HY2186

HY2186.Significant digits in the geoid height do not necessarily reflect accuracy.

HY2186.GEOID18 height accuracy estimate available here.

HY2186

HY2186.Click photographs - Photos may exist for this station.

HY2186

HY2186.The Laplace correction was computed from DEFLEC18 derived deflections.

HY2186

HY2186.The dynamic height is computed by dividing the NAVD 88

HY2186.geopotential number by the normal gravity value computed on the

HY2186.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

HY2186.degrees latitude (g = 980.6199 gals.).

HY2186

HY2186.The modeled gravity was interpolated from observed gravity values.

HY2186

HY2186. The following values were computed from the NAD 83(1995) position.

HY2186

HY2186;                    North         East     Units Scale Factor Converg.

HY2186;SPC WV S     -   178,386.946   497,891.849   MT  0.99995210   -0 43 28.9

HY2186;SPC WV S     -   585,257.84  1,633,500.17   sFT  0.99995210   -0 43 28.9

HY2186;UTM  17      - 4,273,197.533   397,923.486   MT  0.99972831   -0 43 53.2

HY2186

HY2186!             -  Elev Factor  x  Scale Factor =   Combined Factor

HY2186!SPC WV S     -   0.99997870  x   0.99995210  =   0.99993080

HY2186!UTM  17      -   0.99997870  x   0.99972831  =   0.99970702

HY2186

HY2186:                Primary Azimuth Mark                     Grid Az

HY2186:SPC WV S     -  EIGHTEEN                                 025 41 20.7

HY2186:UTM  17      -  EIGHTEEN                                 025 41 45.0

HY2186

HY2186_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLC9792373197(NAD 83)

HY2186

HY2186|---------------------------------------------------------------------|

HY2186| PID    Reference Object                     Distance      Geod. Az  |

HY2186|                                                           dddmmss.s |

HY2186| HY2187 EIGHTEEN                            APPROX. 0.9 KM 0245751.8 |

HY2186|---------------------------------------------------------------------|

HY2186

HY2186                          SUPERSEDED SURVEY CONTROL

HY2186

HY2186  NAD 83(1993)-  38 36 05.07537(N)    082 10 20.13287(W) AD(       ) 2

HY2186  NAD 83(1986)-  38 36 05.07947(N)    082 10 20.13395(W) AD(       ) 2

HY2186  NAD 27      -  38 36 04.77759(N)    082 10 20.61068(W) AD(       ) 2

HY2186  NGVD 29 (07/19/86)  169.4    (m)          556.     (f) VERT ANG

HY2186

HY2186.Superseded values are not recommended for survey control.

HY2186

HY2186.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

HY2186.See file dsdata.pdf to determine how the superseded data were derived.

HY2186

HY2186_MARKER: DR = REFERENCE MARK DISK

HY2186_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

HY2186_STAMPING: EIGHTEEN AZ 1976

HY2186_MARK LOGO: NGS

HY2186_PROJECTION: FLUSH

HY2186_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

HY2186+STABILITY: SURFACE MOTION

HY2186_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

HY2186+SATELLITE: SATELLITE OBSERVATIONS - May 08, 2017

HY2186

HY2186  HISTORY     - Date     Condition        Report By

HY2186  HISTORY     - 1976     MONUMENTED       NGS

HY2186  HISTORY     - 1984     GOOD             NGS

HY2186  HISTORY     - 20071104 GOOD             GEOCAC

HY2186  HISTORY     - 20170508 GOOD             WOOLPT

HY2186

HY2186                          STATION DESCRIPTION

HY2186

HY2186'DESCRIBED BY NATIONAL GEODETIC SURVEY 1976 (JRS)

HY2186'AN EDM TRAVERSE TIE WAS MADE FROM EIGHTEEN 1976.

HY2186

HY2186                          STATION RECOVERY (1984)

HY2186

HY2186'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1984

HY2186'3.1 KM (1.95 MI) SOUTH FROM ASHTON.

HY2186'3.1 KM (1.95 MI) SOUTH ALONG STATE HIGHWAY 2 FROM THE POST OFFICE IN

HY2186'ASHTON TO THE MARK ON THE LEFT, 7.46 METERS (24.5 FT) SOUTHEAST OF THE

HY2186'CENTERLINE OF THE HIGHWAY, 27.43 METERS (90.0 FT) NORTHEAST OF THE

HY2186'CENTER OF A GRAVEL DRIVE LEADING SOUTHEAST, 7.10 METERS (23.3 FT)

HY2186'NORTHWEST OF THE NORTHWEST RAIL OF THE TRACKS.

HY2186'THE MARK IS 0.73 METERS  E FROM A WITNESS POST.

HY2186'THE MARK IS 0.5 M ABOVE HIGHWAY.

HY2186

HY2186                          STATION RECOVERY (2007)

HY2186

HY2186'RECOVERY NOTE BY GEOCACHING 2007 (DEB)

HY2186'RECOVERED IN GOOD CONDITION.

MB2974  CBN         -  This is a Cooperative Base Network Control Station.

MB2974  PACS        -  This is a Primary Airport Control Station.

MB2974  DESIGNATION -  EXECPORT

MB2974  PID         -  MB2974

MB2974  STATE/COUNTY-  OH/MAHONING

MB2974  COUNTRY     -  US

MB2974  USGS QUAD   -  CANFIELD (2016)

MB2974

MB2974                         *CURRENT SURVEY CONTROL

MB2974  ______________________________________________________________________

MB2974* NAD 83(2011) POSITION- 41 03 34.89880(N) 080 49 49.49557(W)   ADJUSTED

MB2974* NAD 83(2011) ELLIP HT-   265.559 (meters)        (06/27/12)   ADJUSTED

MB2974* NAD 83(2011) EPOCH   -  2010.00

MB2974* NAVD 88 ORTHO HEIGHT -   299.43  (meters)      982.4   (feet) GPS OBS

MB2974  ______________________________________________________________________

MB2974  NAVD 88 orthometric height was determined with geoid model    GEOID93

MB2974  GEOID HEIGHT    -        -33.933 (meters)                     GEOID93

MB2974  GEOID HEIGHT    -        -33.841 (meters)                     GEOID18

MB2974  NAD 83(2011) X  -    767,534.035 (meters)                     COMP

MB2974  NAD 83(2011) Y  - -4,754,890.152 (meters)                     COMP

MB2974  NAD 83(2011) Z  -  4,167,598.560 (meters)                     COMP

MB2974  LAPLACE CORR    -         -1.82  (seconds)                    DEFLEC18

MB2974

MB2974  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

MB2974  Standards:

MB2974         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

MB2974            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

MB2974  -------------------------------------------------------------------

MB2974  NETWORK    1.14   2.33           0.53   0.37   1.19      0.00228953

MB2974  -------------------------------------------------------------------

MB2974  Click here for local accuracies and other accuracy information.

MB2974

MB2974

MB2974.This mark is at Youngstown Executive Airport (06G)

MB2974

MB2974.The horizontal coordinates were established by GPS observations

MB2974.and adjusted by the National Geodetic Survey in June 2012.

MB2974

MB2974.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

MB2974.been affixed to the stable North American tectonic plate. See

MB2974.NA2011 for more information.

MB2974

MB2974.The horizontal coordinates are valid at the epoch date displayed above

MB2974.which is a decimal equivalence of Year/Month/Day.

MB2974

MB2974.The orthometric height was determined by GPS observations and a

MB2974.high-resolution geoid model.

MB2974

MB2974.GPS derived orthometric heights for airport stations designated as

MB2974.PACS or SACS are published to 2 decimal places.  This maintains

MB2974.centimeter relative accuracy between the PACS and SACS.  It does

MB2974.not indicate centimeter accuracy relative to other marks which are

MB2974.part of the NAVD 88 network.

MB2974

MB2974.Significant digits in the geoid height do not necessarily reflect accuracy.

MB2974.GEOID18 height accuracy estimate available here.

MB2974

MB2974.Click photographs - Photos may exist for this station.

MB2974

MB2974.The X, Y, and Z were computed from the position and the ellipsoidal ht.

MB2974

MB2974.The Laplace correction was computed from DEFLEC18 derived deflections.

MB2974

MB2974.The ellipsoidal height was determined by GPS observations

MB2974.and is referenced to NAD 83.

MB2974

MB2974. The following values were computed from the NAD 83(2011) position.

MB2974

MB2974;                    North         East     Units Scale Factor Converg.

MB2974;SPC OH N     -   156,033.191   740,326.884   MT  0.99993915   +1 05 48.6

MB2974;SPC OH N     -   511,918.89  2,428,889.12   sFT  0.99993915   +1 05 48.6

MB2974;UTM  17      - 4,545,397.560   514,249.449   MT  0.99960250   +0 06 41.0

MB2974

MB2974!             -  Elev Factor  x  Scale Factor =   Combined Factor

MB2974!SPC OH N     -   0.99995835  x   0.99993915  =   0.99989750

MB2974!UTM  17      -   0.99995835  x   0.99960250  =   0.99956086

MB2974

MB2974:                Primary Azimuth Mark                     Grid Az

MB2974:SPC OH N     -  EXECPORT AZ MK                           109 05 44.0

MB2974:UTM  17      -  EXECPORT AZ MK                           110 04 51.6

MB2974

MB2974_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TNF1424945397(NAD 83)

MB2974

MB2974|---------------------------------------------------------------------|

MB2974| PID    Reference Object                     Distance      Geod. Az  |

MB2974|                                                           dddmmss.s |

MB2974| MB2977 EXECPORT AZ MK                      426.969 METERS 1101132.6 |

MB2974|---------------------------------------------------------------------|

MB2974

MB2974                          SUPERSEDED SURVEY CONTROL

MB2974

MB2974  NAD 83(2007)-  41 03 34.89896(N)    080 49 49.49631(W) AD(2002.00) 0

MB2974  ELLIP H (02/10/07)  265.573  (m)                       GP(2002.00)

MB2974  ELLIP H (03/08/05)  265.582  (m)                       GP(       ) 4 2

MB2974  NAD 83(1995)-  41 03 34.89866(N)    080 49 49.49627(W) AD(       ) B

MB2974  ELLIP H (08/20/96)  265.576  (m)                       GP(       ) 4 2

MB2974  NAD 83(1986)-  41 03 34.90654(N)    080 49 49.50473(W) AD(       ) 3

MB2974  NAD 27      -  41 03 34.68735(N)    080 49 50.18072(W) AD(       ) 3

MB2974  NGVD 29 (02/23/89)  299.66   (m)  RAPSU86 model used   GPS OBS

MB2974

MB2974.Superseded values are not recommended for survey control.

MB2974

MB2974.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

MB2974.See file dsdata.pdf to determine how the superseded data were derived.

MB2974

MB2974_MARKER: DH = HORIZONTAL CONTROL DISK

MB2974_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

MB2974_STAMPING: EXECPORT 1986

MB2974_MARK LOGO: NGS

MB2974_PROJECTION: FLUSH

MB2974_MAGNETIC: N = NO MAGNETIC MATERIAL

MB2974_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

MB2974+STABILITY: SURFACE MOTION

MB2974_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

MB2974+SATELLITE: SATELLITE OBSERVATIONS - November 08, 2019

MB2974

MB2974  HISTORY     - Date     Condition        Report By

MB2974  HISTORY     - 1986     MONUMENTED       NGS

MB2974  HISTORY     - 19871027 GOOD

MB2974  HISTORY     - 19880913 GOOD             NGS

MB2974  HISTORY     - 19901027 GOOD             NGS

MB2974  HISTORY     - 19950726 GOOD             NGS

MB2974  HISTORY     - 20060612 GOOD             GEOCAC

MB2974  HISTORY     - 20191108 GOOD             WOOLPT

MB2974

MB2974                          STATION DESCRIPTION

MB2974

MB2974'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

MB2974'STATION IS ABOUT 10 MILES WEST OF YOUNGSTOWN, 3 MILES SOUTHEAST OF

MB2974'NORTH JACKSON, AT THE YOUNGSTOWN EXECUTIVE AIRPORT, AT THE JUNCTION

MB2974'OF THE MAIN RAM AND THE RUNWAY.  OWNERSHIP CITY OF YOUNGSTOWN, C/O

MB2974'AIRPORT MANAGER JOHN IRWIN, 9579 GIBSON ROAD, NORTH JACKSON OH 44451

MB2974'PHONE 216-538-2528

MB2974'TO REACH FROM THE JUNCTION OF THE MAIN STREET AND STATE HIGHWAY 45 IN

MB2974'NORTH JACKSON (JUST NORTH OF THE POST OFFICE), GO SOUTH ON HIGHWAY 45

MB2974'FOR 2.1 MILES TO A PAVED ROAD LEFT.  TURN LEFT, EAST, ON KIRK ROAD

MB2974'FOR 0.7 MILES TO AN OFFSET CROSSROAD.  TURN RIGHT, SOUTH FOR 0.5 MILES

MB2974'TO A PAVED ROAD RIGHT.  TURN RIGHT, SOUTH ON ROAD, THEN RAMP FOR 0.2

MB2974'MILES TO THE RUNWAY AND STATION ON LEFT.  TO REACH THE AZIMUTH MARK

MB2974'FROM STATION, GO SOUTHEAST ON NARROW PAVED LANE FOR 0.3 MILES TO A

MB2974'CURVE JUST BEFORE RUNWAY AND MARK ON THE RIGHT.

MB2974'STATION MARKS ARE STANDARD NGS STATION DISKS STAMPED --EXECPORT

MB2974'1986--. THE SURFACE DISK IS SET IN TOP OF A 25 CM ROUND CONCRETE

MB2974'POST PROJECTION 1 CM.  IT IS 24.3 METERS NORTHEAST OF THE RUNWAY

MB2974'CENTER, 17.7 METERS SOUTHEAST OF THE RAMP CENTER, 5.7 METERS EAST

MB2974'NORTHEAST OF A FIBERGLASS WITNESS POST, 14.9 METERS SOUTHWEST OF THE

MB2974'TARMAC EDGE, AND 5.8 METERS WEST OF SOUTHWEST CORNER OF A MACADAM

MB2974'AIRCRAFT PARKING APRON.  THE SUBSURFACE DISK IS SET IN THE TOP OF AN

MB2974'IRREGULAR CONCRETE MASS ABOUT 1.2 METERS BELOW GROUND.

MB2974'DESCRIBED BY B.L. LAMBERT. TYPED BY JAMES MALONEY 9/05/87.

MB2974

MB2974                          STATION RECOVERY (1987)

MB2974

MB2974'RECOVERED 1987

MB2974'RECOVERED IN GOOD CONDITION.

MB2974

MB2974                          STATION RECOVERY (1988)

MB2974

MB2974'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1988

MB2974'RECOVERED IN GOOD CONDITION.

MB2974

MB2974                          STATION RECOVERY (1990)

MB2974

MB2974'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1990

MB2974'THE STATION IS LOCATED ABOUT 4.82 KM (3.00 MI) SOUTHEAST OF NORTH

MB2974'JACKSON, AT THE YOUNGSTOWN EXECUTIVE AIRPORT AND AT THE JUNCTION OF

MB2974'THE TAXIWAY AND RUNWAY.  OWNERSHIP--CITY OF YOUNGSTOWN,  C/O LEON

MB2974'SYREK, AIRPORT MANAGER, 9579 GIBSON ROAD, NORTH JACKSON, OH 44451,

MB2974'PHONE (216)538-3456.

MB2974'TO REACH THE STATION FROM THE WESTBOUND LANE OF INTERSTATE HIGHWAY 76

MB2974'AND BAILEY ROAD (EXIT 57), LOCATED ABOUT 3.54 KM (2.20 MI) WEST OF

MB2974'NORTH JACKSON, GO SOUTH ON BAILEY ROAD FOR 1.05 KM (0.65 MI) TO THE

MB2974'JUNCTION OF MAHONING ROAD, TURN LEFT, EAST, ON MAHONING ROAD FOR 2.57

MB2974'KM (1.60 MI) TO THE JUNCTION OF STATE HIGHWAY 45, TURN RIGHT, SOUTH,

MB2974'ON STATE HIGHWAY 45 FOR 3.38 KM (2.10 MI) TO THE JUNCTION OF KIRK

MB2974'ROAD ON THE LEFT, TURN LEFT, EAST, ON KIRK ROAD FOR 1.13 KM

MB2974'(0.70 MI) TO THE JUNCTION OF GAULT ROAD ON THE RIGHT, TURN RIGHT,

MB2974'SOUTH, ON GAULT ROAD FOR 0.88 KM (0.55 MI) TO THE JUNCTION OF GIBSON

MB2974'ROAD ON THE LEFT, TURN LEFT, EAST, ON GIBSON ROAD FOR 1.20 KM

MB2974'(0.75 MI) TO THE AIRPORT ENTRANCE ROAD ON THE RIGHT, TURN RIGHT,

MB2974'SOUTH, ON THE ENTRANCE ROAD FOR 0.16 KM (0.10 MI) TO THE APRON AND

MB2974'THE AIRPORT OFFICE ON THE LEFT, CONTINUE AHEAD TO THE JUNCTION OF THE

MB2974'RUNWAY AND THE STATION ON THE LEFT.

MB2974'THE STATION IS ABOUT FLUSH WITH THE GROUND LOCATED 24.3 M (79.7 FT)

MB2974'NORTHEAST OF THE CENTER OF THE RUNWAY, 17.7 M (58.1 FT) SOUTHEAST OF

MB2974'THE CENTER OF THE TAXIWAY, 14.0 M (45.9 FT) NORTH OF THE NORTH EDGE

MB2974'OF THE APRON AND 5.7 M (18.7 FT) EAST OF A FIBERGLASS WITNESS POST.

MB2974

MB2974                          STATION RECOVERY (1995)

MB2974

MB2974'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

MB2974'NOTE--THIS IS THE PAC STATION.  THE STATION IS LOCATED ABOUT 16 KM

MB2974'(9.95 MI) WEST OF YOUNGSTOWN, 6 KM (3.70 MI) SOUTH OF INTERSTATE

MB2974'HIGHWAY 76, 5 KM (3.10 MI) SOUTHEAST OF NORTH JACKSON AT THE

MB2974'YOUNGSTOWN EXECUTIVE AIRPORT, ADJACENT TO THE APRON, IN THE NORTHEAST

MB2974'QUADRANT OF THE JUNCTION OF THE RUNWAY AND A CONNECTOR TAXI AT

MB2974'MIDFIELD.  OWNERSHIP--CITY OF YOUNGSTOWN.  AIRPORT MANAGER IS LEON

MB2974'SYREK, 9579 GIBSON ROAD, NORTH JACKSON, OH.  44451.  PHONE

MB2974'330-538-3456.  TO REACH FROM THE JUNCTION OF INTERSTATE HIGHWAY 76 AND

MB2974'BAILEY ROAD (EXIT 57) , ABOUT 4 KM (2.50 MI) WEST OF NORTH JACKSON, GO

MB2974'SOUTH ON BAILEY ROAD FOR 1.05 KM (0.65 MI) TO A PAVED CROSSROAD

MB2974'(MAHONING ROAD) .  TURN LEFT, EAST, ON MAHONING ROAD FOR 2.57 KM (1.60

MB2974'MI) TO THE JUNCTION OF STATE HIGHWAY 45.  TURN RIGHT, SOUTH, ON

MB2974'HIGHWAY 45 FOR 3.38 KM (2.10 MI) TO THE JUNCTION OF KIRK ROAD ON THE

MB2974'LEFT.  TURN LEFT, EAST, ON KIRK ROAD FOR 1.13 KM (0.70 MI) TO THE

MB2974'JUNCTION OF GAULT ROAD ON THE RIGHT.  TURN RIGHT, SOUTH, ON GAULT ROAD

MB2974'FOR 0.88 KM (0.55 MI) TO THE JUNCTION OF GIBSON ROAD ON THE LEFT.

MB2974'TURN LEFT, EAST, ON GIBSON ROAD FOR 1.2 KM (0.75 MI) TO THE AIRPORT

MB2974'ENTRANCE ROAD ON THE RIGHT.  TURN RIGHT, SOUTH, ON THE ENTRANCE ROAD

MB2974'FOR 0.16 KM (0.10 MI) TO THE APRON AND AIRPORT OFFICE ON THE LEFT.

MB2974'CONTINUE AHEAD, SOUTHWEST, ACROSS THE APRON AND ALONG TAXI FOR 0.16 KM

MB2974'(0.10 MI) TO THE JUNCTION OF THE RUNWAY AND THE STATION ON THE LEFT.

MB2974'THE STATION IS SET IN THE TOP OF A 30 CM CONCRETE POST SET FLUSH WITH

MB2974'THE GROUND.  IT IS 24.3 M (79.7 FT) NORTHEAST OF THE CENTER OF THE

MB2974'RUNWAY, 17.7 M (58.1 FT) SOUTHEAST OF THE CENTER OF THE TAXI, 14.0 M

MB2974'(45.9 FT) SOUTHWEST OF THE SOUTHWEST EDGE OF THE APRON, AND 5.7 M

MB2974'(18.7 FT) EAST OF A FIBERGLASS WITNESS POST.

MB2974

MB2974                          STATION RECOVERY (2006)

MB2974

MB2974'RECOVERY NOTE BY GEOCACHING 2006 (RLM)

MB2974'THE AIRPORT IS NOT IN USE AND THE LAND IS FOR SALE.

HY0076  DESIGNATION -  F 200

HY0076  PID         -  HY0076

HY0076  STATE/COUNTY-  OH/GALLIA

HY0076  COUNTRY     -  US

HY0076  USGS QUAD   -  RODNEY (2016)

HY0076

HY0076                         *CURRENT SURVEY CONTROL

HY0076  ______________________________________________________________________

HY0076* NAD 83(1986) POSITION- 38 51 01.     (N) 082 17 40.     (W)   SCALED

HY0076* NAVD 88 ORTHO HEIGHT -   213.044 (meters)      698.96  (feet) ADJUSTED

HY0076  ______________________________________________________________________

HY0076  GEOID HEIGHT    -        -33.759 (meters)                     GEOID18

HY0076  DYNAMIC HEIGHT  -        212.908 (meters)      698.52  (feet) COMP

HY0076  MODELED GRAVITY -    979,984.9   (mgal)                       NAVD 88

HY0076

HY0076  VERT ORDER      -  SECOND    CLASS 0

HY0076

HY0076.The horizontal coordinates were scaled from a map and have

HY0076.an estimated accuracy of +/- 6 seconds.

HY0076.

HY0076.The orthometric height was determined by differential leveling and

HY0076.adjusted by the NATIONAL GEODETIC SURVEY

HY0076.in June 1991.

HY0076

HY0076.Significant digits in the geoid height do not necessarily reflect accuracy.

HY0076.GEOID18 height accuracy estimate available here.

HY0076

HY0076.Click photographs - Photos may exist for this station.

HY0076

HY0076.The dynamic height is computed by dividing the NAVD 88

HY0076.geopotential number by the normal gravity value computed on the

HY0076.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

HY0076.degrees latitude (g = 980.6199 gals.).

HY0076

HY0076.The modeled gravity was interpolated from observed gravity values.

HY0076

HY0076;                    North         East    Units  Estimated Accuracy

HY0076;SPC OH S     -    94,410.      617,840.      MT  (+/- 180 meters Scaled)

HY0076

HY0076_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD876009(NAD 83)

HY0076

HY0076                          SUPERSEDED SURVEY CONTROL

HY0076

HY0076  NGVD 29 (??/??/92)  213.233  (m)          699.58   (f) ADJ UNCH    2 0

HY0076

HY0076.Superseded values are not recommended for survey control.

HY0076

HY0076.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

HY0076.See file dsdata.pdf to determine how the superseded data were derived.

HY0076

HY0076_MARKER: DB = BENCH MARK DISK

HY0076_SETTING: 31 = SET IN A PAVEMENT SUCH AS STREET, SIDEWALK, CURB, ETC.

HY0076_SP_SET: CATCH BASIN

HY0076_STAMPING: F 200 1958

HY0076_MARK LOGO: CGS

HY0076_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

HY0076_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

HY0076+SATELLITE: SATELLITE OBSERVATIONS - April 13, 2017

HY0076

HY0076  HISTORY     - Date     Condition        Report By

HY0076  HISTORY     - 1958     MONUMENTED       CGS

HY0076  HISTORY     - 1976     GOOD             NGS

HY0076  HISTORY     - 20170413 GOOD             WOOLPT

HY0076

HY0076                          STATION DESCRIPTION

HY0076

HY0076'DESCRIBED BY COAST AND GEODETIC SURVEY 1958

HY0076'5.1 MI SE FROM RIO GRANDE.

HY0076'ABOUT 5.1 MILES SOUTHEAST ALONG U.S. HIGHWAY 35 FROM THE CALVARY

HY0076'BAPTIST CHURCH AT RIO GRANDE, IN SECTION 19 OR 24, R 15W, T 3 OR 4N,

HY0076'ABOUT 0.25 MILES SOUTHEAST OF THE SIGN RODNEY, 0.2 MILE NORTHWEST

HY0076'OF NORTHWEST END OF A CUT, SET IN THE TOP OF THE NORTH WALL OF A

HY0076'2-FOOT BY 2-FOOT CATCH BASIN, 29 FEET NORTH OF THE CENTER LINE

HY0076'OF THE HIGHWAY, 36 FEET EAST OF THE CENTER LINE OF A DRIVE NORTH

HY0076'TO A FIELD, ABOUT 0.35 MILE SOUTHEAST OF A NORTH-SOUTH CROSS

HY0076'ROAD AT RODNEY, ABOUT 2 FEET BELOW THE LEVEL OF THE HIGHWAY.

HY0076'NOTE-- THE MARK MAY BE REACHED FROM GALLIPOLIS, GALLIA COUNTY,

HY0076'BY GOING NORTHWEST FOR 6.25 MILES ALONG U.S. HIGHWAY 35 FROM ITS

HY0076'CROSSING OF SECOND AVENUE.

HY0076

HY0076                          STATION RECOVERY (1976)

HY0076

HY0076'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

HY0076'RECOVERED IN GOOD CONDITION.

HY0076

HY0076                          STATION RECOVERY (2017)

HY0076

HY0076'RECOVERY NOTE BY WOOLPERT CONSULTANTS 2017

HY0076'TO REACH THE STATION FROM THE INTERSECTION OF HIGHWAY 35 AND STATE

HY0076'ROUTE 325 NEAR RIO GRANDE, TRAVEL EASTBOUND ON US HIGHWAY 35 ABOUT

HY0076'4.88 MILES AND TAKE THE EXIT FOR STATE ROUTE 850, THEN TRAVEL SOUTH ON

HY0076'STATE ROUTE 850 FOR 1.71 MILES TO REACH THE INTERSECTION WITH JACKSON

HY0076'PIKE, THEN TRAVEL EAST ON JACKSON PIKE FOR ABOUT 0.38 MILES.

HY0076'ABOUT 26 FEET NORTH OF THE CENTERLINE OF THE HIGHWAY, OR ABOUT 22 FEET

HY0076'SOUTHEAST OF THE MOST EASTERLY WOODEN FENCE POST.

HY0076'THE STATION IS A BRASS DISK SET IN THE TOP OF A CONCRETE BLOCK FLUSH.

HY0076'WITH THE GROUND.

AH7726  DESIGNATION -  FAIRFIELD COUNTY 7

AH7726  PID         -  AH7726

AH7726  STATE/COUNTY-  OH/FAIRFIELD

AH7726  COUNTRY     -  US

AH7726  USGS QUAD   -  BALTIMORE (2016)

AH7726

AH7726                         *CURRENT SURVEY CONTROL

AH7726  ______________________________________________________________________

AH7726* NAD 83(2011) POSITION- 39 52 03.54531(N) 082 35 09.10930(W)   ADJUSTED

AH7726* NAD 83(2011) ELLIP HT-   245.315 (meters)        (06/27/12)   ADJUSTED

AH7726* NAD 83(2011) EPOCH   -  2010.00

AH7726* NAVD 88 ORTHO HEIGHT -   279.5   (meters)      917.    (feet) GPS OBS

AH7726  ______________________________________________________________________

AH7726  NAVD 88 orthometric height was determined with geoid model    GEOID96

AH7726  GEOID HEIGHT    -        -34.094 (meters)                     GEOID96

AH7726  GEOID HEIGHT    -        -34.186 (meters)                     GEOID18

AH7726  NAD 83(2011) X  -    632,597.784 (meters)                     COMP

AH7726  NAD 83(2011) Y  - -4,861,342.076 (meters)                     COMP

AH7726  NAD 83(2011) Z  -  4,066,874.823 (meters)                     COMP

AH7726  LAPLACE CORR    -         -0.47  (seconds)                    DEFLEC18

AH7726

AH7726  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

AH7726  Standards:

AH7726         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

AH7726            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

AH7726  -------------------------------------------------------------------

AH7726  NETWORK    2.11   4.63           0.95   0.73   2.36      0.23501109

AH7726  -------------------------------------------------------------------

AH7726  Click here for local accuracies and other accuracy information.

AH7726

AH7726

AH7726.The horizontal coordinates were established by GPS observations

AH7726.and adjusted by the National Geodetic Survey in June 2012.

AH7726

AH7726.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

AH7726.been affixed to the stable North American tectonic plate. See

AH7726.NA2011 for more information.

AH7726

AH7726.The horizontal coordinates are valid at the epoch date displayed above

AH7726.which is a decimal equivalence of Year/Month/Day.

AH7726

AH7726.The orthometric height was determined by GPS observations and a

AH7726.high-resolution geoid model.

AH7726

AH7726.Significant digits in the geoid height do not necessarily reflect accuracy.

AH7726.GEOID18 height accuracy estimate available here.

AH7726

AH7726.Click photographs - Photos may exist for this station.

AH7726

AH7726.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AH7726

AH7726.The Laplace correction was computed from DEFLEC18 derived deflections.

AH7726

AH7726.The ellipsoidal height was determined by GPS observations

AH7726.and is referenced to NAD 83.

AH7726

AH7726. The following values were computed from the NAD 83(2011) position.

AH7726

AH7726;                    North         East     Units Scale Factor Converg.

AH7726;SPC OH S     -   207,341.567   592,653.843   MT  0.99997142   -0 03 16.1

AH7726;SPC OH S     -   680,253.12  1,944,398.48   sFT  0.99997142   -0 03 16.1

AH7726;UTM  17      - 4,414,271.391   364,367.045   MT  0.99982648   -1 01 00.2

AH7726

AH7726!             -  Elev Factor  x  Scale Factor =   Combined Factor

AH7726!SPC OH S     -   0.99996152  x   0.99997142  =   0.99993294

AH7726!UTM  17      -   0.99996152  x   0.99982648  =   0.99978800

AH7726

AH7726_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLE6436714271(NAD 83)

AH7726

AH7726                          SUPERSEDED SURVEY CONTROL

AH7726

AH7726  NAD 83(2007)-  39 52 03.54526(N)    082 35 09.11032(W) AD(2002.00) 0

AH7726  ELLIP H (02/10/07)  245.344  (m)                       GP(2002.00)

AH7726  ELLIP H (10/07/05)  245.331  (m)                       GP(       ) 4 2

AH7726  NAD 83(1995)-  39 52 03.54552(N)    082 35 09.11019(W) AD(       ) 1

AH7726  ELLIP H (02/25/99)  245.364  (m)                       GP(       ) 4 2

AH7726

AH7726.Superseded values are not recommended for survey control.

AH7726

AH7726.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AH7726.See file dsdata.pdf to determine how the superseded data were derived.

AH7726

AH7726_MARKER: DD = SURVEY DISK

AH7726_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

AH7726_STAMPING: FAIRFIELD COUNTY 7 1998

AH7726_MARK LOGO: OH-089

AH7726_PROJECTION: FLUSH

AH7726_MAGNETIC: M = MARKER EQUIPPED WITH BAR MAGNET

AH7726_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

AH7726+STABILITY: SURFACE MOTION

AH7726_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AH7726+SATELLITE: SATELLITE OBSERVATIONS - 1998

AH7726

AH7726  HISTORY     - Date     Condition        Report By

AH7726  HISTORY     - 1998     MONUMENTED       GEOMET

AH7726

AH7726                          STATION DESCRIPTION

AH7726

AH7726'DESCRIBED BY GEOMETRICS GPS INCORPORATED 1998 (DAR)

AH7726'THE STATION IS LOCATED IN WALNUT TOWNSHIP IN FAIRFIELD COUNTY, OHIO.

AH7726'THE REACH THE STATION FROM THE INTERSECTION OF BLACKLICK-EASTERN ROAD

AH7726'(SR 204) AND CHERRY LANE ROAD (T-245) PROCEED SOUTH ALONG CHERRY LANE

AH7726'ROAD (T-245) A DISTANCE OF 2.3 MILES (3.7 KM) TO AN OLD CEMETARY

AH7726'(MUSSER AND WARNER GRAVES) AND THE STATION ON THE LEFT.  THE STATION

AH7726'IS A STANDARD 3 1/4 INCH LICKING COUNTY SURVEY DISK SET IN A POURED IN

AH7726'PLACE CONCRETE MONUMENT ONE INCH BELOW THE GROUND SURFACE AND STAMPED

AH7726'FAIRFIELD COUNTY 7 1998.  THE STATION IS 14.2 FEET (4.3 M) AZIMUTH 05

AH7726'DEGREES TO A CONCRETE FILLED PVC PIPE, 26.2 FEET (8.0 M) AZIMUTH 67

AH7726'DEGREES TO A CONCRETE POST, 152.8 FEET (46.6 M) AZIMUTH 203 DEGREES TO

AH7726'A UTILITY POLE WITH THE NUMBER 6-3 NAB-4, AND 17.0 FEET (5.2 M) EAST

AH7726'FROM THE CENTERLINE OF CHERRY LAND ROAD.

KY1500  CBN         -  This is a Cooperative Base Network Control Station.

KY1500  DESIGNATION -  FORD

KY1500  PID         -  KY1500

KY1500  STATE/COUNTY-  OH/GUERNSEY

KY1500  COUNTRY     -  US

KY1500  USGS QUAD   -  BLOOMFIELD (2016)

KY1500

KY1500                         *CURRENT SURVEY CONTROL

KY1500  ______________________________________________________________________

KY1500* NAD 83(2011) POSITION- 40 02 21.94067(N) 081 38 48.85657(W)   ADJUSTED

KY1500* NAD 83(2011) ELLIP HT-   314.246 (meters)        (06/27/12)   ADJUSTED

KY1500* NAD 83(2011) EPOCH   -  2010.00

KY1500* NAVD 88 ORTHO HEIGHT -   348.089 (meters)     1142.02  (feet) ADJUSTED

KY1500  ______________________________________________________________________

KY1500  GEOID HEIGHT    -        -33.822 (meters)                     GEOID18

KY1500  NAD 83(2011) X  -    710,404.886 (meters)                     COMP

KY1500  NAD 83(2011) Y  - -4,838,256.452 (meters)                     COMP

KY1500  NAD 83(2011) Z  -  4,081,540.417 (meters)                     COMP

KY1500  LAPLACE CORR    -         -1.01  (seconds)                    DEFLEC18

KY1500  DYNAMIC HEIGHT  -        347.902 (meters)     1141.41  (feet) COMP

KY1500  MODELED GRAVITY -    980,077.5   (mgal)                       NAVD 88

KY1500

KY1500  VERT ORDER      -  FIRST     CLASS II

KY1500

KY1500  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY1500  Standards:

KY1500         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY1500            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY1500  -------------------------------------------------------------------

KY1500  NETWORK    1.02   1.88           0.45   0.38   0.96     -0.04909309

KY1500  -------------------------------------------------------------------

KY1500  Click here for local accuracies and other accuracy information.

KY1500

KY1500

KY1500.The horizontal coordinates were established by GPS observations

KY1500.and adjusted by the National Geodetic Survey in June 2012.

KY1500

KY1500.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY1500.been affixed to the stable North American tectonic plate. See

KY1500.NA2011 for more information.

KY1500

KY1500.The horizontal coordinates are valid at the epoch date displayed above

KY1500.which is a decimal equivalence of Year/Month/Day.

KY1500

KY1500.The orthometric height was determined by differential leveling and

KY1500.adjusted by the NATIONAL GEODETIC SURVEY

KY1500.in June 1991.

KY1500

KY1500.Significant digits in the geoid height do not necessarily reflect accuracy.

KY1500.GEOID18 height accuracy estimate available here.

KY1500

KY1500.Click photographs - Photos may exist for this station.

KY1500

KY1500.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY1500

KY1500.The Laplace correction was computed from DEFLEC18 derived deflections.

KY1500

KY1500.The ellipsoidal height was determined by GPS observations

KY1500.and is referenced to NAD 83.

KY1500

KY1500.The dynamic height is computed by dividing the NAVD 88

KY1500.geopotential number by the normal gravity value computed on the

KY1500.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

KY1500.degrees latitude (g = 980.6199 gals.).

KY1500

KY1500.The modeled gravity was interpolated from observed gravity values.

KY1500

KY1500. The following values were computed from the NAD 83(2011) position.

KY1500

KY1500;                    North         East     Units Scale Factor Converg.

KY1500;SPC OH S     -   226,754.708   672,806.195   MT  1.00000121   +0 32 28.7

KY1500;SPC OH S     -   743,944.40  2,207,364.99   sFT  1.00000121   +0 32 28.7

KY1500;UTM  17      - 4,432,333.783   444,811.986   MT  0.99963749   -0 24 58.2

KY1500

KY1500!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY1500!SPC OH S     -   0.99995070  x   1.00000121  =   0.99995191

KY1500!UTM  17      -   0.99995070  x   0.99963749  =   0.99958821

KY1500

KY1500:                Primary Azimuth Mark                     Grid Az

KY1500:SPC OH S     -  COLUMBUS PHILADELPH AWY BN 6B            083 56 13.6

KY1500:UTM  17      -  COLUMBUS PHILADELPH AWY BN 6B            084 53 40.5

KY1500

KY1500_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME4481132333(NAD 83)

KY1500

KY1500|---------------------------------------------------------------------|

KY1500| PID    Reference Object                     Distance      Geod. Az  |

KY1500|                                                           dddmmss.s |

KY1500| KY3438 COLUMBUS PHILADELPH AWY BN 6B       APPROX. 6.6 KM 0842842.3 |

KY1500| KY3440 CAMBRIDGE PARK SCHOOL FINIAL        APPROX. 5.7 KM 1061540.1 |

KY1500| KY3437 CAMBRIDGE FIRST ME CH SPIRE         APPROX. 4.9 KM 1070919.9 |

KY1500| KY3439 CAMBRIDGE GUERNSEY CO CTHSE         APPROX. 5.1 KM 1074739.2 |

KY1500| KY3441 CAMBRIDGE AM STEEL CO STACK         APPROX. 6.4 KM 1154903.0 |

KY1500| KY3448 COLUMBUS PHILADELPH AWY BN 5        APPROX.12.7 KM 2742854.7 |

KY1500| KY1499 FORD RM 1                            88.773 METERS 29542     |

KY1500| KY1501 FORD RM 2                            84.323 METERS 34836     |

KY1500| KY3444 COLLEGE                              43.023 METERS 35303     |

KY1500|---------------------------------------------------------------------|

KY1500

KY1500                          SUPERSEDED SURVEY CONTROL

KY1500

KY1500  NAD 83(2007)-  40 02 21.94114(N)    081 38 48.86022(W) AD(2002.00) 0

KY1500  ELLIP H (02/10/07)  314.280  (m)                       GP(2002.00)

KY1500  ELLIP H (03/08/05)  314.252  (m)                       GP(       ) 4 2

KY1500  NAD 83(1995)-  40 02 21.94099(N)    081 38 48.85721(W) AD(       ) B

KY1500  ELLIP H (08/20/96)  314.283  (m)                       GP(       ) 4 2

KY1500  NAD 83(1986)-  40 02 21.94692(N)    081 38 48.86485(W) AD(       ) 1

KY1500  NAD 27      -  40 02 21.68100(N)    081 38 49.40800(W) AD(       ) 1

KY1500  NAVD 88             348.09   (m)         1142.0    (f) LEVELING    3

KY1500  NAVD 88             348.09   (m)         1142.0    (f) LEVELING    3

KY1500  NGVD 29 (??/??/92)  348.313  (m)         1142.76   (f) ADJ UNCH    1 2

KY1500  NGVD 29             348.31   (m)         1142.7    (f) LEVELING    3

KY1500

KY1500.Superseded values are not recommended for survey control.

KY1500

KY1500.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY1500.See file dsdata.pdf to determine how the superseded data were derived.

KY1500

KY1500_MARKER: DS = TRIANGULATION STATION DISK

KY1500_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY1500_STAMPING: FORD 1933

KY1500_MARK LOGO: CGS

KY1500_PROJECTION: PROJECTING 23 CENTIMETERS

KY1500_MAGNETIC: O = OTHER; SEE DESCRIPTION

KY1500_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY1500+STABILITY: SURFACE MOTION

KY1500_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY1500+SATELLITE: SATELLITE OBSERVATIONS - May 19, 2017

KY1500

KY1500  HISTORY     - Date     Condition        Report By

KY1500  HISTORY     - 1933     MONUMENTED       CGS

KY1500  HISTORY     - 1954     GOOD             CGS

KY1500  HISTORY     - 1957     GOOD             CGS

KY1500  HISTORY     - 1957     GOOD             LOCENG

KY1500  HISTORY     - 1975     GOOD             THOMAS

KY1500  HISTORY     - 19890101 GOOD             OH-119

KY1500  HISTORY     - 19920909 GOOD             AEP

KY1500  HISTORY     - 19921217 GOOD             OH-119

KY1500  HISTORY     - 19950807 GOOD             NGS

KY1500  HISTORY     - 20170519 GOOD             OH-119

KY1500

KY1500                          STATION DESCRIPTION

KY1500

KY1500'DESCRIBED BY COAST AND GEODETIC SURVEY 1933 (GLA)

KY1500'ABOUT 3 MILES WEST- NORTHWEST OF CAMBRIDGE, AT THE TOP OF COLLEGE

KY1500'HILL, AND ABOUT 250 YARDS SOUTHEAST OF THE COLLEGE HILL SCHOOL

KY1500'BUILDING, SCHOOL IS ON THE HIGHEST PART OF THE HILL.

KY1500'

KY1500'TO REACH FROM CAMBRIDGE, GO WEST ON WHEELING AVENUE (U.S.  HIGHWAY 40)

KY1500'FOR 0.25 MILE FROM COURT HOUSE, CROSS VIADUCT OVER BALTIMORE AND OHIO

KY1500'RAILROAD AND PROCEED 0.75 MILE ON U.S. HIGHWAY 40 TO T-ROAD RIGHT AT

KY1500'THE TOP OF A HILL, TURN RIGHT AND FOLLOW GRAVEL ROAD FOR 2.0 MILES TO

KY1500'T-ROAD LEFT, TURN LEFT AND PROCEED 0.5 MILE TO COLLEGE HILL SCHOOL

KY1500'HOUSE AT TOP OF HILL.  STATION IS IN AN OPEN FIELD ACROSS THE ROAD

KY1500'FROM THE SCHOOL AND JUST SOUTH OF A PEACH ORCHARD, ABOUT 300 FEET

KY1500'SOUTHEAST OF THE CENTER LINE OF THE ROAD AND 277 FEET SOUTH OF FENCE

KY1500'AND CABLE LINE ALONG THE SOUTH EDGE OF THE PEACH ORCHARD.  MARK

KY1500'PROJECTS 3 INCHES ABOVE THE GROUND.

KY1500'

KY1500'SURFACE, UNDERGROUND AND REFERENCE MARKS ARE STANDARD BRONZE DISKS IN

KY1500'CONCRETE.

KY1500'

KY1500'REFERENCE MARK NO. 1 IS NORTHWEST OF THE STATION, 1 FOOT EAST OF

KY1500'RIGHT-OF-WAY FENCE, 15 FEET EAST OF THE CENTER LINE OF THE ROAD AT THE

KY1500'INTERSECTION OF AN OLD ROADWAY AND NEW ROAD, ABOUT 150 FEET SOUTH OF

KY1500'SCHOOL BUILDING, AND PROJECTS 8 INCHES ABOVE THE GROUND.  REFERENCE

KY1500'MARK NO. 2 IS NORTH OF THE STATION, 1 FOOT SOUTH OF FENCE AND CABLE

KY1500'LINE ALONG THE SOUTH EDGE OF PEACH ORCHARD, 3 FEET WEST OF THE SECOND

KY1500'CABLE LINE POLE EAST OF THE ROAD, AND 180 FEET EAST OF THE ROAD, AND

KY1500'PROJECTS 8 INCHES ABOVE THE GROUND.

KY1500'

KY1500'A UNITED STATES GEOLOGICAL SURVEY BENCH MARK IS NORTH OF THE STATION

KY1500'AND 135 FEET SOUTH OF FENCE AND CABLE LINE, IT IS A STANDARD U.S.G.S.

KY1500'DISK SET IN A 6- BY 6-INCH CONCRETE POST.  THERE ARE NO IDENTIFICATION

KY1500'MARKS STAMPED ON THE DISK, THE POST IS NOT SOLID IN THE GROUND AND

KY1500'MARK PROJECTS 2 INCHES ABOVE THE GROUND.

KY1500'

KY1500'HEIGHT OF LIGHT ABOVE STATION MARK - 30 METERS.

KY1500

KY1500                          STATION RECOVERY (1954)

KY1500

KY1500'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1954

KY1500'5.1 MI W FROM CAMBRIDGE.  2.8 MILE SOUTHWEST ALONG TE BALTIMORE AND

KY1500'OHIO RAILROAD FROM THE STATION AT CAMBRIDGE, THENCE ABOUT 2.3 MILES

KY1500'NORTH AND NORTHWEST ALONG A GRAVEL ROAD, NEAR THE TOP OF A HILL, 10.8

KY1500'FEET EAST OF SOUTHEAST CORNER OF CONCRETE BASE OF WILE RADIO TOWER,

KY1500'1/2 FOOT EAST OF WOODEN FENCE AROUND TOWER, 291 FEET EAST-SOUTHEAST OF

KY1500'RM NO. 1, 277 FEET SOUTH OF RM NO. 2, AND SET IN THE TOP OF A CONCRETE

KY1500'POST PROJECTING 6 INCHES.

KY1500

KY1500                          STATION RECOVERY (1957)

KY1500

KY1500'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1957 (JBW)

KY1500'THE STATION WAS RECOVERED AND THE STATION MARK AND REFERENCE MARKS 1

KY1500'AND 2 WERE FOUND IN GOOD CONDITION.  THE U.S.G.S. MARK WAS NOT

KY1500'RECOVERED.  THE OLD DESCRIPTION IS ADEQUATE WITH THE FOLLOWING

KY1500'ADDITIONS- THE STATION MARK IS NOW 13 FEET EAST OF THE MAST OF RADIO

KY1500'STATION WILE.

KY1500'

KY1500'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 40 AND STATE

KY1500'HIGHWAY 209 AT THE WEST EDGE OF CAMBRIDGE, GO NORTHWEST ON HIGHWAY 209

KY1500'FOR 2.0 MILES TO A PAVED ROAD LEFT.  TURN LEFT AND GO WEST ON THE

KY1500'PAVED ROAD FOR 0.6 MILE TO THE TOP OF A HILL AND THE STATION AT THE

KY1500'RADIO MAST ABOUT 400 FEET SOUTH OF THE ROAD.

KY1500

KY1500                          STATION RECOVERY (1957)

KY1500

KY1500'RECOVERY NOTE BY LOCAL ENGINEER (INDIVIDUAL OR FIRM) 1957

KY1500'REPORT OF LORENZI, DODDS AND CUNNILL, ENGINEERS, BERGER BLDG,

KY1500'PITTSBURG 19, PENN. DATED 30 SEPT. 1957.

KY1500'

KY1500'SURFACE MARK NOW PROJECTS 9 INCHES ABOVE GROUND.  SURFACE AND

KY1500'REFERENCE MARKS ARE IN GOOD CONDITION.  CAMBRIDGE IS AVAILABLE AS AN

KY1500'AZIMUTH MARK, VISIBLE FROM GROUND LEVEL, BUT CAMBRIDGE SHOULD BE

KY1500'CHECKED FOR POSSIBLE DISPLACEMENT BEFORE USING.  USGS STATION COLLEGE

KY1500'COULD NOT BE RECOVERED.  STATION FORD IS 10.8 FEET EAST OF THE

KY1500'SOUTHEAST CORNER OF CONCRETE BASE OF WILE RADIO TOWER.

KY1500

KY1500                          STATION RECOVERY (1975)

KY1500

KY1500'RECOVERY NOTE BY THOMAS ENG AND SURV 1975 (HBW)

KY1500'FORD-1933 UNDISTURBED.  (RM 1) FORD 1933 UNDISTURBED.  (RM 2) FORD

KY1500'1933 UNDISTURBED.  STATION IS UNDER A HEAVY PICKET FENCE SURROUNDING

KY1500'TRANSMITTER TOWER AND NOT OCCUPAYABLE WITHOUT DISTURBING THE FENCE.

KY1500'RM 1 NOW PROJECTS 10 INCHES ABOVE THE GROUND AND RM 2 IS NOW FLUSH AND

KY1500'ON A SLOPE FROM A FILL.

KY1500'

KY1500'DISTANCE AND DIRECTION FROM NEAREST TOWN--3 MILES WNW OF COMBRIDGE,

KY1500'OHIO.

KY1500

KY1500                          STATION RECOVERY (1989)

KY1500

KY1500'RECOVERY NOTE BY MUSKINGUM COUNTY OHIO 1989

KY1500'THE STATION IS LOCATED ABOUT 34 KM (21.15 MI) ENE OF ZANESVILLE AND

KY1500'ABOUT 3 KM (1.85 MI) W OF CAMBRIDGE AT THE WILE RADIO STATION MAST IN

KY1500'A FIELD BEHIND THE RADIO STATION.  OWNERSHIP-- MR. GRANT HAFLEY, P.O.

KY1500'BOX 338, CAMBRIDGE OH 43725, PHONE (614) 432-5605.

KY1500'TO REACH THE STATION FROM THE WESTERN JUNCTION OF U.S. HIGHWAYS 22

KY1500'AND40 AND STATE ROUTE 209 ON THE WEST SIDE OF CAMBRIDGE, GO NW ON

KY1500'STATE ROUTE 209 FOR 2.6 KM (1.60 MI) TO A SIDE ROAD LEFT (COUNTY ROAD

KY1500'418).  TURN LEFT AND GO 2.8 KM (1.75 MI) TO A FORK.  BEAR LEFT FOR

KY1500'0.08KM (0.05 MI) TO WILE RADIO STATION ON THE LEFT AND THE STATION

KY1500'AT THE MAST.

KY1500'THE MARK IS SET IN A 0.3 M (1.0 FT) DIAMETER CONCRETE MONUMENT

KY1500'PROJECTING 0.25 M (0.8 FT) ABOVE THE GROUND.  IT IS 3.4 M (11.2 FT) E

KY1500'OF THE MAST, 4.8 M (15.7 FT) SSE OF THE NE CORNER OF A WOODEN FENCE,

KY1500'2.7 M (8.9 FT) NNE OF THE SE CORNER OF THE FENCE AND 0.5 M (1.6 FT) E

KY1500'OF THE 1.83 M (6.0 FT) HIGH FENCE.

KY1500

KY1500                          STATION RECOVERY (1992)

KY1500

KY1500'RECOVERY NOTE BY AMERICAN ELECTRIC POWER 1992

KY1500'RECOVERED IN GOOD CONDITION.

KY1500

KY1500                          STATION RECOVERY (1992)

KY1500

KY1500'RECOVERY NOTE BY MUSKINGUM COUNTY OHIO 1992 (LCC)

KY1500'RECOVERED AS DESCRIBED.

KY1500'

KY1500'THE STATION IS LOCATED ABOUT 34 KM (21.10 MI) ENE OF ZANESVILLE AND

KY1500'ABOUT 3 KM (1.85 MI) W OF CAMBRIDGE AT THE WILE RADIO STATION MAST IN

KY1500'A FIELD BEHIND THE RADIO STATION. OWNERSIP-- MR. GRNAT HAFLEY, P.O.

KY1500'BOX 338, CAMBRIDGE OH 43725, PHONE (614) 432-5606.

KY1500'

KY1500'TO REACH THE STATION FROM THE WESTERN JUNCTION OF U.S. HIGHWAYS 22 AND

KY1500'40 AND STATE ROUTE 209 ON THE WEST SIDE OF CAMBRIDGE, GO NW ON STATE

KY1500'ROUTE 209 FOR 2.6 KM (1.60 MI) TO A SIDE ROAD LEFT (COUNTY ROAD 418).

KY1500'TURN LEFT AND GO 2.8 KM (1.75 MI) TO A FORK. BEAR LEFT FOR 0.08 KM

KY1500'(0.05 MI) TO 'WILE' RADIO STATION ON THE LEFT AND THE STATION AT THE

KY1500'MAST.

KY1500'

KY1500'THE MARK IS SET IN A 0.3 M (1.0 FT) DIAMTETER CONCRETE MONUMENT

KY1500'PROJECTING 0.25 M (0.82 FT) ABOVE THE GROUND. IT IS 3.4 M (11.2 FT) E

KY1500'OF THE MAST, 4.8 M (15.7 FT) SSE OF THE NE CORNER OF A WOODEN FENCE.

KY1500'2.7 M (8.9 FT) NNE OF THE SE CORNER OF THE FENCE AND 0.5 M (1.6 FT) E

KY1500'OF THE 1.83 M (6.00 FT) HIGH FENCE.

KY1500

KY1500                          STATION RECOVERY (1995)

KY1500

KY1500'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY1500'THE STATION IS LOCATED ABOUT 34 KM (21.10 MI) EAST-NORTHEAST OF

KY1500'ZANESVILLE, 3.2 KM (2.00 MI) WEST OF CAMBRIDGE, AT WILE RADIO STATION,

KY1500'IN A FIELD NEAR THE TOWER.  OWNERSHIP, AVC COMMUNICATIONS INC., DAVID

KY1500'WILSON, OPERATIONS MANAGER, P O BOX 338, CAMBRIDGE, OH 43725,

KY1500'TELEPHONE (614) 432-5605.  ADVANCE NOTICE OF INTENT TO USE THE STATION

KY1500'AFTER NORMAL BUSINESS HOURS IS REQUESTED.  TO REACH THE STATION FROM

KY1500'THE WESTERN JUNCTION OF US HIGHWAYS 22 AND 40 AND STATE ROUTE 209 ON

KY1500'THE WEST SIDE OF CAMBRIDGE, GO NORTHWEST ON ROUTE 209 FOR 2.6 KM (1.60

KY1500'MI) TO COUNTY ROAD 615 (COLLEGE HILL ROAD) ON THE LEFT, TURN LEFT AND

KY1500'GO WESTERLY FOR 2.8 KM (1.75 MI) TO A FORK, BEAR LEFT ONTO COUNTRY

KY1500'ROAD 418 (SKYLINE DRIVE) AND CONTINUE 0.08 KM (0.05 MI) TO WILE RADIO

KY1500'STATION ON THE LEFT.  VEHICLE ACCESS TO THE STATION CAN BE ATTAINED

KY1500'FROM THE EAST END OF THE PARKING LOT BY DRIVING SOUTH ALONG THE MOWED

KY1500'EDGE OF THE FIELD ABOUT 12 M (39.4 FT) EAST OF THE STATION BUILDING.

KY1500'THE MARK IS 3.4 M (11.2 FT) EAST OF THE TOWER, 4.8 M (15.7 FT)

KY1500'SOUTH-SOUTHEAST OF THE NORTHEAST CORNER OF A 1.8 M (5.9 FT) TALL

KY1500'WOODEN FENCE WHICH SURROUNDS THE TOWER, 0.5 M (1.6 FT) EAST OF THE

KY1500'FENCE, SET IN TOP OF A ROUND CONCRETE MONUMENT PROJECTING 0.25 M (0.82

KY1500'FT) .

KY1500

KY1500                          STATION RECOVERY (2017)

KY1500

KY1500'RECOVERY NOTE BY MUSKINGUM COUNTY OHIO 2017 (DPS)

KY1500'RECOVERED AS DESCRIBED.

JX0797  DESIGNATION -  G 236

JX0797  PID         -  JX0797

JX0797  STATE/COUNTY-  OH/WASHINGTON

JX0797  COUNTRY     -  US

JX0797  USGS QUAD   -  LOWER SALEM (2016)

JX0797

JX0797                         *CURRENT SURVEY CONTROL

JX0797  ______________________________________________________________________

JX0797* NAD 83(1995) POSITION- 39 30 20.34606(N) 081 28 10.14603(W)   ADJUSTED

JX0797* NAD 83(1995) ELLIP HT-   162.661 (meters)        (04/01/98)   ADJUSTED

JX0797* NAVD 88 ORTHO HEIGHT -   196.824 (meters)      645.75  (feet) ADJUSTED

JX0797  ______________________________________________________________________

JX0797  GEOID HEIGHT    -        -34.234 (meters)                     GEOID18

JX0797  NAD 83(1995) X  -    730,990.385 (meters)                     COMP

JX0797  NAD 83(1995) Y  - -4,873,414.541 (meters)                     COMP

JX0797  NAD 83(1995) Z  -  4,035,891.155 (meters)                     COMP

JX0797  LAPLACE CORR    -          1.93  (seconds)                    DEFLEC18

JX0797  DYNAMIC HEIGHT  -        196.706 (meters)      645.36  (feet) COMP

JX0797  MODELED GRAVITY -    980,026.0   (mgal)                       NAVD 88

JX0797

JX0797  HORZ ORDER      -  THIRD

JX0797  VERT ORDER      -  FIRST     CLASS II

JX0797  ELLP ORDER      -  FOURTH    CLASS I

JX0797

JX0797.The horizontal coordinates were established by GPS observations

JX0797.and adjusted by the National Geodetic Survey in April 1998.

JX0797

JX0797.The orthometric height was determined by differential leveling and

JX0797.adjusted by the NATIONAL GEODETIC SURVEY

JX0797.in June 1991.

JX0797

JX0797.Significant digits in the geoid height do not necessarily reflect accuracy.

JX0797.GEOID18 height accuracy estimate available here.

JX0797

JX0797.Click photographs - Photos may exist for this station.

JX0797

JX0797.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JX0797

JX0797.The Laplace correction was computed from DEFLEC18 derived deflections.

JX0797

JX0797.The ellipsoidal height was determined by GPS observations

JX0797.and is referenced to NAD 83.

JX0797

JX0797.The dynamic height is computed by dividing the NAVD 88

JX0797.geopotential number by the normal gravity value computed on the

JX0797.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JX0797.degrees latitude (g = 980.6199 gals.).

JX0797

JX0797.The modeled gravity was interpolated from observed gravity values.

JX0797

JX0797. The following values were computed from the NAD 83(1995) position.

JX0797

JX0797;                    North         East     Units Scale Factor Converg.

JX0797;SPC OH S     -   167,653.398   688,623.532   MT  0.99993814   +0 39 14.0

JX0797;SPC OH S     -   550,042.86  2,259,259.04   sFT  0.99993814   +0 39 14.0

JX0797;UTM  17      - 4,372,996.936   459,637.324   MT  0.99962006   -0 17 55.2

JX0797

JX0797!             -  Elev Factor  x  Scale Factor =   Combined Factor

JX0797!SPC OH S     -   0.99997448  x   0.99993814  =   0.99991262

JX0797!UTM  17      -   0.99997448  x   0.99962006  =   0.99959455

JX0797

JX0797_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SMD5963772996(NAD 83)

JX0797

JX0797                          SUPERSEDED SURVEY CONTROL

JX0797

JX0797  NAD 83(1986)-  39 30 20.35275(N)    081 28 10.14993(W) AD(       ) 3

JX0797  NGVD 29 (??/??/92)  197.001  (m)          646.33   (f) ADJ UNCH    1 2

JX0797  NGVD 29             197.00   (m)          646.3    (f) LEVELING    3

JX0797

JX0797.Superseded values are not recommended for survey control.

JX0797

JX0797.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JX0797.See file dsdata.pdf to determine how the superseded data were derived.

JX0797

JX0797_MARKER: DB = BENCH MARK DISK

JX0797_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JX0797_STAMPING: G 236 1959

JX0797_MARK LOGO: CGS

JX0797_PROJECTION: PROJECTING 5 CENTIMETERS

JX0797_MAGNETIC: N = NO MAGNETIC MATERIAL

JX0797_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JX0797+STABILITY: SURFACE MOTION

JX0797_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JX0797+SATELLITE: SATELLITE OBSERVATIONS - October 04, 2016

JX0797

JX0797  HISTORY     - Date     Condition        Report By

JX0797  HISTORY     - 1959     MONUMENTED       CGS

JX0797  HISTORY     - 19901031 GOOD             NGS

JX0797  HISTORY     - 20161004 GOOD             GCT

JX0797

JX0797                          STATION DESCRIPTION

JX0797

JX0797'DESCRIBED BY COAST AND GEODETIC SURVEY 1959

JX0797'2.6 MI SE FROM LOWELL.

JX0797'ABOUT 2.65 MILES SOUTHEAST ALONG THE BALTIMORE AND OHIO RAILROAD

JX0797'FROM THE STATION AT LOWELL, AT MILEPOLE 61-21, AT A PRIVATE

JX0797'DRIVE LEADING SOUTHWEST TO A 1 1/2 STORY HOUSE, 18 1/2 FEET

JX0797'SOUTHWEST OF THE SOUTHWEST RAIL, 11 FEET NORTHWEST OF THE CENTER

JX0797'LINE OF THE DRIVE, 23 FEET WEST OF THE CENTER LINE OF THE

JX0797'CROSSING, 5 FEET NORTHWEST OF A FENCE CORNER, 1 1/2 FEET NORTHEAST

JX0797'OF THE NORTHWEST-SOUTHEAST FENCE LINE, 54 FEET SOUTH OF AND

JX0797'ACROSS THE TRACK FROM THE MILEPOLE, 2 FEET SOUTHEAST OF A STEEL

JX0797'WITNESS POST, ABOUT 3 FEET ABOVE THE LEVEL OF THE TRACK, AND

JX0797'SET IN TOP OF A CONCRETE POST PROJECTING 4 INCHES ABOVE THE

JX0797'GROUND.

JX0797

JX0797                          STATION RECOVERY (1990)

JX0797

JX0797'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1990

JX0797'THE STATION IS LOCATED ABOUT 4.82 KM (3.00 MI) EAST-SOUTHEAST OF

JX0797'LOWELL, 19.3 KM (12.0 MI) NORTHWEST OF MARIETTA, 35.4 KM (22.0 MI)

JX0797'SOUTH-SOUTHWEST OF MACKSBURG AND BETWEEN THE WEST SIDE OF A GRAVEL

JX0797'ROAD AND THE BALTIMORE AND OHIO RAILROAD.  OWNERSHIP--UNKNOWN.

JX0797'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 60 AND 530 IN

JX0797'LOWELL, GO WEST ON STATE HIGHWAY 60 FOR 0.32 KM (0.20 MI) TO THE

JX0797'JUNCTION OF COUNTY ROAD 60 ON THE RIGHT, TURN RIGHT, SOUTH, AND

JX0797'CROSSING THE MUSKINGUM RIVER ON COUNTY ROAD 60 FOR 0.64 KM (0.40 MI)

JX0797'TO A PAVED ROAD ON THE LEFT, TURN LEFT, EAST-SOUTHEASTERLY, FOR 4.26

JX0797'KM (2.65 MI) TO THE STATION ON THE RIGHT, ON THE NORTHWEST SIDE OF A

JX0797'DIRT DRIVE CROSSING THE RAILROAD TRACKS AND LEADING UPHILL AND ACROSS

JX0797'THE ROAD FROM THE ENTRANCE DRIVE TO A FARM HOUSE.

JX0797'THE STATION IS ABOUT 0.92 M (3.02 FT) ABOVE THE TRACKS LOCATED 18.4 M

JX0797'(60.4 FT) SOUTHWEST OF THE CENTER OF THE ROAD, 5.6 M (18.4 FT)

JX0797'SOUTHWEST OF THE SOUTHWEST RAIL OF THE TRACKS, 4.0 M (13.1 FT)

JX0797'NORTHWEST OF THE CENTER OF THE DIRT DRIVE AND 0.6 M (2.0 FT) SOUTHEAST

JX0797'OF A FIBERGLASS WITNESS POST.

JX0797

JX0797                          STATION RECOVERY (2016)

JX0797

JX0797'RECOVERY NOTE BY GUSTIN, COTHERN, AND TUCKER, I 2016 (HWW)

JX0797'RECOVERED IN GOOD CONDITION.

AB6092  CBN         -  This is a Cooperative Base Network Control Station.

AB6092  DESIGNATION -  HOC GAR

AB6092  PID         -  AB6092

AB6092  STATE/COUNTY-  OH/HOCKING

AB6092  COUNTRY     -  US

AB6092  USGS QUAD   -  LOGAN (2016)

AB6092

AB6092                         *CURRENT SURVEY CONTROL

AB6092  ______________________________________________________________________

AB6092* NAD 83(2011) POSITION- 39 32 10.87827(N) 082 26 38.20547(W)   ADJUSTED

AB6092* NAD 83(2011) ELLIP HT-   201.008 (meters)        (06/27/12)   ADJUSTED

AB6092* NAD 83(2011) EPOCH   -  2010.00

AB6092* NAVD 88 ORTHO HEIGHT -   235.1   (meters)      771.    (feet) GPS OBS

AB6092  ______________________________________________________________________

AB6092  NAVD 88 orthometric height was determined with geoid model    GEOID93

AB6092  GEOID HEIGHT    -        -34.268 (meters)                     GEOID93

AB6092  GEOID HEIGHT    -        -34.111 (meters)                     GEOID18

AB6092  NAD 83(2011) X  -    647,722.441 (meters)                     COMP

AB6092  NAD 83(2011) Y  - -4,883,020.316 (meters)                     COMP

AB6092  NAD 83(2011) Z  -  4,038,545.196 (meters)                     COMP

AB6092  LAPLACE CORR    -          0.53  (seconds)                    DEFLEC18

AB6092

AB6092  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

AB6092  Standards:

AB6092         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

AB6092            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

AB6092  -------------------------------------------------------------------

AB6092  NETWORK    1.08   2.16           0.51   0.33   1.10      0.02168536

AB6092  -------------------------------------------------------------------

AB6092  Click here for local accuracies and other accuracy information.

AB6092

AB6092

AB6092.The horizontal coordinates were established by GPS observations

AB6092.and adjusted by the National Geodetic Survey in June 2012.

AB6092

AB6092.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

AB6092.been affixed to the stable North American tectonic plate. See

AB6092.NA2011 for more information.

AB6092

AB6092.The horizontal coordinates are valid at the epoch date displayed above

AB6092.which is a decimal equivalence of Year/Month/Day.

AB6092

AB6092.The orthometric height was determined by GPS observations and a

AB6092.high-resolution geoid model.

AB6092

AB6092.Significant digits in the geoid height do not necessarily reflect accuracy.

AB6092.GEOID18 height accuracy estimate available here.

AB6092

AB6092.Click photographs - Photos may exist for this station.

AB6092

AB6092.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AB6092

AB6092.The Laplace correction was computed from DEFLEC18 derived deflections.

AB6092

AB6092.The ellipsoidal height was determined by GPS observations

AB6092.and is referenced to NAD 83.

AB6092

AB6092. The following values were computed from the NAD 83(2011) position.

AB6092

AB6092;                    North         East     Units Scale Factor Converg.

AB6092;SPC OH S     -   170,557.834   604,818.594   MT  0.99993941   +0 02 08.0

AB6092;SPC OH S     -   559,571.83  1,984,309.00   sFT  0.99993941   +0 02 08.0

AB6092;UTM  17      - 4,377,294.680   375,913.132   MT  0.99978957   -0 55 09.4

AB6092

AB6092!             -  Elev Factor  x  Scale Factor =   Combined Factor

AB6092!SPC OH S     -   0.99996847  x   0.99993941  =   0.99990788

AB6092!UTM  17      -   0.99996847  x   0.99978957  =   0.99975804

AB6092

AB6092_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7591377294(NAD 83)

AB6092

AB6092                          SUPERSEDED SURVEY CONTROL

AB6092

AB6092  NAD 83(2007)-  39 32 10.87837(N)    082 26 38.20625(W) AD(2002.00) 0

AB6092  ELLIP H (02/10/07)  201.021  (m)                       GP(2002.00)

AB6092  ELLIP H (03/08/05)  201.021  (m)                       GP(       ) 4 2

AB6092  NAD 83(1995)-  39 32 10.87857(N)    082 26 38.20615(W) AD(       ) B

AB6092  ELLIP H (08/20/96)  201.030  (m)                       GP(       ) 4 2

AB6092

AB6092.Superseded values are not recommended for survey control.

AB6092

AB6092.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AB6092.See file dsdata.pdf to determine how the superseded data were derived.

AB6092

AB6092_MARKER: I = METAL ROD

AB6092_SETTING: 59 = STAINLESS STEEL ROD IN SLEEVE (10 FT.+)

AB6092_STAMPING: HOC GAR 1995

AB6092_MARK LOGO: OHDT

AB6092_PROJECTION: FLUSH

AB6092_MAGNETIC: N = NO MAGNETIC MATERIAL

AB6092_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

AB6092_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AB6092+SATELLITE: SATELLITE OBSERVATIONS - July 05, 2017

AB6092_ROD/PIPE-DEPTH: 8.5  meters

AB6092_SLEEVE-DEPTH  : 0.91 meters

AB6092

AB6092  HISTORY     - Date     Condition        Report By

AB6092  HISTORY     - 1995     MONUMENTED       OHDT

AB6092  HISTORY     - 20050618 GOOD             GEOCAC

AB6092  HISTORY     - 20170705 GOOD             OHDNR

AB6092

AB6092                          STATION DESCRIPTION

AB6092

AB6092'DESCRIBED BY OHIO DEPARTMENT OF TRANSPORTATION 1995 (CWP)

AB6092'THE MARK IS LOCATED AT THE HOCKING COUNTY, OHIO DEPARTMENT OF

AB6092'TRANSPORTATION GARAGE, APPROXIMATELY 0.23 KM (0.15 MI) SOUTH OF

AB6092'OVERPASS OF US HIGHWAY 33 ON STATE ROUTE 664.  THE MARK IS TO THE LEFT

AB6092'OF ENTRANCE DRIVE TO GARAGE AND DIRECTLY ACROSS FROM SECOND DRIVE TO

AB6092'PARKING LOT FOR HOCKING COUNTY VISITORS CENTER.  TO REACH THE STATION

AB6092'FROM US HIGHWAY 33 TAKE EXIT STATE ROUTE 664 SOUTH, FOR 0.23 KM (0.15

AB6092'MI) TO THE ENTRANCE ROAD TO GARAGE SITE AND THE HOCKING COUNTY

AB6092'VISITORS CENTER.  CONTINUE ON ENTRANCE ROAD TO SECOND ENTRANCE TO

AB6092'VISITORS CENTER PARKING LOT.  ROD MARK IS DIRECTLY NORTH OF DRIVE.

AB6092'THE MARK IS 11.50 M (37.73 FT) NORTH OF NORTH SIDE OF PAVED DRIVE,

AB6092'31.70 M (104.00 FT) EAST OF LIGHT POLE AND 26.60 M (87.27 FT) WEST OF

AB6092'LIGHT POLE.  THE MARK IS A STAINLESS STEEL ROD CLASS B MONUMENT FLUSH

AB6092'WITH THE GROUND AND DRIVEN TO A DEPTH OF 8.5 M (27.9 FT) , ROD MARK IS

AB6092'STAMPED --HOC GAR 1995--.  FIBERGLASS WITNESS POST IS 1.0 M (3.3 FT)

AB6092'WEST OF MARK.  OWNERSHIP--STATE OF OHIO, DEPARTMENT OF TRANSPORTATION,

AB6092'DISTRICT 10 DEPUTY DIRECTOR, MUSKINGUM DRIVE, MARIETTA, OHIO 45750.

AB6092'THE MARK IS ACESSIBLE AT ALL TIMES. NOTE--SLEEVE DEPTH DOES NOT MEET

AB6092'CLASS A REQUIREMENTS.

AB6092

AB6092                          STATION RECOVERY (2005)

AB6092

AB6092'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

AB6092'RECOVERED IN GOOD CONDITION.

AB6092

AB6092                          STATION RECOVERY (2017)

AB6092

AB6092'RECOVERY NOTE BY OHIO DEPARTMENT OF NATURAL RESOURCES 2017 (WAM)

AB6092'FOR MORE INFORMATION CONTACT

AB6092'OHIO DEPARTMENT OF NATURAL RESOURCES

AB6092'OFFICE OF REAL ESTATE

AB6092'SURVEY SECTION

AB6092'2045 MORSE RD. BLDG. E2

AB6092'COLUMBUS, OH 43229

JY1383  DESIGNATION -  HORNER

JY1383  PID         -  JY1383

JY1383  STATE/COUNTY-  OH/MEIGS

JY1383  COUNTRY     -  US

JY1383  USGS QUAD   -  POMEROY (2016)

JY1383

JY1383                         *CURRENT SURVEY CONTROL

JY1383  ______________________________________________________________________

JY1383* NAD 83(1995) POSITION- 39 06 36.38508(N) 082 05 37.30214(W)   ADJUSTED

JY1383* NAVD 88 ORTHO HEIGHT -   308.9   (meters)     1013.    (feet) VERTCON

JY1383  ______________________________________________________________________

JY1383  GEOID HEIGHT    -        -34.133 (meters)                     GEOID18

JY1383  LAPLACE CORR    -         -0.71  (seconds)                    DEFLEC18

JY1383  HORZ ORDER      -  FIRST

JY1383

JY1383.The horizontal coordinates were established by classical geodetic methods

JY1383.and adjusted by the National Geodetic Survey in April 1998.

JY1383.

JY1383.The NAVD 88 height was computed by applying the VERTCON shift value to

JY1383.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY1383

JY1383.Significant digits in the geoid height do not necessarily reflect accuracy.

JY1383.GEOID18 height accuracy estimate available here.

JY1383

JY1383.Click photographs - Photos may exist for this station.

JY1383

JY1383.The Laplace correction was computed from DEFLEC18 derived deflections.

JY1383

JY1383. The following values were computed from the NAD 83(1995) position.

JY1383

JY1383;                    North         East     Units Scale Factor Converg.

JY1383;SPC OH S     -   123,315.523   635,140.151   MT  0.99994730   +0 15 28.1

JY1383;SPC OH S     -   404,577.68  2,083,788.98   sFT  0.99994730   +0 15 28.1

JY1383;UTM  17      - 4,329,564.734   405,440.799   MT  0.99971010   -0 41 23.9

JY1383

JY1383!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1383!SPC OH S     -   0.99995689  x   0.99994730  =   0.99990419

JY1383!UTM  17      -   0.99995689  x   0.99971010  =   0.99966700

JY1383

JY1383:                Primary Azimuth Mark                     Grid Az

JY1383:SPC OH S     -  WEST COLUMBIA CATV MAST                  172 55 38.4

JY1383:UTM  17      -  WEST COLUMBIA CATV MAST                  173 52 30.4

JY1383

JY1383_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SMD0544029564(NAD 83)

JY1383

JY1383|---------------------------------------------------------------------|

JY1383| PID    Reference Object                     Distance      Geod. Az  |

JY1383|                                                           dddmmss.s |

JY1383| JY1377 POMEROY RAD MAST WMPO               APPROX. 7.2 KM 1415639.1 |

JY1383| JY1376 POMEROY GENERAL TEL RAD MAST        APPROX.10.3 KM 1490917.1 |

JY1383| JY1378 POMEROY MEDICAL RAD MAST            APPROX. 9.2 KM 1535649.6 |

JY1383| HY2193 WEST COLUMBIA CATV MAST             APPROX.15.1 KM 1731106.5 |

JY1383| JY1384 HORNER                                6.643 METERS 18517     |

JY1383| HY2196 CHESHIRE GAVIN STACK                APPROX.19.5 KM 1854810.6 |

JY1383| HY2211 KYGER CREEK POWER PLANT STK         APPROX.21.9 KM 1875331.4 |

JY1383| AI9787 HORNER RM 1                          16.919 METERS 19439     |

JY1383| JY1386 HORNER RM 3 A PT                    267.292 METERS 31805     |

JY1383| JY1385 HORNER RM 3 AZIMUTH                 271.337 METERS 31857     |

JY1383| AI9788 HORNER RM 2                          14.610 METERS 34853     |

JY1383|---------------------------------------------------------------------|

JY1383

JY1383                          SUPERSEDED SURVEY CONTROL

JY1383

JY1383  NAD 83(1993)-  39 06 36.38811(N)    082 05 37.30806(W) AD(       ) 1

JY1383  NAD 83(1986)-  39 06 36.38955(N)    082 05 37.30628(W) AD(       ) 1

JY1383  NAD 27      -  39 06 36.11368(N)    082 05 37.79435(W) AD(       ) 1

JY1383  NGVD 29 (07/19/86)  309.1    (m)         1014.     (f) VERT ANG

JY1383

JY1383.Superseded values are not recommended for survey control.

JY1383

JY1383.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1383.See file dsdata.pdf to determine how the superseded data were derived.

JY1383

JY1383_MARKER: DH = HORIZONTAL CONTROL DISK

JY1383_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1383_STAMPING: HORNER 1975

JY1383_MARK LOGO: NGS

JY1383_PROJECTION: FLUSH

JY1383_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY1383+STABILITY: SURFACE MOTION

JY1383_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY1383+SATELLITE: SATELLITE OBSERVATIONS - September 29, 2002

JY1383

JY1383  HISTORY     - Date     Condition        Report By

JY1383  HISTORY     - 1975     MONUMENTED       NGS

JY1383  HISTORY     - 20020929 GOOD             USPSQD

JY1383

JY1383                          STATION DESCRIPTION

JY1383

JY1383'DESCRIBED BY NATIONAL GEODETIC SURVEY 1975 (JRS)

JY1383'THE STATION IS ABOUT 7 MILES NORTHWEST OF POMEROY, 10 MILES

JY1383'SOUTHEAST OF ALBANY AND ON PROPERTY MRS. IRENE ARNOLD, WHO

JY1383'LIVES 0.35 MILE NORTH OF THE STATION.

JY1383'

JY1383'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 7 AND

JY1383'143, NORTHWEST OF POMEROY, GO NORTHWEST ON STATE HIGHWAY 143 FOR

JY1383'5.6 MILES TO A FORK AT THE SUMMIT OF A GRADE. BEAR RIGHT AND GO

JY1383'NORTHERLY AND UPHILL ON A GRAVEL ROAD FOR 0.45 MILE TO A SIDE

JY1383'ROAD RIGHT. TURN RIGHT AND GO EAST FOR 0.3 MILE TO A T-ROAD

JY1383'INTERSECTION, TURN RIGHT AND GO SOUTH FOR 0.05 MILE TO THE

JY1383'AZIMUTH MARK ON THE LEFT, CONTINUE SOUTH FOR 0.25 MILE TO A

JY1383'GATE IN THE FENCE LINE ON THE LEFT, TURN LEFT AND GO UPHILL

JY1383'FOR 0.05 MILE ACROSS PASTURE TO THE TOP OF THE HILL AND THE

JY1383'STATION.

JY1383'

JY1383'STATION MARK IS A STANDARD DISK STAMPED---HORNER 1975---, SET IN

JY1383'THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT IS

JY1383'FLUSH WITH THE GROUND. THERE ARE NO LOCAL OBJECTS TO REFERENCE

JY1383'TO.

JY1383'

JY1383'REFERENCE MARK 1 IS A STANDARD DISK STAMPED---HORNER NO 1 1975---,

JY1383'SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT

JY1383'IS FLUSH WITH THE GROUND. IT IS ABOUT 1 FOOT LOWER THAN THE

JY1383'STATION AND THERE ARE NO LOCAL OBJECTS.

JY1383'

JY1383'REFERENCE MARK 2 IS A STANDARD DISK STAMPED---HORNER NO 2 1975---,

JY1383'SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT

JY1383'IS FLUSH WITH THE GROUND. IT IS ABOUT 2 FEET LOWER THAN THE

JY1383'STATION AND THERE ARE NO LOCAL OBJECTS.

JY1383'

JY1383'HORNER USGS 1976 IS A BRONZE DISK OF THE U.S. GEOLOGICAL SURVEY

JY1383'THAT IS SET IN THE TOP OF AN 8-INCH SQUARE CONCRETE MONUMENT

JY1383'THAT PROJECTS 9 INCHES ABOVE THE GROUND. IT IS ABOUT THE SAME

JY1383'ELEVATION AS THE STATION AND THERE ARE NO LOCAL OBJECTS.

JY1383'

JY1383'AZIMUTH MARK IS A STANDARD DISK STAMPED ---HORNER AZ NO 3 1975---,

JY1383'SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT

JY1383'IS FLUSH WITH THE GROUND. IT IS 2.2 FEET SOUTH OF A METAL

JY1383'WITNESS POST, 1.7 FEET EAST OF A FENCE AND 10.0 FEET EAST OF THE

JY1383'APPROXIMATE CENTER OF A GRAVELED ROAD.

JY1383'

JY1383'HEIGHT OF LIGHT ABOVE THE STATION WAS 22.3 METERS.

JY1383'A TRAVERSE TIE WAS MADE AT THIS DATE TO HORNER USGS 1976.

JY1383

JY1383                          STATION RECOVERY (2002)

JY1383

JY1383'RECOVERY NOTE BY US POWER SQUADRON 2002 (BAS)

JY1383'STATION DESCRIPTION - THERE IS NO LONGER A FENCE LINE OR PASTURE.  THE

JY1383'STATION IS ON TOP OF THE HILL.  THE PROPERTY BELONGS TO DANNY AND

JY1383'PATRICIA ARNOLD WHERE THEY HAVE A HOME ON TOP OF ARNOLD MOUNTAIN

JY0252  DESIGNATION -  L 233

JY0252  PID         -  JY0252

JY0252  STATE/COUNTY-  OH/ATHENS

JY0252  COUNTRY     -  US

JY0252  USGS QUAD   -  SHADE (2016)

JY0252

JY0252                         *CURRENT SURVEY CONTROL

JY0252  ______________________________________________________________________

JY0252* NAD 83(2011) POSITION- 39 14 20.39664(N) 082 03 45.26431(W)   ADJUSTED

JY0252* NAD 83(2011) ELLIP HT-   257.209 (meters)        (06/27/12)   ADJUSTED

JY0252* NAD 83(2011) EPOCH   -  2010.00

JY0252* NAVD 88 ORTHO HEIGHT -   291.435 (meters)      956.15  (feet) ADJUSTED

JY0252  ______________________________________________________________________

JY0252  GEOID HEIGHT    -        -34.207 (meters)                     GEOID18

JY0252  NAD 83(2011) X  -    683,109.234 (meters)                     COMP

JY0252  NAD 83(2011) Y  - -4,899,392.920 (meters)                     COMP

JY0252  NAD 83(2011) Z  -  4,013,064.960 (meters)                     COMP

JY0252  LAPLACE CORR    -         -1.37  (seconds)                    DEFLEC18

JY0252  DYNAMIC HEIGHT  -        291.255 (meters)      955.56  (feet) COMP

JY0252  MODELED GRAVITY -    980,001.8   (mgal)                       NAVD 88

JY0252

JY0252  VERT ORDER      -  SECOND    CLASS 0

JY0252

JY0252  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

JY0252  Standards:

JY0252         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

JY0252            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

JY0252  -------------------------------------------------------------------

JY0252  NETWORK    1.87   3.16           0.85   0.65   1.61     -0.06107428

JY0252  -------------------------------------------------------------------

JY0252  Click here for local accuracies and other accuracy information.

JY0252

JY0252

JY0252.The horizontal coordinates were established by GPS observations

JY0252.and adjusted by the National Geodetic Survey in June 2012.

JY0252

JY0252.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

JY0252.been affixed to the stable North American tectonic plate. See

JY0252.NA2011 for more information.

JY0252

JY0252.The horizontal coordinates are valid at the epoch date displayed above

JY0252.which is a decimal equivalence of Year/Month/Day.

JY0252

JY0252.The orthometric height was determined by differential leveling and

JY0252.adjusted by the NATIONAL GEODETIC SURVEY

JY0252.in June 1991.

JY0252

JY0252.Significant digits in the geoid height do not necessarily reflect accuracy.

JY0252.GEOID18 height accuracy estimate available here.

JY0252

JY0252.Click photographs - Photos may exist for this station.

JY0252

JY0252.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JY0252

JY0252.The Laplace correction was computed from DEFLEC18 derived deflections.

JY0252

JY0252.The ellipsoidal height was determined by GPS observations

JY0252.and is referenced to NAD 83.

JY0252

JY0252.The dynamic height is computed by dividing the NAVD 88

JY0252.geopotential number by the normal gravity value computed on the

JY0252.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0252.degrees latitude (g = 980.6199 gals.).

JY0252

JY0252.The modeled gravity was interpolated from observed gravity values.

JY0252

JY0252. The following values were computed from the NAD 83(2011) position.

JY0252

JY0252;                    North         East     Units Scale Factor Converg.

JY0252;SPC OH S     -   137,636.559   637,762.436   MT  0.99993911   +0 16 39.2

JY0252;SPC OH S     -   451,562.61  2,092,392.26   sFT  0.99993911   +0 16 39.2

JY0252;UTM  17      - 4,343,837.018   408,299.183   MT  0.99970354   -0 40 19.9

JY0252

JY0252!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY0252!SPC OH S     -   0.99995965  x   0.99993911  =   0.99989876

JY0252!UTM  17      -   0.99995965  x   0.99970354  =   0.99966320

JY0252

JY0252_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SMD0829943837(NAD 83)

JY0252

JY0252                          SUPERSEDED SURVEY CONTROL

JY0252

JY0252  NAD 83(2007)-  39 14 20.39677(N)    082 03 45.26507(W) AD(2002.00) 0

JY0252  ELLIP H (02/10/07)  257.207  (m)                       GP(2002.00)

JY0252  NAD 83(1995)-  39 14 20.39704(N)    082 03 45.26511(W) AD(       ) 1

JY0252  ELLIP H (11/14/05)  257.262  (m)                       GP(       ) 4 2

JY0252  NAVD 88             291.44   (m)          956.2    (f) LEVELING    3

JY0252  NGVD 29 (??/??/92)  291.615  (m)          956.74   (f) ADJ UNCH    2 0

JY0252

JY0252.Superseded values are not recommended for survey control.

JY0252

JY0252.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0252.See file dsdata.pdf to determine how the superseded data were derived.

JY0252

JY0252_MARKER: DB = BENCH MARK DISK

JY0252_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0252_STAMPING: L 233 1958

JY0252_MARK LOGO: CGS

JY0252_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0252+STABILITY: SURFACE MOTION

JY0252_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY0252+SATELLITE: SATELLITE OBSERVATIONS - November 11, 2004

JY0252

JY0252  HISTORY     - Date     Condition        Report By

JY0252  HISTORY     - 1958     MONUMENTED       CGS

JY0252  HISTORY     - 20041111 GOOD             LOCSUR

JY0252

JY0252                          STATION DESCRIPTION

JY0252

JY0252'DESCRIBED BY COAST AND GEODETIC SURVEY 1958

JY0252'2.2 MI NW FROM SHADE.

JY0252'ABOUT 1.25 MILES NORTHWEST ALONG U.S. HIGHWAY 33 FROM THE SCHOOL

JY0252'AT SHADE, THENCE 0.95 MILE WEST ALONG A GRAVEL ROAD TO PLEASANTON,

JY0252'ABOUT 2.2 MILES EAST ALONG A GRAVEL ROAD FROM THE METHODIST

JY0252'CHURCH AND CEMETERY AT PLEASANTON, 45 FEET SOUTH OF THE CENTER

JY0252'LINE OF THE ROAD, 1 FOOT WEST OF A FENCE AROUND CEMETERY, 25 FEET

JY0252'SOUTH OF FENCE CORNER, 20 1/2 FEET SOUTH OF THE CENTER OF CEMETERY

JY0252'GATE, 42 FEET WEST-NORTHWEST OF THE NORTHWEST CORNER OF GEORGE

JY0252'AND ELIZABETH JEFFERS TOMBSTONE, ABOUT 6 FEET ABOVE THE LEVEL

JY0252'OF THE ROAD, AND SET IN THE TOP OF A CONCRETE POST PROJECTING 3

JY0252'INCHES.

JY0252

JY0252                          STATION RECOVERY (2004)

JY0252

JY0252'RECOVERY NOTE BY LOCAL SURVEYOR (INDIVIDUAL OR FIRM) 2004 (WDN)

JY0252'FOUND AS DESCRIBED

DG7198  CBN         -  This is a Cooperative Base Network Control Station.

DG7198  DESIGNATION -  LANC

DG7198  PID         -  DG7198

DG7198  STATE/COUNTY-  OH/FAIRFIELD

DG7198  COUNTRY     -  US

DG7198  USGS QUAD   -  LANCASTER (2016)

DG7198

DG7198                         *CURRENT SURVEY CONTROL

DG7198  ______________________________________________________________________

DG7198* NAD 83(2011) POSITION- 39 42 30.96549(N) 082 35 06.48243(W)   ADJUSTED

DG7198* NAD 83(2011) ELLIP HT-   213.053 (meters)        (06/27/12)   ADJUSTED

DG7198* NAD 83(2011) EPOCH   -  2010.00

DG7198* NAVD 88 ORTHO HEIGHT -   247.2   (meters)      811.    (feet) GPS OBS

DG7198  ______________________________________________________________________

DG7198  NAVD 88 orthometric height was determined with geoid model    GEOID03

DG7198  GEOID HEIGHT    -        -34.170 (meters)                     GEOID03

DG7198  GEOID HEIGHT    -        -34.170 (meters)                     GEOID18

DG7198  NAD 83(2011) X  -    634,114.994 (meters)                     COMP

DG7198  NAD 83(2011) Y  - -4,872,516.478 (meters)                     COMP

DG7198  NAD 83(2011) Z  -  4,053,283.919 (meters)                     COMP

DG7198  LAPLACE CORR    -         -0.62  (seconds)                    DEFLEC18

DG7198

DG7198  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

DG7198  Standards:

DG7198         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

DG7198            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

DG7198  -------------------------------------------------------------------

DG7198  NETWORK    0.72   1.86           0.32   0.26   0.95     -0.01993262

DG7198  -------------------------------------------------------------------

DG7198  Click here for local accuracies and other accuracy information.

DG7198

DG7198

DG7198.The horizontal coordinates were established by GPS observations

DG7198.and adjusted by the National Geodetic Survey in June 2012.

DG7198

DG7198.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

DG7198.been affixed to the stable North American tectonic plate. See

DG7198.NA2011 for more information.

DG7198

DG7198.The horizontal coordinates are valid at the epoch date displayed above

DG7198.which is a decimal equivalence of Year/Month/Day.

DG7198

DG7198.The orthometric height was determined by GPS observations and a

DG7198.high-resolution geoid model.

DG7198

DG7198.Significant digits in the geoid height do not necessarily reflect accuracy.

DG7198.GEOID18 height accuracy estimate available here.

DG7198

DG7198.Click photographs - Photos may exist for this station.

DG7198

DG7198.The X, Y, and Z were computed from the position and the ellipsoidal ht.

DG7198

DG7198.The Laplace correction was computed from DEFLEC18 derived deflections.

DG7198

DG7198.The ellipsoidal height was determined by GPS observations

DG7198.and is referenced to NAD 83.

DG7198

DG7198. The following values were computed from the NAD 83(2011) position.

DG7198

DG7198;                    North         East     Units Scale Factor Converg.

DG7198;SPC OH S     -   189,682.799   592,699.623   MT  0.99995188   -0 03 14.5

DG7198;SPC OH S     -   622,317.65  1,944,548.68   sFT  0.99995188   -0 03 14.5

DG7198;UTM  17      - 4,396,616.705   364,116.822   MT  0.99982732   -1 00 46.3

DG7198

DG7198!             -  Elev Factor  x  Scale Factor =   Combined Factor

DG7198!SPC OH S     -   0.99996658  x   0.99995188  =   0.99991846

DG7198!UTM  17      -   0.99996658  x   0.99982732  =   0.99979390

DG7198

DG7198_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD6411696616(NAD 83)

DG7198

DG7198                          SUPERSEDED SURVEY CONTROL

DG7198

DG7198  NAD 83(2007)-  39 42 30.96562(N)    082 35 06.48320(W) AD(2002.00) 0

DG7198  ELLIP H (02/10/07)  213.067  (m)                       GP(2002.00)

DG7198  NAD 83(1995)-  39 42 30.96563(N)    082 35 06.48317(W) AD(       ) A

DG7198  ELLIP H (09/23/04)  213.065  (m)                       GP(       ) 4 1

DG7198

DG7198.Superseded values are not recommended for survey control.

DG7198

DG7198.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

DG7198.See file dsdata.pdf to determine how the superseded data were derived.

DG7198

DG7198_MARKER: I = METAL ROD

DG7198_SETTING: 59 = STAINLESS STEEL ROD IN SLEEVE (10 FT.+)

DG7198_STAMPING: LANC 2003

DG7198_MARK LOGO: NGS

DG7198_PROJECTION: FLUSH

DG7198_MAGNETIC: N = NO MAGNETIC MATERIAL

DG7198_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

DG7198_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

DG7198+SATELLITE: SATELLITE OBSERVATIONS - April 01, 2003

DG7198_ROD/PIPE-DEPTH: 25.3 meters

DG7198_SLEEVE-DEPTH  :  0.9 meters

DG7198

DG7198  HISTORY     - Date     Condition        Report By

DG7198  HISTORY     - 20030401 MONUMENTED       WOOLPT

DG7198

DG7198                          STATION DESCRIPTION

DG7198

DG7198'DESCRIBED BY WOOLPERT CONSULTANTS 2003 (GTF)

DG7198'DESCRIBED BY WOOLPERT LLP IN 2003. (GTF)

DG7198'

DG7198'THE STATION IS LOCATED IN LANCASTER, AT THE WATER POLLUTION  CONTROL

DG7198'FACILITY.

DG7198'

DG7198'TO REACH THE STATION FROM THE INTERSECTION OF US HIGHWAYS 33 AND 22

DG7198'(MAIN STREET) IN DOWNTOWN LANCASTER, GO EAST ALONG HIGHWAY 22 FOR 1.0

DG7198'MI TO WAYNEDOTTE STREET.  TURN RIGHT AND GO SOUTH ALONG  WAYNEDOTTE

DG7198'STREET FOR 0.2 MI TO LAWRENCE STREET.  TURN LEFT  AND GO SOUTHEAST FOR

DG7198'0.15 MI TO THE ENTRANCE DRIVE OF WATER  POLLUTION CONTROL FACILITY.

DG7198'TURN RIGHT AND GO SOUTHWEST  FOR ABOUT 100 FT TO THE STATION ON THE

DG7198'LEFT.

DG7198'

DG7198'THE STATION IS AN ENCASED STAINLESS STEEL ROD DRIVEN TO REFUSAL.   THE

DG7198'STATION IS LOCATED 141.5 FT SOUTH-SOUTHWEST OF THE CENTERLINE  OF

DG7198'LAWRENCE STREET, 102.4 FT SOUTHEAST OF CENTERLINE OF  ENTRANCE DRIVE,

DG7198'82.6 FT SOUTHEAST OF POWER POLE NUMBER 910D4-230, 147.4 FT  NORTH OF

DG7198'FACILITY FENCE LINE, 153.1 FT WEST OF POWER POLE NUMBER  910D4-262,

DG7198'AND 1.0 FT NORTH OF A CARSONITE WITNESS POST.

KY3543  CBN         -  This is a Cooperative Base Network Control Station.

KY3543  PACS        -  This is a Primary Airport Control Station.

KY3543  DESIGNATION -  MILPORT

KY3543  PID         -  KY3543

KY3543  STATE/COUNTY-  OH/HOLMES

KY3543  COUNTRY     -  US

KY3543  USGS QUAD   -  MILLERSBURG (2016)

KY3543

KY3543                         *CURRENT SURVEY CONTROL

KY3543  ______________________________________________________________________

KY3543* NAD 83(2011) POSITION- 40 32 15.13913(N) 081 57 29.31306(W)   ADJUSTED

KY3543* NAD 83(2011) ELLIP HT-   339.167 (meters)        (06/27/12)   ADJUSTED

KY3543* NAD 83(2011) EPOCH   -  2010.00

KY3543* NAVD 88 ORTHO HEIGHT -   372.40  (meters)     1221.8   (feet) GPS OBS

KY3543  ______________________________________________________________________

KY3543  NAVD 88 orthometric height was determined with geoid model    GEOID93

KY3543  GEOID HEIGHT    -        -33.279 (meters)                     GEOID93

KY3543  GEOID HEIGHT    -        -33.230 (meters)                     GEOID18

KY3543  NAD 83(2011) X  -    679,111.441 (meters)                     COMP

KY3543  NAD 83(2011) Y  - -4,806,647.229 (meters)                     COMP

KY3543  NAD 83(2011) Z  -  4,123,749.168 (meters)                     COMP

KY3543  LAPLACE CORR    -          0.55  (seconds)                    DEFLEC18

KY3543

KY3543  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY3543  Standards:

KY3543         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY3543            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY3543  -------------------------------------------------------------------

KY3543  NETWORK    1.67   3.23           0.78   0.54   1.65      0.04158475

KY3543  -------------------------------------------------------------------

KY3543  Click here for local accuracies and other accuracy information.

KY3543

KY3543

KY3543.This mark is at Holmes County Airport (10G)

KY3543

KY3543.The horizontal coordinates were established by GPS observations

KY3543.and adjusted by the National Geodetic Survey in June 2012.

KY3543

KY3543.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY3543.been affixed to the stable North American tectonic plate. See

KY3543.NA2011 for more information.

KY3543

KY3543.The horizontal coordinates are valid at the epoch date displayed above

KY3543.which is a decimal equivalence of Year/Month/Day.

KY3543

KY3543.The orthometric height was determined by GPS observations and a

KY3543.high-resolution geoid model.

KY3543

KY3543.GPS derived orthometric heights for airport stations designated as

KY3543.PACS or SACS are published to 2 decimal places.  This maintains

KY3543.centimeter relative accuracy between the PACS and SACS.  It does

KY3543.not indicate centimeter accuracy relative to other marks which are

KY3543.part of the NAVD 88 network.

KY3543

KY3543.Significant digits in the geoid height do not necessarily reflect accuracy.

KY3543.GEOID18 height accuracy estimate available here.

KY3543

KY3543.Click photographs - Photos may exist for this station.

KY3543

KY3543.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY3543

KY3543.The Laplace correction was computed from DEFLEC18 derived deflections.

KY3543

KY3543.The ellipsoidal height was determined by GPS observations

KY3543.and is referenced to NAD 83.

KY3543

KY3543. The following values were computed from the NAD 83(2011) position.

KY3543

KY3543;                    North         East     Units Scale Factor Converg.

KY3543;SPC OH N     -    96,849.908   645,905.298   MT  0.99998167   +0 21 21.5

KY3543;SPC OH N     -   317,748.41  2,119,107.63   sFT  0.99998167   +0 21 21.5

KY3543;UTM  17      - 4,487,862.515   418,857.566   MT  0.99968104   -0 37 22.0

KY3543

KY3543!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY3543!SPC OH N     -   0.99994680  x   0.99998167  =   0.99992847

KY3543!UTM  17      -   0.99994680  x   0.99968104  =   0.99962786

KY3543

KY3543:                Primary Azimuth Mark                     Grid Az

KY3543:SPC OH N     -  MILPORT AZ MK                            091 31 23.0

KY3543:UTM  17      -  MILPORT AZ MK                            092 30 06.5

KY3543

KY3543_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME1885787862(NAD 83)

KY3543

KY3543|---------------------------------------------------------------------|

KY3543| PID    Reference Object                     Distance      Geod. Az  |

KY3543|                                                           dddmmss.s |

KY3543| KY3546 MILPORT AZ MK                       APPROX. 0.5 KM 0915244.5 |

KY3543|---------------------------------------------------------------------|

KY3543

KY3543                          SUPERSEDED SURVEY CONTROL

KY3543

KY3543  NAD 83(2007)-  40 32 15.13948(N)    081 57 29.31378(W) AD(2002.00) 0

KY3543  ELLIP H (02/10/07)  339.167  (m)                       GP(2002.00)

KY3543  ELLIP H (03/08/05)  339.186  (m)                       GP(       ) 4 2

KY3543  NAD 83(1995)-  40 32 15.13905(N)    081 57 29.31364(W) AD(       ) B

KY3543  ELLIP H (08/20/96)  339.188  (m)                       GP(       ) 4 2

KY3543  NAD 83(1986)-  40 32 15.14196(N)    081 57 29.31496(W) AD(       ) 3

KY3543  NAD 27      -  40 32 14.92432(N)    081 57 29.79102(W) AD(       ) 3

KY3543  NGVD 29 (02/23/89)  372.67   (m)  RAPSU86 model used   GPS OBS

KY3543

KY3543.Superseded values are not recommended for survey control.

KY3543

KY3543.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY3543.See file dsdata.pdf to determine how the superseded data were derived.

KY3543

KY3543_MARKER: DH = HORIZONTAL CONTROL DISK

KY3543_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY3543_STAMPING: MILLPORT 1986

KY3543_MARK LOGO: NGS

KY3543_MAGNETIC: N = NO MAGNETIC MATERIAL

KY3543_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY3543+STABILITY: SURFACE MOTION

KY3543_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY3543+SATELLITE: SATELLITE OBSERVATIONS - March 23, 1998

KY3543

KY3543  HISTORY     - Date     Condition        Report By

KY3543  HISTORY     - 1986     MONUMENTED       NGS

KY3543  HISTORY     - 19871013 GOOD

KY3543  HISTORY     - 19950816 GOOD             NGS

KY3543  HISTORY     - 19980323 GOOD             GCS

KY3543

KY3543                          STATION DESCRIPTION

KY3543

KY3543'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

KY3543'THE STATION IS LOCATED AT THE MILLERSBURG/HOLMES COUNTY AIRPORT ABOUT

KY3543'2.3 MILES SOUTHWEST OF MILLERSBURG.  OWNERSHIP HOLMES COUNTY, AIRPORT

KY3543'MANAGER JIM ROCK 4510 CROWN HILL DRIVE MILLERSBURG OHIO 44654

KY3543'PHONE 216-674-7761

KY3543'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 39, 83 AND

KY3543'US HIGHWAY 62 IN MILLERSBURG.  GO WEST ON STATE HIGHWAY 39 FOR 0.75

KY3543'MILES TO COUNTY ROAD 292 ON THE LEFT.  TURN LEFT, SOUTHWEST, AND GO 2.

KY3543'MILES TO A FORK, BEAR LEFT ON TOWNSHIP ROAD 307 FOR

KY3543'0.15 MILE TO THE AIRPORT ADMINISTRATION ON THE RIGHT.  TO REACH THE

KY3543'AZIMUTH MARK FROM THE FRONT OF THE ADMINISTRATION BUILDING GO EAST ON

KY3543'THE TAXIWAY FOR 0.15 MILE TO THE AZIMUTH MARK ON THE LEFT AT THE

KY3543'RUNWAY JUNCTION.  CONTINUE WEST FROM THE ADMINISTRATION BUILDING FOR 0

KY3543'0.1 MILE TO THE WEST END OF THE TARMAC AND THE STATION.

KY3543'THE STATION MARKS ARE STANDARD NGS DISKS STAMPED --MILLPORT 1986--.

KY3543'THE SURFACE DISK IS SET IN THE TOP OF 30 CM ROUND CONCRETE POST FLUSH

KY3543'WITH THE GROUND.  IT IS 25.1 METERS (82.5 FEET) EAST OF AN OLD WELL

KY3543'CASING AND PUMP, 17.4 METERS ( 57 FEET) SOUTH-SOUTHWEST FROM THE

KY3543'SOUTHWEST CORNER OF HANGER, 12.5 METERS ( 41.0 FEET) NORTH OF TAXIWAY

KY3543'CENTERLINE, 6.6 METERS (21.5 FEET) WEST FROM THE WEST EDGE OF TARMAC

KY3543'AND 0.3 METERS ( 1.0 FEET) EAST OF FIBERGLASS WITNESS POST.  THE

KY3543'UNDERGROUND IS ABOUT 1.0 METERS BELOW GROUND.

KY3543'DESCRIBED BY B.L. LAMBERT.  TYPED BY JAMES MALONEY 9/07/87.

KY3543

KY3543                          STATION RECOVERY (1987)

KY3543

KY3543'RECOVERED 1987

KY3543'RECOVERED IN GOOD CONDITION.

KY3543

KY3543                          STATION RECOVERY (1995)

KY3543

KY3543'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY3543'THE STATION IS LOCATED ABOUT 4.0 KM (2.50 MI) SOUTHWEST OF

KY3543'MILLERSBURG, AT THE HOLMES COUNTY AIRPORT.  IN THE GRASS, NEAR THE

KY3543'WEST END OF THE APRON THAT IS IN FRONT OF THE OFFICE AND HANGAR.

KY3543'OWNERSHIP--HOLMES COUNTY, 4501 TOWNSHIP ROAD NUMBER 307, MILLERSBURG,

KY3543'OHIO, 44654.  AIRPORT MANAGER LARRY CLARK, PHONE 330-674-2686.

KY3543'NOTE--THIS STATION WAS SELECTED AS A (PACS) .  THE DESIGNATION WAS

KY3543'CHANGED FROM MILLPORT TO MILPORT, TO COINCIDE WITH THE STAMPING.  TO

KY3543'REACH THE STATION FROM THE JUNCTION OF US HIGHWAY 62 AND STATE

KY3543'HIGHWAYS 39 AND 83 IN MILLERSBURG, GO WEST 1.23 KM (0.75 MI) ON

KY3543'HIGHWAY 39 TO A ROAD LEFT WITH AIRPORT SIGN.  TURN LEFT, SOUTH, 3.17

KY3543'KM (1.95 MI) ALONG WINDING ROAD (COUNTY 292) TO THE AIRPORT ENTRANCE

KY3543'ROAD ON THE LEFT.  BEAR LEFT, SOUTH, THEN WEST, 0.2 KM (0.10 MI) TO

KY3543'THE OFFICE ON THE RIGHT.  CONTINUE WEST 0.15 KM (0.10 MI) TO THE END

KY3543'OF THE APRON AND THE STATION 6.4 M (21.0 FT) WEST OF THE WEST EDGE OF

KY3543'THE APRON.  STATION IS 25.2 M (82.7 FT) EAST OF A WELL HEAD AND PIPE,

KY3543'17.3 M (56.8 FT) SOUTHWEST OF THE SOUTHWEST CORNER OF A LONG HANGAR,

KY3543'8.8 M (28.9 FT) NORTHWEST OF THE TAXI CENTERLINE, 6.4 M (21.0 FT) WEST

KY3543'OF THE WEST EDGE OF THE APRON, 0.3 M (1.0 FT) EAST OF A WITNESS POST,

KY3543'AND THE MONUMENT IS 0.1 M (0.3 FT) ABOVE THE LEVEL OF THE TAXI AND

KY3543'FLUSH WITH THE GROUND SURFACE.

KY3543

KY3543                          STATION RECOVERY (1998)

KY3543

KY3543'RECOVERY NOTE BY GEODETIC CONSULTING SERVICES 1998 (KDZ)

KY3543'RECOVERED AS DESCRIBED.

KY3547  FBN         -  This is a Federal Base Network Control Station.

KY3547  PACS        -  This is a Primary Airport Control Station.

KY3547  DESIGNATION -  PHILPORT

KY3547  PID         -  KY3547

KY3547  STATE/COUNTY-  OH/TUSCARAWAS

KY3547  COUNTRY     -  US

KY3547  USGS QUAD   -  NEW PHILADELPHIA (2016)

KY3547

KY3547                         *CURRENT SURVEY CONTROL

KY3547  ______________________________________________________________________

KY3547* NAD 83(2011) POSITION- 40 28 12.78251(N) 081 25 06.96724(W)   ADJUSTED

KY3547* NAD 83(2011) ELLIP HT-   237.423 (meters)        (06/27/12)   ADJUSTED

KY3547* NAD 83(2011) EPOCH   -  2010.00

KY3547* NAVD 88 ORTHO HEIGHT -   270.882 (meters)      888.72  (feet) ADJUSTED

KY3547  ______________________________________________________________________

KY3547  GEOID HEIGHT    -        -33.463 (meters)                     GEOID18

KY3547  NAD 83(2011) X  -    725,056.686 (meters)                     COMP

KY3547  NAD 83(2011) Y  - -4,804,763.974 (meters)                     COMP

KY3547  NAD 83(2011) Z  -  4,117,998.630 (meters)                     COMP

KY3547  LAPLACE CORR    -         -1.14  (seconds)                    DEFLEC18

KY3547  DYNAMIC HEIGHT  -        270.749 (meters)      888.28  (feet) COMP

KY3547  MODELED GRAVITY -    980,125.7   (mgal)                       NAVD 88

KY3547

KY3547  VERT ORDER      -  SECOND    CLASS II

KY3547

KY3547  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY3547  Standards:

KY3547         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY3547            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY3547  -------------------------------------------------------------------

KY3547  NETWORK    0.45   1.22           0.21   0.15   0.62      0.06730696

KY3547  -------------------------------------------------------------------

KY3547  Click here for local accuracies and other accuracy information.

KY3547

KY3547

KY3547.This mark is at Harry Clever Field Airport (PHD)

KY3547

KY3547.The horizontal coordinates were established by GPS observations

KY3547.and adjusted by the National Geodetic Survey in June 2012.

KY3547

KY3547.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY3547.been affixed to the stable North American tectonic plate. See

KY3547.NA2011 for more information.

KY3547

KY3547.The horizontal coordinates are valid at the epoch date displayed above

KY3547.which is a decimal equivalence of Year/Month/Day.

KY3547

KY3547.The orthometric height was determined by differential leveling and

KY3547.adjusted by the NATIONAL GEODETIC SURVEY

KY3547.in August 1996.

KY3547

KY3547.Significant digits in the geoid height do not necessarily reflect accuracy.

KY3547.GEOID18 height accuracy estimate available here.

KY3547

KY3547.Click photographs - Photos may exist for this station.

KY3547

KY3547.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY3547

KY3547.The Laplace correction was computed from DEFLEC18 derived deflections.

KY3547

KY3547.The ellipsoidal height was determined by GPS observations

KY3547.and is referenced to NAD 83.

KY3547

KY3547.The dynamic height is computed by dividing the NAVD 88

KY3547.geopotential number by the normal gravity value computed on the

KY3547.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

KY3547.degrees latitude (g = 980.6199 gals.).

KY3547

KY3547.The modeled gravity was interpolated from observed gravity values.

KY3547

KY3547. The following values were computed from the NAD 83(2011) position.

KY3547

KY3547;                    North         East     Units Scale Factor Converg.

KY3547;SPC OH N     -    89,800.299   691,705.236   MT  0.99999314   +0 42 37.5

KY3547;SPC OH N     -   294,619.81  2,269,369.60   sFT  0.99999314   +0 42 37.5

KY3547;UTM  17      - 4,480,033.009   464,514.506   MT  0.99961550   -0 16 18.1

KY3547

KY3547!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY3547!SPC OH N     -   0.99996276  x   0.99999314  =   0.99995590

KY3547!UTM  17      -   0.99996276  x   0.99961550  =   0.99957827

KY3547

KY3547:                Primary Azimuth Mark                     Grid Az

KY3547:SPC OH N     -  PHILPORT AZ MK                           319 06 23.1

KY3547:UTM  17      -  PHILPORT AZ MK                           320 05 18.7

KY3547

KY3547_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME6451480033(NAD 83)

KY3547

KY3547|---------------------------------------------------------------------|

KY3547| PID    Reference Object                     Distance      Geod. Az  |

KY3547|                                                           dddmmss.s |

KY3547| AE1807 PHD A                               429.956 METERS 1383702.5 |

KY3547| KY3550 PHILPORT AZ MK                      APPROX. 0.7 KM 3194900.6 |

KY3547|---------------------------------------------------------------------|

KY3547

KY3547                          SUPERSEDED SURVEY CONTROL

KY3547

KY3547  NAD 83(2007)-  40 28 12.78265(N)    081 25 06.96801(W) AD(2002.00) 0

KY3547  ELLIP H (02/10/07)  237.438  (m)                       GP(2002.00)

KY3547  ELLIP H (09/23/04)  237.440  (m)                       GP(       ) 4 1

KY3547  NAD 83(1995)-  40 28 12.78254(N)    081 25 06.96798(W) AD(       ) B

KY3547  ELLIP H (08/20/96)  237.468  (m)                       GP(       ) 4 2

KY3547  NAD 83(1986)-  40 28 12.78644(N)    081 25 06.97207(W) AD(       ) 1

KY3547  NAD 83(1986)-  40 28 12.78644(N)    081 25 06.97207(W) AD(       ) 3

KY3547  NAD 27      -  40 28 12.55493(N)    081 25 07.52971(W) AD(       ) 3

KY3547  NAVD 88             270.88   (m)          888.7    (f) LEVELING    3

KY3547  NGVD 29 (04/22/14)  271.09   (m)          889.4    (f) RESET       3

KY3547  NGVD 29             271.09   (m)          889.4    (f) LEVELING    3

KY3547

KY3547.Superseded values are not recommended for survey control.

KY3547

KY3547.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY3547.See file dsdata.pdf to determine how the superseded data were derived.

KY3547

KY3547_MARKER: DH = HORIZONTAL CONTROL DISK

KY3547_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY3547_STAMPING: PHILPORT 1986

KY3547_MARK LOGO: NGS

KY3547_PROJECTION: PROJECTING 3 CENTIMETERS

KY3547_MAGNETIC: N = NO MAGNETIC MATERIAL

KY3547_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY3547+STABILITY: SURFACE MOTION

KY3547_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY3547+SATELLITE: SATELLITE OBSERVATIONS - March 18, 2012

KY3547

KY3547  HISTORY     - Date     Condition        Report By

KY3547  HISTORY     - 1986     MONUMENTED       NGS

KY3547  HISTORY     - 19871009 GOOD

KY3547  HISTORY     - 19940710 GOOD             HOBBS

KY3547  HISTORY     - 19950809 GOOD             NGS

KY3547  HISTORY     - 19980323 GOOD             GCS

KY3547  HISTORY     - 20030521 GOOD             NGS

KY3547  HISTORY     - 20120318 GOOD             GEOCAC

KY3547

KY3547                          STATION DESCRIPTION

KY3547

KY3547'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

KY3547'THE STATION IS LOCATED 2.5 MILES SOUTHEAST ALONG STATE ROUTE 259 FROM

KY3547'NEW PHILADELPHIA AT THE NEW PHILADELPHIA AIRPORT (CLEVER FIELD) JUST S

KY3547'SOUTH OF THE AIRPORT OFFICE IN A GRASSY AREA AT THE JUNCTION OF THE

KY3547'RUNWAY AND A TAXIWAY.

KY3547'THE STATION MARK IS A STANDARD NGS DISK STAMPED --PHILPORT 1986-- SET

KY3547'IN A 25 CM (10 INCH) DIAMETER CONCRETE POST 2 CM (1 INCH). IT IS

KY3547'ABOUT 0.05 MILE SOUTH OF THE OFFICE WHICH IS AT THE WEST CORNER OF

KY3547'A BRICK HANGER, 278 FEET WEST SOUTHWEST OF THE SOUTH CORNER OF THE

KY3547'BRICK HANGER, 24.1 METERS (79 FEET) NORTHEAST OF THE CENTERLINE OF

KY3547'THE RUNWAY, 20.1 METERS (66 FEET) NORTHWEST OF THE CENTERLINE OF THE T

KY3547'TAXIWAY AND 5.33 METERS (17.5 FEET) EAST-NORTHEAST OF A SQUARE TAXIWAY

KY3547'ENTRANCE SIGN.  THE UNDERGROUND MARK IS RECESSED 1.06 METERS (42 INCH

KY3547'TYPED BY JAMES MALONEY 9/08/87.

KY3547

KY3547                          STATION RECOVERY (1987)

KY3547

KY3547'RECOVERED 1987

KY3547'RECOVERED IN GOOD CONDITION.

KY3547

KY3547                          STATION RECOVERY (1994)

KY3547

KY3547'RECOVERY NOTE BY HOBBS AND ASSOCIATES 1994 (GBC)

KY3547'RECOVERED AS DESCRIBED.

KY3547

KY3547                          STATION RECOVERY (1995)

KY3547

KY3547'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY3547'NOTE--THIS IS THE PAC STATION.  THE STATION IS LOCATED ON THE

KY3547'SOUTHEAST SIDE OF NEW PHILADELPHIA AT HARRY CLEVER FIELD, IN THE

KY3547'SOUTHEAST END OF A GRASS ISLAND THAT IS ADJACENT TO THE APRON, AND AT

KY3547'MIDFIELD.  OWNERSHIP--CITY OF NEW PHILADELPHIA, 1834 EAST HIGH AVENUE,

KY3547'NEW PHILADELPHIA, OH.  44663.  AIRPORT MANAGER IS JOHN HAINES.  PHONE

KY3547'216-339-6078.  CONTACT HIM FOR PERMISSION TO OCCUPY THIS STATION.  TO

KY3547'REACH FROM THE OVERPASS AT THE JUNCTION OF U.S. HIGHWAY 250, BUSINESS

KY3547'HIGHWAY 250, AND STATE HIGHWAY 259 ABOUT 1.5 KM (0.95 MI) SOUTHEAST OF

KY3547'NEW PHILADELPHIA, GO WEST-NORTHWEST ON BUSINESS HIGHWAY 250 AND

KY3547'HIGHWAY 259 FOR 1.75 KM (1.10 MI) TO A ROAD LEFT JUST BEFORE A HANGAR

KY3547'AT THE SOUTHEAST END OF THE AIRFIELD.  TURN LEFT, SOUTHWESTERLY, ON

KY3547'THE ROAD FOR 0.12 KM (0.05 MI) TO THE PARALLEL TAXI.  TURN RIGHT,

KY3547'NORTHWEST, ALONG THE TAXI FOR 0.16 KM (0.10 MI) TO A CONNECTOR TAXI ON

KY3547'THE LEFT, THE APRON ON THE RIGHT, AND THE STATION ON THE LEFT.  THE

KY3547'STATION IS SET IN THE TOP OF A 25 CM ROUND CONCRETE POST SET FLUSH

KY3547'WITH THE GROUND.  IT IS 27.3 M (89.6 FT) SOUTHWEST OF THE EXTENDED

KY3547'CENTER OF THE PARALLEL TAXI, 23.9 M (78.4 FT) NORTHEAST OF THE CENTER

KY3547'OF THE RUNWAY, 19.2 M (63.0 FT) NORTHWEST OF THE CENTER OF THE

KY3547'CONNECTOR TAXI, 5.1 M (16.7 FT) EAST-NORTHEAST OF A LIGHT BOX WITH AN

KY3547'ARROW, AND 0.25 KM (0.15 MI) SOUTHEAST OF A WITNESS POST.  DESCRIBED

KY3547'BY D.G.  AUG

KY3547

KY3547                          STATION RECOVERY (1998)

KY3547

KY3547'RECOVERY NOTE BY GEODETIC CONSULTING SERVICES 1998 (KDZ)

KY3547'RECOVERED AS DESCRIBED.

KY3547

KY3547                          STATION RECOVERY (2003)

KY3547

KY3547'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 2003 (JMW)

KY3547'RECOVERED AS DESCRIBED.  AIRPORT MANAGER IS ERIC HUBBARD, PHONE

KY3547'330-339-6078.

KY3547

KY3547                          STATION RECOVERY (2012)

KY3547

KY3547'RECOVERY NOTE BY GEOCACHING 2012 (RLM)

KY3547'RECOVERED IN GOOD CONDITION.

KY2969  DESIGNATION -  R 312

KY2969  PID         -  KY2969

KY2969  STATE/COUNTY-  WV/BROOKE

KY2969  COUNTRY     -  US

KY2969  USGS QUAD   -  STEUBENVILLE EAST (2016)

KY2969

KY2969                         *CURRENT SURVEY CONTROL

KY2969  ______________________________________________________________________

KY2969* NAD 83(2011) POSITION- 40 16 55.05466(N) 080 36 30.34158(W)   ADJUSTED

KY2969* NAD 83(2011) ELLIP HT-   171.160 (meters)        (12/12/12)   ADJUSTED

KY2969* NAD 83(2011) EPOCH   -  2010.00

KY2969* NAVD 88 ORTHO HEIGHT -   205.220 (meters)      673.29  (feet) ADJUSTED

KY2969  ______________________________________________________________________

KY2969  GEOID HEIGHT    -        -34.053 (meters)                     GEOID18

KY2969  NAD 83(2011) X  -    795,123.884 (meters)                     COMP

KY2969  NAD 83(2011) Y  - -4,807,342.610 (meters)                     COMP

KY2969  NAD 83(2011) Z  -  4,102,029.977 (meters)                     COMP

KY2969  LAPLACE CORR    -         -0.47  (seconds)                    DEFLEC18

KY2969  DYNAMIC HEIGHT  -        205.111 (meters)      672.94  (feet) COMP

KY2969  MODELED GRAVITY -    980,094.4   (mgal)                       NAVD 88

KY2969

KY2969  VERT ORDER      -  FIRST     CLASS II

KY2969

KY2969  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY2969  Standards:

KY2969         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY2969            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY2969  -------------------------------------------------------------------

KY2969  NETWORK    1.12   5.31           0.52   0.37   2.71     -0.04978476

KY2969  -------------------------------------------------------------------

KY2969  Click here for local accuracies and other accuracy information.

KY2969

KY2969

KY2969.The horizontal coordinates were established by GPS observations

KY2969.and adjusted by the National Geodetic Survey in December 2012.

KY2969

KY2969.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY2969.been affixed to the stable North American tectonic plate. See

KY2969.NA2011 for more information.

KY2969

KY2969.The horizontal coordinates are valid at the epoch date displayed above

KY2969.which is a decimal equivalence of Year/Month/Day.

KY2969

KY2969.The orthometric height was determined by differential leveling and

KY2969.adjusted by the NATIONAL GEODETIC SURVEY

KY2969.in June 1991.

KY2969

KY2969.Significant digits in the geoid height do not necessarily reflect accuracy.

KY2969.GEOID18 height accuracy estimate available here.

KY2969

KY2969.Click photographs - Photos may exist for this station.

KY2969

KY2969.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY2969

KY2969.The Laplace correction was computed from DEFLEC18 derived deflections.

KY2969

KY2969.The ellipsoidal height was determined by GPS observations

KY2969.and is referenced to NAD 83.

KY2969

KY2969.The dynamic height is computed by dividing the NAVD 88

KY2969.geopotential number by the normal gravity value computed on the

KY2969.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

KY2969.degrees latitude (g = 980.6199 gals.).

KY2969

KY2969.The modeled gravity was interpolated from observed gravity values.

KY2969

KY2969. The following values were computed from the NAD 83(2011) position.

KY2969

KY2969;                    North         East     Units Scale Factor Converg.

KY2969;SPC WV N     -   198,417.808   505,739.305   MT  1.00000624   -0 42 24.9

KY2969;SPC WV N     -   650,975.76  1,659,246.37   sFT  1.00000624   -0 42 24.9

KY2969;UTM  17      - 4,459,126.285   533,286.611   MT  0.99961364   +0 15 11.4

KY2969

KY2969!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY2969!SPC WV N     -   0.99997315  x   1.00000624  =   0.99997939

KY2969!UTM  17      -   0.99997315  x   0.99961364  =   0.99958680

KY2969

KY2969_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TNE3328659126(NAD 83)

KY2969

KY2969                          SUPERSEDED SURVEY CONTROL

KY2969

KY2969  NAVD 88             205.22   (m)          673.3    (f) LEVELING    3

KY2969

KY2969.Superseded values are not recommended for survey control.

KY2969

KY2969.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY2969.See file dsdata.pdf to determine how the superseded data were derived.

KY2969

KY2969_MARKER: F = FLANGE-ENCASED ROD

KY2969_SETTING: 49 = STAINLESS STEEL ROD W/O SLEEVE (10 FT.+)

KY2969_STAMPING: R 312 1983

KY2969_MARK LOGO: NGS

KY2969_PROJECTION: FLUSH

KY2969_MAGNETIC: N = NO MAGNETIC MATERIAL

KY2969_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

KY2969_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY2969+SATELLITE: SATELLITE OBSERVATIONS - December 15, 2021

KY2969_ROD/PIPE-DEPTH: 12.0 meters

KY2969

KY2969  HISTORY     - Date     Condition        Report By

KY2969  HISTORY     - 1983     MONUMENTED       NGS

KY2969  HISTORY     - 20050621 GOOD             JCLS

KY2969  HISTORY     - 20090104 GOOD             GEOCAC

KY2969  HISTORY     - 20120309 GOOD             CANTER

KY2969  HISTORY     - 20131119 GOOD             GRWAS

KY2969  HISTORY     - 20180508 GOOD             GEOCAC

KY2969  HISTORY     - 20191010 GOOD             USPSQD

KY2969  HISTORY     - 20211215 GOOD             USPSQD

KY2969

KY2969                          STATION DESCRIPTION

KY2969

KY2969'DESCRIBED BY NATIONAL GEODETIC SURVEY 1983

KY2969'IN WELLSBURG.

KY2969'IN WELLSBURG, ALONG STATE HIGHWAY 2, AT THE JUNCTION OF 20TH STREET,

KY2969'ON PROPERTY OWNED BY THE PILLSBURY COMPANY, 12.68 METERS (41.6 FT)

KY2969'NORTHWEST OF THE CENTERLINE OF THE SOUTHBOUND LANE OF THE HIGHWAY,

KY2969'3.35 METERS (11.0 FT) EAST OF UTILITY POLE 424, 1.65 METERS (5.4 FT)

KY2969'SOUTH-SOUTHEAST OF AN IRON PIPE SUPPORTING A SIGN (THE PILLSBURY

KY2969'COMPANY).

KY2969'THE MARK IS 0.30 M ABOVE HIGHWAY.

KY2969

KY2969                          STATION RECOVERY (2005)

KY2969

KY2969'RECOVERY NOTE BY JOHN CHANCE LAND SURVEYS INC 2005 (FJO)

KY2969'RECOVERED IN GOOD CONDITION.

KY2969

KY2969                          STATION RECOVERY (2009)

KY2969

KY2969'RECOVERY NOTE BY GEOCACHING 2009 (RLM)

KY2969'THE MARK WAS RECOVERED IN GOOD CONDITION. ADD TO DESCRIPTION, THE IRON

KY2969'PIPE AND THE PILLSBURY SIGN HAVE BEEN REMOVED.

KY2969'

KY2969

KY2969                          STATION RECOVERY (2012)

KY2969

KY2969'RECOVERY NOTE BY CANTER SURVEYING GPS 2012 (CTS)

KY2969'RECOVERED AS DESCRIBED.

KY2969

KY2969                          STATION RECOVERY (2013)

KY2969

KY2969'RECOVERY NOTE BY GRW AERIAL SURVEY 2013 (TAT)

KY2969'RECOVERED IN GOOD CONDITION.

KY2969

KY2969                          STATION RECOVERY (2018)

KY2969

KY2969'RECOVERY NOTE BY GEOCACHING 2018 (MFM)

KY2969'RECOVERED IN GOOD CONDITION.

KY2969

KY2969                          STATION RECOVERY (2019)

KY2969

KY2969'RECOVERY NOTE BY US POWER SQUADRON 2019 (TJH)

KY2969'EAGLE FABRICATION NOW OCCUPIES THE PLANT.

KY2969

KY2969                          STATION RECOVERY (2021)

KY2969

KY2969'RECOVERY NOTE BY US POWER SQUADRON 2021 (STH)

KY2969'RECOVERED IN GOOD CONDITION.

HX2816  DESIGNATION -  RACINE

HX2816  PID         -  HX2816

HX2816  STATE/COUNTY-  WV/MASON

HX2816  COUNTRY     -  US

HX2816  USGS QUAD   -  NEW HAVEN (2016)

HX2816

HX2816                         *CURRENT SURVEY CONTROL

HX2816  ______________________________________________________________________

HX2816* NAD 83(1995) POSITION- 38 55 25.49822(N) 081 54 28.35768(W)   ADJUSTED

HX2816* NAVD 88 ORTHO HEIGHT -   182.087 (meters)      597.40  (feet) ADJUSTED

HX2816  ______________________________________________________________________

HX2816  GEOID HEIGHT    -        -34.034 (meters)                     GEOID18

HX2816  LAPLACE CORR    -         -0.99  (seconds)                    DEFLEC18

HX2816  DYNAMIC HEIGHT  -        181.972 (meters)      597.02  (feet) COMP

HX2816  MODELED GRAVITY -    979,993.3   (mgal)                       NAVD 88

HX2816

HX2816  HORZ ORDER      -  SECOND

HX2816  VERT ORDER      -  FIRST     CLASS II

HX2816

HX2816.The horizontal coordinates were established by classical geodetic methods

HX2816.and adjusted by the National Geodetic Survey in April 1998.

HX2816.

HX2816.The orthometric height was determined by differential leveling and

HX2816.adjusted by the NATIONAL GEODETIC SURVEY

HX2816.in June 1991.

HX2816

HX2816.Significant digits in the geoid height do not necessarily reflect accuracy.

HX2816.GEOID18 height accuracy estimate available here.

HX2816

HX2816.Click photographs - Photos may exist for this station.

HX2816

HX2816.The Laplace correction was computed from DEFLEC18 derived deflections.

HX2816

HX2816.The dynamic height is computed by dividing the NAVD 88

HX2816.geopotential number by the normal gravity value computed on the

HX2816.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

HX2816.degrees latitude (g = 980.6199 gals.).

HX2816

HX2816.The modeled gravity was interpolated from observed gravity values.

HX2816

HX2816. The following values were computed from the NAD 83(1995) position.

HX2816

HX2816;                    North         East     Units Scale Factor Converg.

HX2816;SPC WV S     -   213,909.376   521,270.088   MT  1.00000886   -0 33 40.5

HX2816;SPC WV S     -   701,801.01  1,710,200.28   sFT  1.00000886   -0 33 40.5

HX2816;UTM  17      - 4,308,706.803   421,300.318   MT  0.99967627   -0 34 13.6

HX2816

HX2816!             -  Elev Factor  x  Scale Factor =   Combined Factor

HX2816!SPC WV S     -   0.99997677  x   1.00000886  =   0.99998563

HX2816!UTM  17      -   0.99997677  x   0.99967627  =   0.99965305

HX2816

HX2816_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SMD2130008706(NAD 83)

HX2816

HX2816|---------------------------------------------------------------------|

HX2816| PID    Reference Object                     Distance      Geod. Az  |

HX2816|                                                           dddmmss.s |

HX2816| DC4148 RACINE RM 1                           7.695 METERS 10626     |

HX2816| DC4149 RACINE RM 2                           8.773 METERS 21250     |

HX2816| HX2815 RACINE AZ MK                        336.403 METERS 23924     |

HX2816|---------------------------------------------------------------------|

HX2816

HX2816                          SUPERSEDED SURVEY CONTROL

HX2816

HX2816  NAD 83(1993)-  38 55 25.50220(N)    081 54 28.36339(W) AD(       ) 2

HX2816  NAD 83(1986)-  38 55 25.50363(N)    081 54 28.36234(W) AD(       ) 2

HX2816  NAD 27      -  38 55 25.21513(N)    081 54 28.87275(W) AD(       ) 2

HX2816  NGVD 29 (07/19/86)  182.7    (m)          599.     (f) VERT ANG

HX2816

HX2816.Superseded values are not recommended for survey control.

HX2816

HX2816.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

HX2816.See file dsdata.pdf to determine how the superseded data were derived.

HX2816

HX2816_MARKER: DH = HORIZONTAL CONTROL DISK

HX2816_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

HX2816_STAMPING: RACINE 1975

HX2816_MARK LOGO: NGS

HX2816_PROJECTION: FLUSH

HX2816_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

HX2816+STABILITY: SURFACE MOTION

HX2816_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

HX2816+SATELLITE: SATELLITE OBSERVATIONS - March 15, 2019

HX2816

HX2816  HISTORY     - Date     Condition        Report By

HX2816  HISTORY     - 1975     MONUMENTED       NGS

HX2816  HISTORY     - 1984     GOOD             NGS

HX2816  HISTORY     - 20090910 GOOD             TEI

HX2816  HISTORY     - 20110723 GOOD             GEOCAC

HX2816  HISTORY     - 20190315 GOOD             DEWDAV

HX2816

HX2816                          STATION DESCRIPTION

HX2816

HX2816'DESCRIBED BY NATIONAL GEODETIC SURVEY 1975 (JRS)

HX2816'THE STATION IS ABOUT 4 MILES SOUTH OF RACINE, OHIO, 4 MILES

HX2816'NORTHWEST OF MT. ALTO, WEST VIRGINIA, 3 MILES NORTHEAST OF

HX2816'LETART, WEST VIRGINIA, ON THE WEST SIDE OF THE OHIO RIVER

HX2816'AND ON PROPERTY OF THE U.S. ARMY CORPS OF ENGINEERS NEAR THE

HX2816'RACINE LOCKS AND DAM.

HX2816'

HX2816'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 33 AND

HX2816'STATE HIGHWAY 2 IN MT. ALTO, WEST VIRGINIA, GO WESTERLY ON

HX2816'HIGHWAY 33 FOR 7.1 MILES TO A SIDE ROAD RIGHT AT THE RACINE

HX2816'LOCKS AND DAM.  TURN RIGHT AND GO EASTERLY FOR 0.1 MILE TO THE

HX2816'AZIMUTH MARK ON THE LEFT.  CONTINUE AHEAD FOR 0.05 MILE TO A

HX2816'FORK.  KEEP LEFT FORK FOR 0.05 MILE TO A FORK.  KEEP RIGHT

HX2816'FORK FOR 0.15 MILE TO A PARKING AREA JUST WEST OF THE RESTROOM

HX2816'AND THE STATION, ABOUT 150 FEET TO THE EAST.

HX2816'

HX2816'STATION MARKS ARE STANDARD DISKS STAMPED---RACINE 1975---.  THE

HX2816'SURFACE DISK IS SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE

HX2816'MONUMENT THAT IS FLUSH WITH THE GROUND.  IT IS 64 FEET NORTHEAST

HX2816'OF A METAL POLE WITH A LIGHT ATOP, 58 FEET SOUTH-SOUTHWEST OF

HX2816'THE SOUTHEAST END OF AN INFORMATION BOARD AND 8 FEET EAST OF THE

HX2816'EAST EDGE OF A SIDEWALK.  THE UNDERGROUND DISK IS SET IN AN

HX2816'IRREGULAR MASS OF CONCRETE 36 INCHES BELOW THE GROUND.

HX2816'

HX2816'REFERENCE MARK 1 IS A STANDARD DISK STAMPED---RACINE NO 1 1975---,

HX2816'SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT

HX2816'IS FLUSH WITH THE GROUND.  IT IS 32 FEET EAST OF THE EAST EDGE

HX2816'OF THE SIDEWALK, 7 FEET SOUTHEAST OF A SHRUB, 8 FEET NORTH

HX2816'OF A SHRUB AND ABOUT THE SAME ELEVATION OF THE STATION.

HX2816'

HX2816'REFERENCE MARK 2 IS A STANDARD DISK STAMPED---RACINE NO 2 1975---,

HX2816'SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE MONUMENT THAT

HX2816'IS FLUSH WITH THE GROUND.  IT IS 38.5 FEET NORTHEAST OF A METAL

HX2816'POLE WITH A LIGHT ATOP, 12 FEET EAST OF THE EAST EDGE OF THE

HX2816'SIDEWALK AND ABOUT THE SAME ELEVATION OF THE STATION.

HX2816'

HX2816'AZIMUTH MARK IS A STANDARD REFERENCE MARK DISK STAMPED---RACINE

HX2816'AZ 1975---, SET IN THE TOP OF A 12-INCH CYLINDRICAL CONCRETE

HX2816'MONUMENT THAT IS FLUSH WITH THE GROUND.  IT IS 45 FEET EAST OF

HX2816'THE NORTHEAST END OF A OVERPASS ABUTMENT, 22 FEET NORTH OF THE

HX2816'APPROXIMATE CENTER OF A PAVED ROAD, 2.5 FEET WEST-NORTHWEST

HX2816'OF A METAL WITNESS POST AND 2 FEET NORTH OF A METAL RAILING.

HX2816'

HX2816'HEIGHT OF LIGHT ABOVE MARK WAS 1.5 METERS.

HX2816

HX2816                          STATION RECOVERY (1984)

HX2816

HX2816'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1984

HX2816'10.8 KM (6.7 MI) SE FROM NEW HAVEN.

HX2816'10.3 KM (6.4 MI) SOUTHEAST ALONG U.S. HIGHWAY 33 FROM THE CITY

HX2816'MUNICIPAL BUILDING IN NEW HAVEN TO THE JUNCTION OF THE ENTRANCE ROAD

HX2816'OF THE RACINE LOCKS AND DAM, THENCE 0.2 KM (0.1 MI) EAST ALONG THE

HX2816'ROAD TO A FORK, THENCE 0.3 KM (0.2 MI) NORTHEAST ALONG A PAVED ROAD

HX2816'LEADING INTO A PARK ALONG THE OHIO RIVER, ABOUT 45.7 METERS (150.0 FT)

HX2816'EAST OF THE PARKING AREA JUST SOUTHWEST OF THE REST ROOMS FOR THE

HX2816'PARK, 21.18 METERS (69.5 FT) SOUTHWEST OF THE SOUTHWEST EDGE OF THE

HX2816'SIDEWALK LEADING INTO AN OVERLOOK AREA WITH A BULLETIN BOARD,

HX2816'17.52 METERS (57.5 FT) SOUTH-SOUTHWEST OF THE SOUTHEAST END OF A

HX2816'1.8 METER (6 FT) HIGH BRICK BULLETIN BOARD, 2.74 METERS (9.0 FT)

HX2816'SOUTHEAST OF THE SOUTHEAST EDGE OF THE SIDEWALK PARALLELING THE RIVER.

HX2816'THE MARK IS ABOVE LEVEL WITH SIDEWALK.

HX2816

HX2816                          STATION RECOVERY (2009)

HX2816

HX2816'RECOVERY NOTE BY TRIAD ENGINEERING INCORPORATED 2009 (SAC)

HX2816'RECOVERED IN GOOD CONDITION.

HX2816

HX2816                          STATION RECOVERY (2011)

HX2816

HX2816'RECOVERY NOTE BY GEOCACHING 2011 (DEB)

HX2816'THIS APPARENTLY IS THE SAME STATION AS HX3154, AS WITNESSED FROM THE

HX2816'DISK STAMPINGS AND THE DESCRIPTION REFERENCES.

HX2816'STATION RECOVERED IN GOOD CONDITON, REFERENCE MARKS NOT SEARCHED FOR.

KY3551  CBN         -  This is a Cooperative Base Network Control Station.

KY3551  PACS        -  This is a Primary Airport Control Station.

KY3551  DESIGNATION -  SEBPORT

KY3551  PID         -  KY3551

KY3551  STATE/COUNTY-  OH/MAHONING

KY3551  COUNTRY     -  US

KY3551  USGS QUAD   -  DAMASCUS (2016)

KY3551

KY3551                         *CURRENT SURVEY CONTROL

KY3551  ______________________________________________________________________

KY3551* NAD 83(2011) POSITION- 40 54 12.50697(N) 080 59 57.66943(W)   ADJUSTED

KY3551* NAD 83(2011) ELLIP HT-   326.357 (meters)        (06/27/12)   ADJUSTED

KY3551* NAD 83(2011) EPOCH   -  2010.00

KY3551* NAVD 88 ORTHO HEIGHT -   360.04  (meters)     1181.2   (feet) GPS OBS

KY3551  ______________________________________________________________________

KY3551  NAVD 88 orthometric height was determined with geoid model    GEOID93

KY3551  GEOID HEIGHT    -        -33.757 (meters)                     GEOID93

KY3551  GEOID HEIGHT    -        -33.668 (meters)                     GEOID18

KY3551  NAD 83(2011) X  -    755,298.118 (meters)                     COMP

KY3551  NAD 83(2011) Y  - -4,768,415.931 (meters)                     COMP

KY3551  NAD 83(2011) Z  -  4,154,540.879 (meters)                     COMP

KY3551  LAPLACE CORR    -         -2.01  (seconds)                    DEFLEC18

KY3551

KY3551  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY3551  Standards:

KY3551         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY3551            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY3551  -------------------------------------------------------------------

KY3551  NETWORK    1.24   2.49           0.57   0.42   1.27     -0.03813583

KY3551  -------------------------------------------------------------------

KY3551  Click here for local accuracies and other accuracy information.

KY3551

KY3551

KY3551.This mark is at Tri City Airport (3G6)

KY3551

KY3551.The horizontal coordinates were established by GPS observations

KY3551.and adjusted by the National Geodetic Survey in June 2012.

KY3551

KY3551.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY3551.been affixed to the stable North American tectonic plate. See

KY3551.NA2011 for more information.

KY3551

KY3551.The horizontal coordinates are valid at the epoch date displayed above

KY3551.which is a decimal equivalence of Year/Month/Day.

KY3551

KY3551.The orthometric height was determined by GPS observations and a

KY3551.high-resolution geoid model.

KY3551

KY3551.GPS derived orthometric heights for airport stations designated as

KY3551.PACS or SACS are published to 2 decimal places.  This maintains

KY3551.centimeter relative accuracy between the PACS and SACS.  It does

KY3551.not indicate centimeter accuracy relative to other marks which are

KY3551.part of the NAVD 88 network.

KY3551

KY3551.Significant digits in the geoid height do not necessarily reflect accuracy.

KY3551.GEOID18 height accuracy estimate available here.

KY3551

KY3551.Click photographs - Photos may exist for this station.

KY3551

KY3551.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY3551

KY3551.The Laplace correction was computed from DEFLEC18 derived deflections.

KY3551

KY3551.The ellipsoidal height was determined by GPS observations

KY3551.and is referenced to NAD 83.

KY3551

KY3551. The following values were computed from the NAD 83(2011) position.

KY3551

KY3551;                    North         East     Units Scale Factor Converg.

KY3551;SPC OH N     -   138,429.943   726,427.843   MT  0.99994323   +0 59 09.1

KY3551;SPC OH N     -   454,165.57  2,383,288.68   sFT  0.99994323   +0 59 09.1

KY3551;UTM  17      - 4,528,041.868   500,054.525   MT  0.99960000   +0 00 01.5

KY3551

KY3551!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY3551!SPC OH N     -   0.99994881  x   0.99994323  =   0.99989204

KY3551!UTM  17      -   0.99994881  x   0.99960000  =   0.99954883

KY3551

KY3551:                Primary Azimuth Mark                     Grid Az

KY3551:SPC OH N     -  SEBPORT AZ MK                            350 53 59.2

KY3551:UTM  17      -  SEBPORT AZ MK                            351 53 06.8

KY3551

KY3551_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TNF0005428041(NAD 83)

KY3551

KY3551|---------------------------------------------------------------------|

KY3551| PID    Reference Object                     Distance      Geod. Az  |

KY3551|                                                           dddmmss.s |

KY3551| KY3554 SEBPORT AZ MK                       APPROX. 0.7 KM 3515308.3 |

KY3551| AD9990 3G6 A                               APPROX. 0.6 KM 3571844.2 |

KY3551|---------------------------------------------------------------------|

KY3551

KY3551                          SUPERSEDED SURVEY CONTROL

KY3551

KY3551  NAD 83(2007)-  40 54 12.50715(N)    080 59 57.67016(W) AD(2002.00) 0

KY3551  ELLIP H (02/10/07)  326.372  (m)                       GP(2002.00)

KY3551  ELLIP H (03/08/05)  326.375  (m)                       GP(       ) 4 2

KY3551  NAD 83(1995)-  40 54 12.50697(N)    080 59 57.67012(W) AD(       ) B

KY3551  ELLIP H (08/20/96)  326.383  (m)                       GP(       ) 4 2

KY3551  NAD 83(1986)-  40 54 12.52164(N)    080 59 57.68241(W) AD(       ) 3

KY3551  NAD 27      -  40 54 12.29221(N)    080 59 58.30524(W) AD(       ) 3

KY3551  NGVD 29 (02/23/89)  360.06   (m)  RAPSU86 model used   GPS OBS

KY3551

KY3551.Superseded values are not recommended for survey control.

KY3551

KY3551.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY3551.See file dsdata.pdf to determine how the superseded data were derived.

KY3551

KY3551_MARKER: DH = HORIZONTAL CONTROL DISK

KY3551_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY3551_STAMPING: SEBPORT 1986

KY3551_MARK LOGO: NGS

KY3551_MAGNETIC: N = NO MAGNETIC MATERIAL

KY3551_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY3551+STABILITY: SURFACE MOTION

KY3551_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY3551+SATELLITE: SATELLITE OBSERVATIONS - April 29, 2009

KY3551

KY3551  HISTORY     - Date     Condition        Report By

KY3551  HISTORY     - 1986     MONUMENTED       NGS

KY3551  HISTORY     - 19871029 GOOD

KY3551  HISTORY     - 19950804 GOOD             NGS

KY3551  HISTORY     - 20090429 GOOD             GEOCAC

KY3551

KY3551                          STATION DESCRIPTION

KY3551

KY3551'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

KY3551'STATION IS LOCATED ABOUT 1.8 MILE SOUTHEAST OF SEBRING, 2.3 MILES

KY3551'WEST OF DAMASCUS, AT THE SEBRING TRI-CITY AIRPORT, IN A GRASS PLOT

KY3551'BORDERED BY THE RUNWAY AND A CIRCULAR RAMP, IN FRONT OF THE ADMINIS-

KY3551'TRATION BUILDING, OWNERSHIP TRI-CITY AIRPORT, C/O AIRPORT MANAGER

KY3551'C. M. COLLINS, 18294 HARRISON VESTIVILLE ROAD, BELOIT OH 04609

KY3551'PHONE 216-938-9388.

KY3551'TO REACH FROM THE JUNCTION OF US HIGHWAY 62 AND STATE HIGHWAYS 174

KY3551'AND 534 IN DAMASCUS, GO WEST ON HIGHWAYS 62 AND 173 FOR 2.0 MILES TO

KY3551'ITS JUNCTION WITH STATE HIGHWAY 165.  CONTINUE AHEAD FOR 0.25 MILES TO

KY3551'THE AIRPORT ENTRANCE ON THE RIGHT.  TURN RIGHT, NORTH, ON CURVING ROAD

KY3551'FOR 0.15 MILE TO A GRAVEL PARKING LOT ON THE LEFT.  TURN LEFT WEST

KY3551'ACROSS LOT FOR 0.05 MILE TO A GAP IN LOW WALL.  PASS THROUGH,

KY3551'THEN BEAR HALF RIGHT, NORTHWEST, FOR 50 METERS (ABOUT 175 FEET) TO

KY3551'THE STATION.  TO REACH AZIMUTH MARK FROM STATION, GO NORTH ALONG EAST

KY3551'SIDE OF RUNWAY FOR 0.4 MILE TO THE MARK NEAR THE NORTH END OF RUNWAY.

KY3551'STATION MARKS ARE STANDARD NGS STATION DISKS STAMPED --SEBPORT 1986--.

KY3551'THE SURFACE DISK IS SET IN THE TOP OF A 25 CM ROUND CONCRETE POST FLUS

KY3551'WITH THE GROUND.  IT IS 14.8 METERS EAST OF THE RUNWAY CENTER, 21.0

KY3551'METERS SOUTH OF THE CENTER OF RAMP, 30.6 METERS NORTH OF THE CENTER

KY3551'OF RAMP, 36.7 METERS WEST OF THE NORTHWEST CORNER OF ADMINISTRATION

KY3551'BUILDING, ON THE EXTENDED LINE OF THE NORTH WALL.  THE SUBSURFACE MARK

KY3551'IS SET IN THE TOP OF AN IRREGULAR CONCRETE MASS ABOUT 1.2 METERS

KY3551'BELOW GROUND.

KY3551'DESCRIBED BY G. R. HEID.  TYPED BY JAMES MALONEY 9/08/87.

KY3551

KY3551                          STATION RECOVERY (1987)

KY3551

KY3551'RECOVERED 1987

KY3551'RECOVERED IN GOOD CONDITION.

KY3551

KY3551                          STATION RECOVERY (1995)

KY3551

KY3551'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

KY3551'THE STATION IS LOCATED ABOUT 2.89 KM (1.80 MI) SOUTHEAST OF SEBRING,

KY3551'3.7 KM (2.30 MI) WEST OF DAMASCUS, AT THE TRI-CITY AIRPORT.  IN A

KY3551'GRASS AREA BORDERED BY THE RUNWAY AND A CIRCULAR RAMP, IN FRONT OF THE

KY3551'OLD ADMINISTRATION BUILDING.  OWNERSHIP--TRI-CITY AIRPORT INC. 18294

KY3551'HARRISBURGE WESTVILLE ROAD, BELOIT, OH.  44609.  AIRPORT MANAGER

KY3551'THOMAS M LANDIS, PHONE 330-938-1216.  NOTE--THIS STATION WAS SELECTED

KY3551'AS A (PACS) .  TO REACH THE STATION FROM THE JUNCTION OF US HIGHWAY 62

KY3551'AND STATE HIGHWAYS 165 AND 173 ABOUT 3.21 KM (2.00 MI) WEST OF

KY3551'DAMASCUS, GO WEST 0.40 KM (0.25 MI) ON HIGHWAY 62 TO THE AIRPORT

KY3551'ENTRANCE ON THE RIGHT.  TURN RIGHT, NORTH, 0.31 KM (0.20 MI) ON

KY3551'CURVING ROAD, PAST THE OLD OFFICE AND FIRST LONG HANGAR TO THE LEFT.

KY3551'TURN LEFT, WEST, BETWEEN HANGARS 0.08 KM (0.05 MI) TO THE OFFICE ON

KY3551'THE RIGHT.  TURN LEFT, SOUTH, 0.06 KM (0.05 MI) ACROSS 2 RAMPS AND

KY3551'GRASS AREA TO THE STATION ON THE RIGHT.  STATION IS 30.8 M (101.0 FT)

KY3551'NORTH OF THE SOUTH RAMP, 24.0 M (78.7 FT) WEST OF THE EAST RAMP, 20.5

KY3551'M (67.3 FT) SOUTH OF THE NORTH RAMP, 15.1 M (49.5 FT) EAST OF THE

KY3551'RUNWAY CENTER, 1.0 M (3.3 FT) NORTH OF A WITNESS POST, AND THE

KY3551'MONUMENT IS 0.1 M (0.3 FT) BELOW THE LEVEL OF THE RUNWAY AND FLUSH

KY3551'WITH THE GROUND SURFACE.

KY3551

KY3551                          STATION RECOVERY (2009)

KY3551

KY3551'RECOVERY NOTE BY GEOCACHING 2009 (RLM)

KY3551'RECOVERED IN GOOD CONDITION. ADD TO DESCRIPTION, THE AIRPORT PARKING

KY3551'LOT IS NOW PAVED AND THE LOW WALL HAS BEEN REMOVED.

JX2260  CBN         -  This is a Cooperative Base Network Control Station.

JX2260  PACS        -  This is a Primary Airport Control Station.

JX2260  DESIGNATION -  WOODPORT

JX2260  PID         -  JX2260

JX2260  STATE/COUNTY-  OH/MONROE

JX2260  COUNTRY     -  US

JX2260  USGS QUAD   -  WOODSFIELD (2016)

JX2260

JX2260                         *CURRENT SURVEY CONTROL

JX2260  ______________________________________________________________________

JX2260* NAD 83(2011) POSITION- 39 46 45.34677(N) 081 06 03.16825(W)   ADJUSTED

JX2260* NAD 83(2011) ELLIP HT-   333.288 (meters)        (06/27/12)   ADJUSTED

JX2260* NAD 83(2011) EPOCH   -  2010.00

JX2260* NAVD 88 ORTHO HEIGHT -   367.09  (meters)     1204.4   (feet) GPS OBS

JX2260  ______________________________________________________________________

JX2260  NAVD 88 orthometric height was determined with geoid model    GEOID93

JX2260  GEOID HEIGHT    -        -33.928 (meters)                     GEOID93

JX2260  GEOID HEIGHT    -        -33.724 (meters)                     GEOID18

JX2260  NAD 83(2011) X  -    759,349.537 (meters)                     COMP

JX2260  NAD 83(2011) Y  - -4,849,592.083 (meters)                     COMP

JX2260  NAD 83(2011) Z  -  4,059,393.548 (meters)                     COMP

JX2260  LAPLACE CORR    -          1.40  (seconds)                    DEFLEC18

JX2260

JX2260  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

JX2260  Standards:

JX2260         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

JX2260            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

JX2260  -------------------------------------------------------------------

JX2260  NETWORK    1.60   3.25           0.74   0.53   1.66     -0.00264626

JX2260  -------------------------------------------------------------------

JX2260  Click here for local accuracies and other accuracy information.

JX2260

JX2260

JX2260.This mark is at Monroe County Airport (4G5)

JX2260

JX2260.The horizontal coordinates were established by GPS observations

JX2260.and adjusted by the National Geodetic Survey in June 2012.

JX2260

JX2260.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

JX2260.been affixed to the stable North American tectonic plate. See

JX2260.NA2011 for more information.

JX2260

JX2260.The horizontal coordinates are valid at the epoch date displayed above

JX2260.which is a decimal equivalence of Year/Month/Day.

JX2260

JX2260.The orthometric height was determined by GPS observations and a

JX2260.high-resolution geoid model.

JX2260

JX2260.GPS derived orthometric heights for airport stations designated as

JX2260.PACS or SACS are published to 2 decimal places.  This maintains

JX2260.centimeter relative accuracy between the PACS and SACS.  It does

JX2260.not indicate centimeter accuracy relative to other marks which are

JX2260.part of the NAVD 88 network.

JX2260

JX2260.Significant digits in the geoid height do not necessarily reflect accuracy.

JX2260.GEOID18 height accuracy estimate available here.

JX2260

JX2260.Click photographs - Photos may exist for this station.

JX2260

JX2260.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JX2260

JX2260.The Laplace correction was computed from DEFLEC18 derived deflections.

JX2260

JX2260.The ellipsoidal height was determined by GPS observations

JX2260.and is referenced to NAD 83.

JX2260

JX2260. The following values were computed from the NAD 83(2011) position.

JX2260

JX2260;                    North         East     Units Scale Factor Converg.

JX2260;SPC OH S     -   198,453.061   719,850.429   MT  0.99995961   +0 53 16.0

JX2260;SPC OH S     -   651,091.42  2,361,709.28   sFT  0.99995961   +0 53 16.0

JX2260;UTM  17      - 4,403,262.904   491,361.244   MT  0.99960092   -0 03 52.4

JX2260

JX2260!             -  Elev Factor  x  Scale Factor =   Combined Factor

JX2260!SPC OH S     -   0.99994772  x   0.99995961  =   0.99990733

JX2260!UTM  17      -   0.99994772  x   0.99960092  =   0.99954866

JX2260

JX2260:                Primary Azimuth Mark                     Grid Az

JX2260:SPC OH S     -  WOODPORT AZ MK                           056 24 43.9

JX2260:UTM  17      -  WOODPORT AZ MK                           057 21 52.3

JX2260

JX2260_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SME9136103262(NAD 83)

JX2260

JX2260|---------------------------------------------------------------------|

JX2260| PID    Reference Object                     Distance      Geod. Az  |

JX2260|                                                           dddmmss.s |

JX2260| JX2263 WOODPORT AZ MK                      473.498 METERS 0571759.9 |

JX2260|---------------------------------------------------------------------|

JX2260

JX2260                          SUPERSEDED SURVEY CONTROL

JX2260

JX2260  NAD 83(2007)-  39 46 45.34696(N)    081 06 03.16912(W) AD(2002.00) 0

JX2260  ELLIP H (02/10/07)  333.306  (m)                       GP(2002.00)

JX2260  ELLIP H (03/08/05)  333.293  (m)                       GP(       ) 4 2

JX2260  NAD 83(1995)-  39 46 45.34712(N)    081 06 03.16887(W) AD(       ) B

JX2260  ELLIP H (08/20/96)  333.335  (m)                       GP(       ) 4 2

JX2260  NAD 83(1986)-  39 46 45.35515(N)    081 06 03.17311(W) AD(       ) 3

JX2260  NAD 27      -  39 46 45.10056(N)    081 06 03.80554(W) AD(       ) 3

JX2260  NGVD 29 (02/23/89)  367.20   (m)  RAPSU86 model used   GPS OBS

JX2260

JX2260.Superseded values are not recommended for survey control.

JX2260

JX2260.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JX2260.See file dsdata.pdf to determine how the superseded data were derived.

JX2260

JX2260_MARKER: DH = HORIZONTAL CONTROL DISK

JX2260_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JX2260_STAMPING: WOODPORT 1986

JX2260_MARK LOGO: NGS

JX2260_PROJECTION: FLUSH

JX2260_MAGNETIC: N = NO MAGNETIC MATERIAL

JX2260_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JX2260+STABILITY: SURFACE MOTION

JX2260_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JX2260+SATELLITE: SATELLITE OBSERVATIONS - July 29, 2020

JX2260

JX2260  HISTORY     - Date     Condition        Report By

JX2260  HISTORY     - 1986     MONUMENTED       NGS

JX2260  HISTORY     - 19870911 GOOD

JX2260  HISTORY     - 19950906 GOOD             NGS

JX2260  HISTORY     - 20150618 GOOD             AEROTE

JX2260  HISTORY     - 20200729 GOOD             GEOPRO

JX2260

JX2260                          STATION DESCRIPTION

JX2260

JX2260'DESCRIBED BY NATIONAL GEODETIC SURVEY 1986

JX2260'THE STATION IS LOCATED AT THE WOODFIELD MONROE COUNTY AIRPORT 1.0

JX2260'MILE NORTH-NORTHEAST OF WOODSFIELD AND BETWEEN THE RUNWAY AND ADMIN-

JX2260'ISTRATION BUILDING.

JX2260'STATION MARKS ARE STANDARD DISK STAMPED --WOODPORT 1986--. THE

JX2260'SURFACE MARK IS SET IN A 28 CM (11 INCH) DIAMETER FLUSH CONCRETE

JX2260'MONUMENT AND 36.9 METERS (121 FEET) WEST-NORTHWEST OF THE NORTHEAST

JX2260'CORNER OF THE ADMIN. BUILDING, 27.7 METERS (91 FEET) NORTHWEST OF THE

JX2260'NORTHWEST CORNER OF THE ADMINISTRATION BUILDING, 33.2 METERS (109 FEET

JX2260'SOUTH-SOUTHEAST OF THE SOUTH EDGE OF THE RUNWAY AND 30.8 METERS (101

JX2260'FEET) NORTH OF THE NORTHEAST CORNER OF A ONE STORY HOUSE.  THE UNDER-

JX2260'GROUND MARK IS SET 91 CM (36 INCHES) BELOW GROUND LEVEL.

JX2260'TYPED BY JAMES MALONEY 9/09/87.

JX2260

JX2260                          STATION RECOVERY (1987)

JX2260

JX2260'RECOVERED 1987

JX2260'RECOVERED IN GOOD CONDITION.

JX2260

JX2260                          STATION RECOVERY (1995)

JX2260

JX2260'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1995 (AJL)

JX2260'THE STATION IS LOCATED ABOUT 2.4 KM (1.50 MI) NORTH-NORTHEAST OF

JX2260'WOODSFIELD, AT THE MONROE COUNTY AIRPORT.  IN THE GRASS, NORTHWEST OF

JX2260'THE OFFICE AND ABOUT 1.0 M (3.3 FT) SOUTHEAST OF THE TOP OF A SLOPED

JX2260'BANK BEFORE THE RUNWAY.  OWNERSHIP--MONROE COUNTY AIRPORT AUTHORITY,

JX2260'47073 STATE ROUTE 26, WOODSFIELD, OH.  43793.  AIRPORT MANAGER HARRY

JX2260'SMITH, PHONE 614-472-1882.  TO REACH THE STATION FROM THE JUNCTION OF

JX2260'STATE HIGHWAYS 800 AND 26 ON THE NORTH SIDE OF WOODSFIELD, GO

JX2260'NORTHEASTERLY 2.44 KM (1.50 MI) ALONG HIGHWAY 26, PASSING THE FAIR

JX2260'GROUNDS, TO THE AIRPORT OFFICE ENTRANCE ON THE LEFT.  TURN LEFT,

JX2260'NORTHWEST, 0.07 KM (0.05 MI) ALONG DRIVEWAY, PAST OFFICE TO THE

JX2260'STATION ON THE LEFT, JUST BEFORE THE DOWNSLOP.  STATION IS 30.8 M

JX2260'(101.0 FT) NORTH-NORTHWEST OF THE NORTHEAST CORNER OF A HOUSE, 30.8 M

JX2260'(101.0 FT) NORTHEAST OF A SATELLITE DISH STAND, 27.7 M (90.9 FT)

JX2260'NORTHWEST OF THE NORTHWEST CORNER OF THE OFFICE, 1.0 M (3.3 FT)

JX2260'SOUTHEAST OF THE TOP OF A SLOPED BANK, 0.3 M (1.0 FT) SOUTHEAST OF A

JX2260'WITNESS POST, AND THE MONUMENT IS ABOUT 3.0 M (9.8 FT) ABOVE THE LEVEL

JX2260'OF THE RUNWAY AND FLUSH WITH THE GROUND SURFACE.  BY R.G. HAYES

JX2260

JX2260                          STATION RECOVERY (2015)

JX2260

JX2260'RECOVERY NOTE BY AEROTEC 2015 (CG)

JX2260'RECOVERED IN GOOD CONDITION.

JX2260

JX2260                          STATION RECOVERY (2020)

JX2260

JX2260'RECOVERY NOTE BY GEOPRO CONSULTANTS LLC 2020 (CTS)

JX2260'RECOVERED IN GOOD CONDITION.

KY2913  CBN         -  This is a Cooperative Base Network Control Station.

KY2913  DESIGNATION -  Y 337

KY2913  PID         -  KY2913

KY2913  STATE/COUNTY-  OH/STARK

KY2913  COUNTRY     -  US

KY2913  USGS QUAD   -  CANTON WEST (2016)

KY2913

KY2913                         *CURRENT SURVEY CONTROL

KY2913  ______________________________________________________________________

KY2913* NAD 83(2011) POSITION- 40 51 07.46150(N) 081 29 21.50257(W)   ADJUSTED

KY2913* NAD 83(2011) ELLIP HT-   295.260 (meters)        (06/27/12)   ADJUSTED

KY2913* NAD 83(2011) EPOCH   -  2010.00

KY2913* NAVD 88 ORTHO HEIGHT -   328.536 (meters)     1077.87  (feet) ADJUSTED

KY2913  ______________________________________________________________________

KY2913  GEOID HEIGHT    -        -33.177 (meters)                     GEOID18

KY2913  NAD 83(2011) X  -    715,044.236 (meters)                     COMP

KY2913  NAD 83(2011) Y  - -4,778,371.812 (meters)                     COMP

KY2913  NAD 83(2011) Z  -  4,150,204.291 (meters)                     COMP

KY2913  LAPLACE CORR    -         -1.60  (seconds)                    DEFLEC18

KY2913  DYNAMIC HEIGHT  -        328.387 (meters)     1077.38  (feet) COMP

KY2913  MODELED GRAVITY -    980,159.3   (mgal)                       NAVD 88

KY2913

KY2913  VERT ORDER      -  FIRST     CLASS II

KY2913

KY2913  Network accuracy estimates per FGDC Geospatial Positioning Accuracy

KY2913  Standards:

KY2913         FGDC (95% conf, cm)     Standard deviation (cm)     CorrNE

KY2913            Horiz  Ellip           SD_N   SD_E   SD_h      (unitless)

KY2913  -------------------------------------------------------------------

KY2913  NETWORK    0.57   1.61           0.26   0.20   0.82     -0.02447952

KY2913  -------------------------------------------------------------------

KY2913  Click here for local accuracies and other accuracy information.

KY2913

KY2913

KY2913.The horizontal coordinates were established by GPS observations

KY2913.and adjusted by the National Geodetic Survey in June 2012.

KY2913

KY2913.NAD 83(2011) refers to NAD 83 coordinates where the reference frame has

KY2913.been affixed to the stable North American tectonic plate. See

KY2913.NA2011 for more information.

KY2913

KY2913.The horizontal coordinates are valid at the epoch date displayed above

KY2913.which is a decimal equivalence of Year/Month/Day.

KY2913

KY2913.The orthometric height was determined by differential leveling and

KY2913.adjusted by the NATIONAL GEODETIC SURVEY

KY2913.in June 1991.

KY2913

KY2913.Significant digits in the geoid height do not necessarily reflect accuracy.

KY2913.GEOID18 height accuracy estimate available here.

KY2913

KY2913.Click photographs - Photos may exist for this station.

KY2913

KY2913.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY2913

KY2913.The Laplace correction was computed from DEFLEC18 derived deflections.

KY2913

KY2913.The ellipsoidal height was determined by GPS observations

KY2913.and is referenced to NAD 83.

KY2913

KY2913.The dynamic height is computed by dividing the NAVD 88

KY2913.geopotential number by the normal gravity value computed on the

KY2913.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

KY2913.degrees latitude (g = 980.6199 gals.).

KY2913

KY2913.The modeled gravity was interpolated from observed gravity values.

KY2913

KY2913. The following values were computed from the NAD 83(2011) position.

KY2913

KY2913;                    North         East     Units Scale Factor Converg.

KY2913;SPC OH N     -   132,128.117   685,218.243   MT  0.99994619   +0 39 50.3

KY2913;SPC OH N     -   433,490.33  2,248,086.85   sFT  0.99994619   +0 39 50.3

KY2913;UTM  17      - 4,522,451.129   458,756.653   MT  0.99962094   -0 19 12.2

KY2913

KY2913!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY2913!SPC OH N     -   0.99995369  x   0.99994619  =   0.99989988

KY2913!UTM  17      -   0.99995369  x   0.99962094  =   0.99957464

KY2913

KY2913_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TMF5875622451(NAD 83)

KY2913

KY2913                          SUPERSEDED SURVEY CONTROL

KY2913

KY2913  NAD 83(2007)-  40 51 07.46165(N)    081 29 21.50333(W) AD(2002.00) 0

KY2913  ELLIP H (02/10/07)  295.273  (m)                       GP(2002.00)

KY2913  NAD 83(1995)-  40 51 07.46166(N)    081 29 21.50337(W) AD(       ) A

KY2913  ELLIP H (09/23/04)  295.274  (m)                       GP(       ) 4 1

KY2913  NAVD 88 (09/23/04)  328.4    (m)  GEOID03 model used   GPS OBS

KY2913  NGVD 29 (06/03/92)  328.727  (m)         1078.50   (f) ADJUSTED    1 2

KY2913

KY2913.Superseded values are not recommended for survey control.

KY2913

KY2913.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY2913.See file dsdata.pdf to determine how the superseded data were derived.

KY2913

KY2913_MARKER: F = FLANGE-ENCASED ROD

KY2913_SETTING: 49 = STAINLESS STEEL ROD W/O SLEEVE (10 FT.+)

KY2913_STAMPING: Y 337 1983

KY2913_MARK LOGO: NGS

KY2913_PROJECTION: FLUSH

KY2913_MAGNETIC: N = NO MAGNETIC MATERIAL

KY2913_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

KY2913_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY2913+SATELLITE: SATELLITE OBSERVATIONS - November 29, 2019

KY2913_ROD/PIPE-DEPTH: 9.4  meters

KY2913

KY2913  HISTORY     - Date     Condition        Report By

KY2913  HISTORY     - 1983     MONUMENTED       NGS

KY2913  HISTORY     - 19930422 GOOD             OHDT

KY2913  HISTORY     - 2003     GOOD             OH-151

KY2913  HISTORY     - 20091227 GOOD             GEOCAC

KY2913  HISTORY     - 20161112 GOOD             USPSQD

KY2913  HISTORY     - 20181114 GOOD             USPSQD

KY2913  HISTORY     - 20191129 GOOD             USPSQD

KY2913

KY2913                          STATION DESCRIPTION

KY2913

KY2913'DESCRIBED BY NATIONAL GEODETIC SURVEY 1983

KY2913'6.5 KM (4.05 MI) NORTH FROM MASSILLON.

KY2913'6.5 KM (4.05 MI) NORTH ALONG STATE HIGHWAY 241 FROM THE JUNCTION OF

KY2913'STATE HIGHWAY 172 (LINCOLN WAY) IN MASSILLON TO THE MARK ON THE LEFT,

KY2913'ON THE EAST SIDE OF THE SHADY HOLLOW COUNTRY CLUB SIGN, ALSO 9.9 KM

KY2913'(6.2 MI) SOUTH ALONG STATE HIGHWAY 241 FROM THE POST OFFICE IN

KY2913'GREENSBURG, 10.21 METERS (33.5 FT) WEST OF THE CENTERLINE OF THE

KY2913'HIGHWAY, 3.35 METERS (11.0 FT) SOUTHEAST OF THE NORTH FLOOD LIGHT FOR

KY2913'THE SIGN, 0.30 METER (1.0 FT) EAST OF THE EAST END OF THE SIGN.

KY2913'THE MARK IS 0.91 M ABOVE HIGHWAY.

KY2913

KY2913                          STATION RECOVERY (1993)

KY2913

KY2913'RECOVERY NOTE BY OHIO DEPARTMENT OF TRANSPORTATION 1993 (BM)

KY2913'----REPORT BY ODOT ------YEAR 1993-------CONDITION - GOOD--------

KY2913

KY2913                          STATION RECOVERY (2003)

KY2913

KY2913'RECOVERY NOTE BY STARK COUNTY OHIO 2003 (BW)

KY2913'RECOVERED AS DESCRIBED.

KY2913

KY2913                          STATION RECOVERY (2009)

KY2913

KY2913'RECOVERY NOTE BY GEOCACHING 2009 (RLM)

KY2913'RECOVERED IN GOOD CONDITION.

KY2913

KY2913                          STATION RECOVERY (2016)

KY2913

KY2913'RECOVERY NOTE BY US POWER SQUADRON 2016 (JTH)

KY2913'RECOVERED IN GOOD CONDITION.

KY2913

KY2913                          STATION RECOVERY (2018)

KY2913

KY2913'RECOVERY NOTE BY US POWER SQUADRON 2018 (TJH)

KY2913'RECOVERED IN GOOD CONDITION.

KY2913

KY2913                          STATION RECOVERY (2019)

KY2913

KY2913'RECOVERY NOTE BY US POWER SQUADRON 2019 (TJH)

KY2913'RECOVERED IN GOOD CONDITION.

KY0956  DESIGNATION -  YODER

KY0956  PID         -  KY0956

KY0956  STATE/COUNTY-  OH/HOLMES

KY0956  COUNTRY     -  US

KY0956  USGS QUAD   -  HOLMESVILLE (2016)

KY0956

KY0956                         *CURRENT SURVEY CONTROL

KY0956  ______________________________________________________________________

KY0956* NAD 83(1995) POSITION- 40 37 41.09123(N) 081 59 23.53227(W)   ADJUSTED

KY0956* NAD 83(1995) ELLIP HT-   332.049 (meters)        (04/01/98)   ADJUSTED

KY0956* NAVD 88 ORTHO HEIGHT -   365.101 (meters)     1197.84  (feet) ADJUSTED

KY0956  ______________________________________________________________________

KY0956  GEOID HEIGHT    -        -33.157 (meters)                     GEOID18

KY0956  NAD 83(1995) X  -    675,537.402 (meters)                     COMP

KY0956  NAD 83(1995) Y  - -4,800,539.856 (meters)                     COMP

KY0956  NAD 83(1995) Z  -  4,131,380.875 (meters)                     COMP

KY0956  LAPLACE CORR    -          1.16  (seconds)                    DEFLEC18

KY0956  DYNAMIC HEIGHT  -        364.930 (meters)     1197.27  (feet) COMP

KY0956  MODELED GRAVITY -    980,144.5   (mgal)                       NAVD 88

KY0956

KY0956  HORZ ORDER      -  SECOND

KY0956  VERT ORDER      -  SECOND    CLASS 0

KY0956  ELLP ORDER      -  FOURTH    CLASS I

KY0956

KY0956.The horizontal coordinates were established by classical geodetic methods

KY0956.and adjusted by the National Geodetic Survey in April 1998.

KY0956.

KY0956.The orthometric height was determined by differential leveling and

KY0956.adjusted by the NATIONAL GEODETIC SURVEY

KY0956.in June 1991.

KY0956

KY0956.Significant digits in the geoid height do not necessarily reflect accuracy.

KY0956.GEOID18 height accuracy estimate available here.

KY0956

KY0956.Click photographs - Photos may exist for this station.

KY0956

KY0956.The X, Y, and Z were computed from the position and the ellipsoidal ht.

KY0956

KY0956.The Laplace correction was computed from DEFLEC18 derived deflections.

KY0956

KY0956.The ellipsoidal height was determined by GPS observations

KY0956.and is referenced to NAD 83.

KY0956

KY0956.The dynamic height is computed by dividing the NAVD 88

KY0956.geopotential number by the normal gravity value computed on the

KY0956.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

KY0956.degrees latitude (g = 980.6199 gals.).

KY0956

KY0956.The modeled gravity was interpolated from observed gravity values.

KY0956

KY0956. The following values were computed from the NAD 83(1995) position.

KY0956

KY0956;                    North         East     Units Scale Factor Converg.

KY0956;SPC OH N     -   106,887.614   643,158.614   MT  0.99996840   +0 20 06.5

KY0956;SPC OH N     -   350,680.45  2,110,096.22   sFT  0.99996840   +0 20 06.5

KY0956;UTM  17      - 4,497,942.759   416,283.576   MT  0.99968626   -0 38 40.5

KY0956

KY0956!             -  Elev Factor  x  Scale Factor =   Combined Factor

KY0956!SPC OH N     -   0.99994792  x   0.99996840  =   0.99991632

KY0956!UTM  17      -   0.99994792  x   0.99968626  =   0.99963419

KY0956

KY0956:                Primary Azimuth Mark                     Grid Az

KY0956:SPC OH N     -  SHREVE                                   331 06 03.9

KY0956:UTM  17      -  SHREVE                                   332 04 50.9

KY0956

KY0956_U.S. NATIONAL GRID SPATIAL ADDRESS: 17TME1628397942(NAD 83)

KY0956

KY0956|---------------------------------------------------------------------|

KY0956| PID    Reference Object                     Distance      Geod. Az  |

KY0956|                                                           dddmmss.s |

KY0956| KY0955 YODER RM 2                            7.815 METERS 10820     |

KY0956| KY0957 YODER RM 1                            6.296 METERS 27145     |

KY0956| KZ1760 SHREVE                              APPROX. 9.7 KM 3312610.4 |

KY0956|---------------------------------------------------------------------|

KY0956

KY0956                          SUPERSEDED SURVEY CONTROL

KY0956

KY0956  NAD 83(1986)-  40 37 41.09574(N)    081 59 23.53370(W) AD(       ) 2

KY0956  NAD 27      -  40 37 40.88661(N)    081 59 23.99813(W) AD(       ) 2

KY0956  NGVD 29 (??/??/92)  365.310  (m)         1198.52   (f) ADJ UNCH    2 0

KY0956  NGVD 29             365.31   (m)         1198.5    (f) LEVELING    3

KY0956

KY0956.Superseded values are not recommended for survey control.

KY0956

KY0956.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

KY0956.See file dsdata.pdf to determine how the superseded data were derived.

KY0956

KY0956_MARKER: DS = TRIANGULATION STATION DISK

KY0956_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

KY0956_STAMPING: YODER 1959

KY0956_MARK LOGO: CGS

KY0956_MAGNETIC: N = NO MAGNETIC MATERIAL

KY0956_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

KY0956+STABILITY: SURFACE MOTION

KY0956_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

KY0956+SATELLITE: SATELLITE OBSERVATIONS - December 18, 2011

KY0956

KY0956  HISTORY     - Date     Condition        Report By

KY0956  HISTORY     - 1959     MONUMENTED       CGS

KY0956  HISTORY     - 1959     SEE DESCRIPTION  CGS

KY0956  HISTORY     - 1958     GOOD             CGS

KY0956  HISTORY     - 1959     GOOD             CGS

KY0956  HISTORY     - 1986     GOOD             NGS

KY0956  HISTORY     - 19871014 GOOD

KY0956  HISTORY     - 20111218 GOOD             GEOCAC

KY0956

KY0956                          STATION DESCRIPTION

KY0956

KY0956'DESCRIBED BY COAST AND GEODETIC SURVEY 1958

KY0956'3.8 MI NW FROM HOLMESVILLE.

KY0956'ABOUT 0.8 MILE WEST ALONG COUNTY ROAD 320 FROM THE CHURCH OF

KY0956'CHRIST AT HOLMESVILLE, THENCE ABOUT 3.05 MILES NORTHWEST ALONG

KY0956'COUNTY ROAD 329, 0.25 MILE WEST OF THE INTERSECTION OF COUNTY

KY0956'ROAD 331, ON THE WEST SLOPE OF A HILL, IN THE SOUTH EDGE OF A

KY0956'CULTIVATED FIELD, 24 1/2 FEET NORTH OF THE CENTER LINE OF THE

KY0956'ROAD, 4 1/2 FEET SOUTH OF THE SECOND POWER LINE POLE WEST OF

KY0956'THE TOP OF HILL, 5 FEET NORTH OF A EAST-WEST FENCE LINE,

KY0956'4 1/2 FEET SOUTH OF A STEEL WITNESS POST, ABOUT 2 1/2 FEET ABOVE

KY0956'THE LEVEL OF THE ROAD, AND SET IN TOP OF A CONCRETE POST

KY0956'PROJECTING 3 INCHES.

KY0956

KY0956                          STATION RECOVERY (1959)

KY0956

KY0956'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1959 (WER)

KY0956'THE STATION IS ABOUT 4 MILES SOUTH-SOUTHEAST OF SHREVE, 3-3/4 MILES

KY0956'WEST OF HOLMESVILLE, IN PRAIRIE TOWNSHIP, IN THE SOUTHWEST QUARTER OF

KY0956'SECTION 6, AND ON LAND OWNED BY MR. ANDY M. YODER.

KY0956'

KY0956'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAY 226 AND WAYNE

KY0956'COUNTY ROUTE 157 IN SHREVE, GO SOUTH ON COUNTY ROUTE 157 FOR 0.8 MILE

KY0956'TO A T-ROAD JUNCTION WITH COUNTY ROUTE NO. 1.  TURN LEFT AND FOLLOW

KY0956'ROUTE NO. 1 FOR 0.2 MILE TO HOLMES COUNTY ROAD NO. 318.  TURN RIGHT

KY0956'AND FOLLOW ROAD NO. 318 FOR 1.0 MILE TO THE JUNCTION OF HOLMES COUNTY

KY0956'ROAD 339.  BEAR LEFT AND FOLLOW ROAD NO. 339 FOR 1.4 MILE TO A T-ROAD

KY0956'JUNCTION OF A BLACKTOP ROAD.  TURN LEFT AND GO EASTERLY 1.0 MILE TO

KY0956'THE MOOREHEAD CHURCH OF NAZARENE ON THE LEFT.  CONTINUE STRAIGHT AHEAD

KY0956'FOR 0.4 MILE TO THE STATION ON THE LEFT.

KY0956'

KY0956'STATION MARKS ARE STANDARD DISKS STAMPED YODER 1959.  THE SURFACE DISK

KY0956'IS SET IN A SQUARE CONCRETE POST WHICH PROJECTS 3 INCHES.  IT IS 26

KY0956'FEET NORTH OF THE CENTERLINE OF THE BLACKTOP ROAD, 4 FEET SOUTH OF THE

KY0956'FOURTH POWERLINE POLE WHICH IS WEST OF THE DRIVEWAY LEADING TO THE

KY0956'HOME OF MR. ANDY YODER, 4-1/2 FEET NORTH OF THE RIGHT-OF-WAY FENCELINE

KY0956'AND 4.2 FEET SOUTH OF A STANDARD METAL WITNESS POST.  THE UNDERGROUND

KY0956'DISK IS SET IN AN IRREGULAR MASS OF CONCRETE 27 INCHES BELOW THE

KY0956'SURFACE OF THE GROUND.

KY0956'

KY0956'REFERENCE MARK NO. 1, A STANDARD DISK STAMPED YODER NO 1 1959, IS SET

KY0956'IN A SQUARE CONCRETE POST WHICH PROJECTS 3 INCHES.  IT IS 23 FEET

KY0956'NORTH OF THE CENTERLINE OF THE BLACKTOP ROAD, 20 FEET WEST OF THE

KY0956'FOURTH POWERLINE POLE WHICH IS WEST OF MR. YODERS DRIVEWAY, AND 1 FOOT

KY0956'NORTH OF THE RIGHT-OF-WAY FENCELINE.

KY0956'

KY0956'REFERENCE MARK NO. 2, A STANDARD DISK STAMPED YODER NO 2 1959, IS SET

KY0956'IN A SQUARE CONCRETE POST WHICH PROJECTS 5 INCHES.  IT IS 23 FEET

KY0956'NORTH OF THE CENTERLINE OF THE BLACKTOP ROAD, 27 FEET EAST OF THE

KY0956'FOURTH POWERLINE POLE WHICH IS WEST OF MR. YODERS DRIVEWAY, AND 1 FOOT

KY0956'NORTH OF THE RIGHT-OF-WAY FENCELINE.

KY0956'

KY0956'NO AZIMUTH MARK WAS SET.

KY0956

KY0956                          STATION RECOVERY (1986)

KY0956

KY0956'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1986

KY0956'STATION MARK WAS RECOVERED IN GOOD CONDITION.  OTHER MARKS NOT

KY0956'SEARCHED FOR.  STATION IS ON GRASSY RIGHT OF WAY ALONG FIELD EDGE.

KY0956'OWNERSHIP OHIO DEPARTMENT OF TRANSPORTATION.

KY0956'TO REACH FORM THE JUNCTION OF STATE HIGHWAY 83 AND COUNTY ROUTE 320 AT

KY0956'THE CHURCH OF CHRIST IN HOLMESVILLE, GO WEST ON ROUTE 320 FOR 0.85

KY0956'MILE TO A T-ROAD JUNCTION JUST PAST A GIRDER BRIDGE.  TURN RIGHT,

KY0956'NORTH, THEN WEST, ON COUNTY ROUTE 329 FOR 2.8 MILES TO A CROSS ROAD

KY0956'(COUNTY ROUTE 551). CONTINUE AHEAD FOR 0.2 MILES TO TOP OF GRADE

KY0956'AND NORTHWEST CORNER OF CEMETERY ON THE LEFT.  CONTINUE AHEAD FOR

KY0956'58 METERS ( ABOUT 200 FEET) TO THE STATION ON THE RIGHT.

KY0956'STATION MARK IS A STANDARD CGS STATION DISK STAMPED --YODER 1958--

KY0956'SET IN THE TOP OF A 30 CM SQUARE CONCRETE POST FLUSH WITH THE GROUND.

KY0956'IT IS 7.6 METERS NORTH OF , AND 0.3 METER HIGHER THAN THE ROAD

KY0956'CENTER, 1.3 METERS SOUTH OF A UTILITY POLE, AND 1.3 METER SOUTH

KY0956'OF A METAL WITNESS POST.

KY0956'DESCRIBED BY G R HEID.  TYPED BY JAMES MALONEY 9/10/87.

KY0956

KY0956                          STATION RECOVERY (1987)

KY0956

KY0956'RECOVERED 1987

KY0956'RECOVERED IN GOOD CONDITION.

KY0956

KY0956                          STATION RECOVERY (2011)

KY0956

KY0956'RECOVERY NOTE BY GEOCACHING 2011 (RLM)

KY0956'THE STATION MARK AND REFERENCE MARK 1 WERE RECOVERED IN GOOD

KY0956'CONDITION. REFERENCE MARK 2 WAS RECOVERED IN POOR CONDITION. THE DISK

KY0956'IS MISSING, BUT THE STEM IS INTACT.