Texas statewide bare earth Lidar data was received from the contractor in ASCII X,Y,Z format, UTM projection, NAD83-NAVD88 datum, meter horizontal & vertical units.  Project ASCII files were converted to LAS format files with Global Mapper software.  LAS file point density supported 1-meter griding.  One meter GM grid format files in 1500 -meter tiles were generated from the LAS data for Quality Assurance review and vertical accuracy testing.   The GM grid tiles were reformatted and resampled to Arc Float  .00003086420 arc-degree grids, quarter-quads).