NRCS Logan County, Illinois

4ppsm

LiDAR Report

Prepared For:

United States Geological Survey

National Geospatial Technical Operations Center

1400 Independence Road

Rolla, Missouri 65401

Prepared By:

Quantum Spatial

4020 Technology Parkway

Sheboygan, WI 53083

P: 920.457.3631

F:920.457.0410

Contract ID: G10PC00025

Task Order: G13PD00885

Quantum Spatial Project No: 1130905

Table of Contents

United States Geological Survey

NRCS Logan County, Illinois LiDAR

TITLE SECTION Introduction 1 Geodetic Control 2 LiDAR Acquisition and Procedures 3 Quality Control Surveys 4 LiDAR Processing 5 Conclusion 6 Flight Logs 7 LiDAR GPS Processing Plots 8 QA/QC Output Control Report 9 Ground Control Survey Reports 10

1. Introduction

This report contains a summary of the Light Detection and Ranging (LiDAR) data acquisition and processing for the project area to include Logan County, Illinois. The United States Geological Survey (USGS) requires the LiDAR data to aid in analysis of land use, hydrologic, vegetation and recreational management.

1.1 Contact Info

Questions regarding the technical aspects of this report should be addressed to:

Quantum Spatial 
4020 Technology Parkway 
Sheboygan, WI 53083

Attention: Chris Guy (LiDAR Manager) 
Phone: 920.457.3631

Email: cguy@quantumspatial.com

1.2 Purpose

Quantum Spatial acquired high accuracy LiDAR data of Logan County, Illinois for the USGS in accordance with requirements specified to produce such a dataset as outlined in contract ID G10PC00025 and as defined by United States Geological Survey National Geospatial Program Base LiDAR Specification, Version 1.0 (ILMF).

1.3 Project Locations

This project consisted of Logan County, Illinois.   The area of acquisition is approximately 618 square miles (1600 square kilometers) located in central Illinois.  Image 1.3a on the following page shows the relative location of this area.

1.4 Time Period

LiDAR data acquisition for complete coverage of the project was acquired between November 23, 2013 and November 30, 2013.  See Image 3.3a and 3.3b for the project swaths collected.  Flight logs can be found in Section 7 of this document.

Project data includes eight (8) flight missions totaling one hundred eleven (111) flight lines.

1.5 Project Scope

Data collection was accomplished by the staff of Quantum Spatial.  Multiple flights were required to collect LiDAR data coverage of the entire county of Logan.

As documented in the Task Order, collected data was to achieve a Fundamental Vertical Accuracy (FVA) of 18.13cm (0.59 ft) at a 95% confidence level, from an RSME of 9.25cm (0.30 ft) in the open terrain land cover category based on a Triangulated Integrated Network (TIN) of the LiDAR points. And to achieve the same values from Digital Elevation Models (DEM) derived from LiDAR data.

Consolidated Vertical Accuracy (CVA) is to achieve 26.9 cm (0.88 ft) at 95th Percentile based on the DEM.

Supplemental Vertical Accuracy (SVA) has a target for each of the ground cover category of 26.9 cm (0.88ft) at 95th Percentile.

Ground cover categories are Bare Earth/Open Terrain, Urban, Tall Weeds, Brush, and Forest.

Section 5.6 contains the vertical accuracy assessment result values.

Image 1.3a:  The area outlined in black shows the location of Logan County, Illinois.

2. Geodetic Control

The field survey report and an accuracy report are included with the project delivery.  See “Final_Delivery_Report” and “Accuracy_Report” pdfs attached.

Ground survey was performed by Compass Data, Inc. between December 1, 2013 and December 12, 2013.  Check points were collected in various land cover categories for data calibration and vertical assessment analysis. Ground control check points were compared to airborne data. The comparison of vertical differences provides for calculation of vertical accuracies in the various land cover categories.

3.  LiDAR Acquisition and Procedures

3.1 Acquisition Time Period

LiDAR data acquisition and Airborne GPS control were completed between November 23, 2013 and November 30, 2013. Data from the eight (8) flight missions is included in the project.

3.2 LiDAR Planning

The LiDAR data for this project was collected with aircraft operated by Quantum Spatial. The aircraft is equipped with LiDAR sensor systems as well as systems to collect GPS and IMU positioning data during flight. All flight planning was completed using Leica MissionPro software and data collection was completed using a Leica ALS70 sensor.

3.3 LiDAR Acquisition

Data acquired from eight (8) flight missions was utilized to provide project area coverage. Refer to Table 3.3a and 3.3b for acquisition parameters. Acquired swaths can be seen in Image 3.3a and 3.3b on the following page. Section 7 contains the flight logs.

A Leica sensor used on board a Piper Navajo Twin aircraft. Airborne GPS and IMU position and trajectory data of the LiDAR sensor were also acquired during the time of flight.

Before take-off, the LiDAR system and the Airborne GPS and IMU system were initialized for a period of five minutes and in operation after landing for another five minutes. The missions acquired data according to the planned flight lines and included a minimum of one (usually two) cross flights. The cross flights were flown perpendicular to the planned flight lines and their data used in the in-situ calibration of the sensor.

| Table 3.3a: Acquisition Parameters 4ppsm |  |
| --- | --- |
| Sensor Type | Leica ALS - 70 |
| Sensor ID | SN7220 |
| Field of View | 40° |
| Flying Height (Above mean sea level) | 1433 - 1518m |
| Pulse Rate Frequency | 358 kHz |
| Mirror Scan Rate Frequency | 53.4 Hz |
| Scan Angle (degrees) | 20° |
| Ground Speed | 150 kts |
| Average Point Density | 4.2 pts/m2 |
| Minimum Overlap | 11.84% |

| Table 3.3b: Acquisition Parameters 8ppsm |  |
| --- | --- |
| Sensor Type | Leica ALS - 70 |
| Sensor ID | NS7220 |
| Field of View | 36° |
| Flying Height (Above mean sea level) | 751 - 800m |
| Pulse Rate Frequency | 335.4 kHz |
| Mirror Scan Rate Frequency | 56 Hz |
| Scan Angle (degrees) | 18° |
| Ground Speed | 150 kts |
| Average Point Density | 8.36 pts/m2 |
| Minimum Overlap | 19.28% |

Image 3.3a:   Swaths for 4ppsm data, colored by mission.  Mission 20131124-135651 is orange, 20131127-144235 is blue, 20131127-213804 is yellow, 20131128-135545 is red, and 20131130-155001 is green.

Image 3.3b:   Swaths for 8ppsm data. colored by mission.  Mission 20131123-223305 is orange, 20131124-191118 is purple, and 20131126-144704 is green.

3.4 LiDAR Trajectory Processing

Missions were processed using a Continuously Operating Reference Station (CORS) station and at least one base station.  The image below illustrates locations of these sites relative to the project area.

Image 3.4a:  Relative location of CORS station HDIL and base station used in flight calibration.

4. Quality Control Surveys

A field survey was performed by Compass Data, Inc. between December 1, 2013 and December 12, 2013. One hundred eighty six (186) check points were collected to calibrate and evaluate airborne LiDAR data in various land coverage categories throughout Logan County.

5. Final LiDAR Processing

5.1 ABGPS and IMU Processing

Airborne GPS

Applanix - POSGPS
Utilizing carrier phase ambiguity resolution on the fly (i.e., without initialization), the solution to sub-decimeter kinematic positioning without the operational constraint of static initialization as used in semi-kinematic or stop-and-go positioning was utilized for the airborne GPS post-processing.

The processing technique used by Applanix, Inc. for achieving the desired accuracy is Kinematic Ambiguity Resolution (KAR). KAR searches for ambiguities and uses a special method to evaluate the relative quality of each intersection (RMS). The quality indicator is used to evaluate the accuracy of the solution for each processing computation. In addition to the quality indicator, the software will compute separation plots between any two solutions, which will ultimately determine the acceptance of the airborne GPS post processing.

Inertial Data
The post-processing of inertial and aiding sensor data (i.e. airborne GPS post processed data) is to compute an optimally blended navigation solution. The Kalman filter-based aided inertial navigation algorithm generates an accurate (in the sense of least-square error) navigation solution that will retain the best characteristics of the processed input data. An example of inertial/GPS sensor blending is the following: inertial data is smooth in the short term. However, a free- inertial navigation solution has errors that grow without bound with time. A GPS navigation solution exhibits short-term noise but has errors that are bounded. This optimally blended navigation solution will retain the best features of both, i.e. the blended navigation solution has errors that are smooth and bounded. The resultant processing generates the following data:

- Position:	Latitude, Longitude, Altitude

- Velocity:	North, East, and Down components

- 3-axis attitude:	roll, pitch, true heading

- Acceleration:	x, y, z components

- Angular rates:	x, y, z components

The Applanix software, version 4.4, was used to determine both the ABGPS trajectory and the blending of inertial data.

The airborne GPS and blending of inertial and GPS post-processing were completed in multiple steps.

1. The collected data was transferred from the field data collectors to the main computer. Data was saved under the project number and separated between LiDAR mission dates. Inside each mission date, a sub-directory was created with the aircraft’s tail number and an A or B suffix was attached for the time of when the data was collected. Inside the tail number sub-directory, five sub- directories were also created EO, GPS, IMU, PROC, and RAW.

2. The aircraft raw data (IMU and GPS data combined) was run through a data extractor program. This separated the IMU and GPS data. In addition to the extracting of data, it provided the analyst the first statistics on the overall flight. The program was POSPac (POS post-processing PACkage).

3. Executing POSGPS program to derive accurate GPS positions for all flights: Applanix POSGPS

The software utilized for the data collected was PosGPS, a kinematic on- the-fly (OTF) processing software package. Post processing of the data is computed from each base station (Note: only base stations within the flying area were used) in both a forward and backward direction. This provides the analyst the ability to Quality Check (QC) the post processing, since different ambiguities are determined from different base stations and also with the same data from different directions.

The trajectory separation program is designed to display the time of week that the airborne or roving antenna traveled, and compute the differences found between processing runs. Processed data can be compared between a forward/reverse solution from one base station, a reverse solution from one base station and a forward solution from the second base station, etc. For the Applanix POSGPS processing, this is considered the final QC check for the given mission. If wrong ambiguities were found with one or both runs, the analyst would see disagreements from the trajectory plot, and re-processing would continue until an agreement was determined.

Once the analyst accepts a forward and reverse processing solution, the trajectory plot is analyzed and the combined solution is stored in a file format acceptable for the IMU post processor.

Please see Section 8 of the control report for the final accepted trajectory plots.

4. When the processed trajectory (either through POSGPS) data was accepted after quality control analysis, the combined solution is stored in a file format acceptable for the IMU post processor (i.e. POSProc).

5. Execute POS Proc. POS Proc comprises a set of individual processing interface tools that execute and provide the following functions:

The diagram below shows the organization of these tools, and is a function of the POSProc processing components.

Integrated Inertial Navigation (iin) Module. 
The name iin is a contraction of Integrated Inertial Navigation. iin reads inertial data and aiding data from data files specified in a processing environment file and computes the aided inertial navigation solution. The inertial data comes from a strapdown IMU. iin outputs the navigation data between start and end times at a data rate as specified in the environment file. iin also outputs Kalman filter data for analysis of estimation error statistics and smoother data that the smoothing program smth uses to improve the navigation solution accuracy.

iin implements a full strapdown inertial navigator that solves Newton’s equation of motion on the earth using inertial data from a strapdown IMU. The inertial navigator implements coning and sculling compensation to handle potential problems caused by vibration of the IMU.

Smoother Module (smth).
smth is a companion processing module to iin. smth is comprised of two individual functions that run in sequence. smth first runs the smoother function and then runs the navigation correction function.

The smth smoother function performs backwards-in-time processing of the forwards-in-time blended navigation solution and Kalman filter data generated by iin to compute smoothed error estimates. smth implements a modified Bryson-Frazier smoothing algorithm specifically designed for use with the iin Kalman filter. The resulting smoothed strapdown navigator error estimates at a given time point are the optimal estimates based on all input data before and after the given time point. In this sense, smth makes use of all available information in the input data. smth writes the smoothed error estimates and their RMS estimation errors to output data files.

The smth navigation correction function implements a feedforward error correction mechanism similar to that in the iin strapdown navigation solution using the smoothed strapdown navigation errors. smth reads in the smoothed error estimates and with these, corrects the strapdown

navigation data. The resulting navigation solution is called a Best Estimate of Trajectory (BET), and is the best obtainable estimate of vehicle trajectory with the available inertial and aiding sensor data.

The above mentioned modules provide the analyst the following statistics to ensure that the most optimal solution was achieved: a log of the iin processing, the Kalman filter Measurement Residuals, Smoothed RMS Estimation Errors, and Smoothed Sensor Errors and RMS,

5.2 LiDAR “Point Cloud” Processing

The ABGPS/IMU post processed data along with the LiDAR raw measurements were processed using Optech Incorporated’s ASDA software. This software was used to match the raw LiDAR measurements with the computed ABGPS/IMU positions and attitudes of the LiDAR sensor. The result was a “point cloud” of LiDAR measured points referenced to the ground control system.

5.3 LiDAR CALIBRATION

Introduction

The purpose of the LiDAR system calibration is to refine the system parameters in order for the post-processing software to produce a “point cloud” that best fits the actual ground.

The following report outlines the calibration techniques employed for this project.

Calibration Procedures

All Companies involved in collection routinely performs two types of calibrations on its airborne LiDAR system. The first calibration, system calibration, is performed whenever the LiDAR system is installed in the aircraft. This calibration is performed to define the system parameters affected by the physical misalignment of the system versus aircraft. The second calibration, in-situ calibration, is performed for each mission using that missions data. This calibration is performed to refine the system parameters that are affected by the on site conditions as needed.

System Calibration

The system calibration is performed whenever the LiDAR system is installed in the aircraft.  This calibration is performed to define the system parameters affected by the physical misalignment of the system versus aircraft. The main system parameters that are affected are the heading, pitch, roll, and mirror scale.

The system calibration is performed by collecting data over a known test site that incorporates a flat surface and a large, flat roofed building. A ground survey is completed to define the flat surface and the building corners. The processed LiDAR data and ground survey data is input into TerraSolid's TerraMatch software to determine the systematic errors. The system parameters are then corrected according to the determined errors and used in the processing of future LiDAR acquisition missions.

In-situ Calibration

The in-situ calibration is performed as needed using the mission’s data. This calibration is performed to refine the system parameters that are affected by the on site conditions.

For each mission, LiDAR data for at least one cross flight is acquired over the mission’s acquisition site. The processed data of the cross flight is compared to the perpendicular flight lines using either the Optech proprietary software or TerraSolid's TerraMatch software to determine if any systematic errors are present. In this calibration, the data of individual flight lines are compared against each other and their systematic errors are corrected in the final processed data.

5.4 LiDAR Processing

The LAS files are imported, verified, and parsed into manageable, tiled grids using GeoCue version 2012.1.27.7. GeoCue allows for ease of data management and process tracking.  Relative accuracy of flightline to flightline alignment is assessed.  Image 5.4a illustrates relative vertical alignment of flightlines.

- Green indicates a flightline difference of less than 0.16 feet;

- Yellow….	0.16 – 0.33 feet;

- Orange…	0.33 – 0.48 feet;

- Red……..	 0.48 – 0.64 feet;

- Magenta..	 0.64 feet or greater.

Image 5.4a:  Relative Accuracy Assessment

Areas containing dense vegetation coverage or inundation from water will show a greater elevation offset then is actually present in the ground data. This is due to these regions having a high number of returns from vegetation or non-ground objects and few returns from the ground causing the elevation offset to be exaggerated.

A few tiles are evaluated to ensure that the desired point density has been met.  Image 5.4b illustrates tiles analyzed for point density.  Quantum Spatial utilizes proprietary software to complete this task. A grid, sized according to the USGS version 13 specifications, based on the nominal post spacing, is used for point analysis. The USGS version 13 specification allows that a grid size up to 2 times the nominal post spacing be used. Point density is analyzed on the basis of this grid space size or cell and the result indicates the point density of the sampled tiles.

Image 5.4b: Locations of sample tiles in point density analysis.

Fifty eight tiles were analyzed.

3.2 foot grid size/point spacing

Total number of cells: 24050487
Total number of cells with one or more points: 23601542
Percentage of cells with 1 point or more: 98.13%

Once both the accuracy between swaths and data density is accepted an automated classification algorithm is performed using TerraSolid’s TerraScan, version 013.011. This produces the majority of the bare-earth datasets. Further, the data is processed to classify specific vegetation classes and man-made structures.

The remainder of the data is classified using manual classification techniques. The majority of the manual editing involves changing points initially classified as ground (class 2), to unclassified or non-ground (class 1). Erroneous low points and high points, including clouds, are classified to Noise (class

5.5 Check Point Validation

To ensure position of the assembled data it is verified against surveyed ground control data. TerraScan computes the vertical differences between surveyed ground control points and LiDAR collected points.

Check points are surveyed within the project area to provide calibration checks of the LiDAR point cloud. A report indicating comparative positional statistics is produced when LiDAR has been adjusted to control and can be found in Section 9, of this report.

Forty five (45) ground check points were made across the project area to be used in adjusting the data to position. These forty five points were collected by Quantum Spatial, as part of the one hundred eighty six (186) control points collected for the project as described in Section 4, acquired from December 1, 2013 and December 12, 2013.

5.6 Vertical Accuracy Assessment

Vertical accuracy assessment is conducted by comparing ground survey check point z values to processed LiDAR data z values by horizontal proximity. Differences in z values are calculated to express an RMSEz value.

The Fundamental Vertical Accuracy (FVA) of the LAS data achieved 7.39 cm at a 95% confidence level with an RMSE of 3.77 cm utilizing twenty-one (21) Open Terrain ground survey check points compared to a Triangulated Integrated Network (TIN) of the LiDAR ground points.

See attached “Final_Delivery_Report” and “Accuracy_Report”  for details of the ground survey control data.

| FVA Bare Earth Data Compared to TIN |  |  |  |
| --- | --- | --- | --- |
|  | Ground Cover Category | Number of Checkpoints | Result cm (ft.) |
| FVA | Open Terrain | 26 | 7.39 (0.243) |
| RMSEz | Open Terrain | 26 | 3.77 (0.124) |

The Supplemental Vertical Accuracy (SVA) and Consolidated Vertical Accuracy (CVA) results are in the following table. Ground survey check points made in various ground cover categories are compared to Digital Elevation Models (DEM) derived from the LiDAR data.

| Accuracy Results |  |  |  |
| --- | --- | --- | --- |
|  | Ground Cover Category | Number of Checkpoints | Result cm (ft.) |
| FVA | Open Terrain | 26 | 10.6 (0.349) |
| CVA | All categories | 119 | 13.50 (0.443) |
| SVA | Open Terrain | 26 | 9.90 (0.325) |
| SVA | Urban | 30 | 9.23 (0.303) |
| SVA | Tall Grass | 20 | 12.67 (0.416) |
| SVA | Brush | 20 | 17.83 (0.585) |
| SVA | Forest | 23 | 13.38 (0.439) |

5.7 LiDAR Data Delivery

Raw point cloud data supplied is in the following format:

- LAS, version 1.2

- GPS times adjusted to GPS Absolute

- Full swaths and delivered as 1 file per swath which did not exceed 2 gigabytes.

Classified point cloud data is also being supplied using the following criteria.

- LAS, version 1.2 in 5280 foot grid

- GPS times adjusted to GPS Absolute

- Classification scheme:

  - 1 – Processed, but unclassified

  - 2 – Bare Earth, Ground

  - 7 – Noise (Low or High, Manually identified, if needed)

  - 9 – Water

  - 10 – Ignored Ground (Breakline proximity)

Deliverables:

Break line polygons are collected in a Microstation environment to the project specifications using heads up and stereo techniques for collection of drainage and hydro features. They are checked for QC/QA. Upon acceptance the breaklines, either polygons or lines, are translated into ARC and imported to the final geo-database as separate features in ESRI standard.

Ground survey point locations in ESRI shapefile format.

Calibrated LiDAR points as full swaths per flightline.

Classified points as LAS following the standard established by The American Society for Photogrammetry and Remote Sensing (ASPRS) for LAS data on a per tile basis.

Bare earth Digital Elevation Models (DEM), hyrdo flattened on a per tile basis.

Intensity raster images are produced for each tile through GeoCue, in GeoTIFF format.

Metadata

6.	Conclusion

Sound procedures and use of new technologies ensure this project data will serve the United States Geological Survey and all users of the provided LiDAR derivative products well into the future. The models produced are accurate and representative of surface conditions at the time of data acquisition on Logan County, Illinois.

7	Flight Logs and Opus Solutions

Mission 20131123_223305, page 1

Mission 20131123_223305, page 2.

Mission 20131124_135651, page 1.

Mission 20131124_135651, page 2.

Mission 20131124_191118

Mission 20131126_144704

Mission 20131127_144235

Mission 20131127_213804

Mission 20131128_135545

Mission 20131130_155001

8	LiDAR GPS Processing Plots

Image 8.1a:  PDOP Plot for mission 20131123_223305.

Image 8.1b:  Separation Plot for mission 20131123_223305.

Image 8.2a:  PDOP Plot for mission 20131124_135651

Image 8.2b:  Separation Plot for mission 20131124_135651.

Image 8.3a:  PDOP Plot for mission 20131124_191118.

Image 8.3b:  Separation Plot for mission 20141124_191118.

Image 8.4a: PDOP Plot for mission 20131126_144704.

Image 8.4b:  Separation Plot for mission 20131126_144704.

Image 8.5a:  PDOP Plot for mission 20131127_144235.

Image 8.5b:  Separation Plot for mission 20131127_144235.

Image 8.6a:  PDOP Plot for mission 20131127_213804.

Image 8.6b:  Separation Plot for mission 20131127_213804.

Image 8.7a:  PDOP Plot for mission 20131128_135545.

Image 8.7b:  Separation Plot for mission 20131128_135545.

Image 8.8a:  PDOP Plot for mission 20131130_155001.

Image 8.8b:  Separation Plot for mission 20131130_155001.

9	QA/QC Output Control Report

Output Control Report on check points collected across the Logan County project area and used to calibrate LiDAR data position.

10  Ground Control Survey Reports

Refer to “Final_Delivery_Report” and “Accuracy_Report” for ground control survey results.